- Series

OFE-ECM

Energy Efficient Nortron Electric Furnace

Features

Color	-	Standard: grey.

Voltage	-	240V, 1-phase.

Construction	-	Steel casing, all insulated for reduced heat loss and noise.

-	One panel on casing provides access to internal parts.

Fan	-	High quality precision balanced blower for quiet operation and optimal performance.

Motor	-  Variable speed electronically commutated motor (ECM), permanently lubricated and rubber mounted for maximum vibration and noise reduction.

-	Relay and printed circuit board (PCB) control automatic operation of motor and allow for the addition of air conditioning or heat pump.

Heating element	-	Heavy duty, reliable Helical coil nickel-chromium alloy wire provide superior heat transfer with unrestricted air flow.

- Heating elements can be individually replaced.

- High-limit temperature control with automatic reset.

- Elements modulation is done by a low voltage printed circuit boards with rectifier/timers to prevent power surges.

Control	-	Heavy duty 24VAC transformer provides power to relays and controls.

-	Easy access built-in breakers with manual reset (except the 10kW models).

-	“Speed Selection” switch: “Low” or “High”.

-	“Heat Activation” switch: “Slow” or “Fast”.

-	Compatible with 1 or 2-stage thermostat with or without external unit.

Installation	-	Multi-position installation, sides, back, or bottom air returns.

- Approved for zero clearance installation against combustible surfaces.

Warranty	-	5-year warranty against defects.

Application	-	For cottages, homes or offices, also approved for mobile homes use. May be installed in a basement, closet, crawl space or any unused area or alcove.

- Models with Variable Speed Motor (ECM)

| # Product # | Volts (1 ph) 2 wires | kW (BTU/hr) | Amp. including motor | Motor HP | Static Pressure at 0.2" W.C. |  | Weight |  | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Motor Speed1 | cfm2 | lb | kg |  |
| 21ECM10 | 240 | 10.0 (34152) | 43.0 | 1/3 | Low | 775 | 108 | 50.0 |  |
| 21ECM15 | 240 | 15.0 (51228) | 64.0 | 1/3 | Medium | 1020 | 108 | 50.0 |  |
| 21ECM18 | 240 | 18.0 (61473) | 76.0 | 1/3 | Medium | 1149 | 110 | 49.9 |  |
| 21ECM20 | 240 | 20.0 (68304) | 84.0 | 1/3 | Medium | 1149 | 110 | 49.9 |  |
| 21ECM23 | 240 | 22.5 (76842) | 94.0 | 1/3 | Medium | 1149 | 115 | 52.2 |  |

- Option

| # Product # # Kit | Description | Qty |
| --- | --- | --- |
| FSB13 | Downflow sub-base |  |

1 Factory settings.

2 Refer to the installation instructions and owner’s manual for complete air flow tables.

3 Available with 10kW,15kW,18kW and 20kW models.

Standard color is grey.

Wall Controls

Compatible with any low voltage thermostat.

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |