Series

OAC	Commercial Wall Fan Heater

Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal, aluminum, 
semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 480V, 600V, 1 or 3-phase.

Construction	-	18-gauge steel front cover.

-	Bottom air outlet.

-	High-limit temperature control with automatic reset.

Fan	-	Closed, factory-lubricated motor.

-	160 cfm fan (single unit: 55 dBA); 2 X 160 cfm fan (double unit: 58 dBA);

3 X 160 cfm fan (triple unit: 61 dBA).

-	Fan delay purges heater of residual heat.

Heating element	-	Durable tubular heating element with fins.

Control	-	Built-in thermostat included with control knob or tamperproof.

- 24V relay, with or without transformer available.

Note: Thermostat is not included in heater with factory installed relay.

Any remote thermostat or relay must be connected to the heater control terminal block.

Installation	-	Minimum clearance from the floor and adjacent walls: 10 in. (25.4 cm).

-	Surface mounting box available.

- 	Semi recessed trim kit available.

Warranty	-	3-year warranty against defects.

Application	-	Apartment building, commercial building, entrance way, stairwell, garage.

.

Models

| Product #1 | Qty | Volts (ph) | Watts | BTU/hr | Weight |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | lb | kg |  |
| OAC01502-T |  | 120 (1) | 1500 | 5123 | 24,0 | 10,9 | # single |
| OAC02008-T |  | 208 (1) | 2000 | 6830 | 24,0 | 10,9 | # single |
| OAC04008-T |  | 208 (1) | 4000 | 13661 | 24,0 | 10,9 | # single |
| OAC04038-T |  | 208 (3) | 4000 | 13661 | 26,0 | 12,0 | # single |
| OAC04808-T |  | 208 (1) | 4800 | 16393 | 24,0 | 10,9 | # single |
| OAC05038-T |  | 208 (3) | 5000 | 17076 | 26,0 | 12,0 | # single |
| OAC06038-T |  | 208 (3) | 6000 | 20491 | 54,0 | 24,5 | # double |
| OAC08008-T |  | 208 (1) | 8000 | 27321 | 50,0 | 22,7 | # double |
| /OAC09038-T |  | 208 (3) | 9000 | 30737 | 90,0 | 40,9 | # triple |
| OAC12008-T |  | 208 (1) | 12000 | 40982 | 90,0 | 40,9 | # triple |
| OAC12038-T |  | 208 (3) | 12000 | 40982 | 90,0 | 40,9 | # triple |
| OAC01500-T |  | 240/208 (1) | 1500/11252 | 5123/3842 | 24,0 | 10,9 | # single |
| OAC02000-T |  | 240/208 (1) | 2000/15002 | 6830/5123 | 24,0 | 10,9 | # single |
| OAC03000-T |  | 240/208 (1) | 3000/22502 | 10246/7684 | 24,0 | 10,9 | # single |
| OAC03030-T |  | 240/208 (3) | 3000/22502 | 10246/7684 | 26,0 | 12,0 | single |
| OAC04000-T |  | 240/208 (1) | 4000/30002 | 13661/10246 | 24,0 | 10,9 | # single |
| OAC04030-T |  | 240/208 (3) | 4000/30002 | 13661/10246 | 26,0 | 12,0 | single |
| OAC04800-T |  | 240/208 (1) | 4800/36002 | 16393/12295 | 24,0 | 10,9 | # single |
| OAC05030-T |  | 240/208 (3) | 5000/37502 | 17076/12807 | 26,0 | 12,0 | # single |
| OAC06000-T |  | 240/208 (1) | 6000/45002 | 20491/15368 | 50,0 | 22,7 | double |
| OAC08000-T |  | 240/208 (1) | 8000/60002 | 27321/20491 | 50,0 | 22,7 | double |
| OAC12000-T |  | 240/208 (1) | 12000/90002 | 40982/30737 | 90,0 | 40,9 | triple |
| OAC01507-T |  | 347 (1) | 1500 | 5123 | 28,0 | 12,7 | single |
| OAC02007-T |  | 347 (1) | 2000 | 6830 | 28,0 | 12,7 | single |
| OAC03007-T |  | 347 (1) | 3000 | 10246 | 28,0 | 12,7 | single |
| OAC04007-T |  | 347 (1) | 4000 | 13661 | 28,0 | 12,7 | single |
| OAC04807-T |  | 347 (1) | 4800 | 16393 | 28,0 | 12,7 | single |
| OAC06007-T |  | 347 (1) | 6000 | 20491 | 55,0 | 25,0 | double |
| OAC08007-T |  | 347 (1) | 8000 | 27321 | 55,0 | 25,0 | double |
| OAC12007-T |  | 347 (1) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAC01504-T |  | 480 (1) | 1500 | 5123 | 28,0 | 12,7 | single |
| OAC02004-T |  | 480 (1) | 2000 | 6830 | 28,0 | 12,7 | single |
| OAC03004-T |  | 480 (1) | 3000 | 10246 | 28,0 | 12,7 | single |
| OAC03034-T |  | 480 (3) | 3000 | 10246 | 30,0 | 14,0 | double |
| OAC04004-T |  | 480 (1) | 4000 | 13661 | 28,0 | 12,7 | single |
| OAC04034-T |  | 480 (3) | 4000 | 13661 | 30,0 | 14,0 | single |
| OAC04804-T |  | 480 (1) | 4800 | 16393 | 28,0 | 12,7 | single |
| OAC05034-T |  | 480 (3) | 5000 | 17076 | 30,0 | 14,0 | single |
| OAC06004-T |  | 480 (1) | 6000 | 20491 | 55,0 | 25,0 | double |
| OAC08004-T |  | 480 (1) | 8000 | 27321 | 55,0 | 25,0 | double |
| OAC12004-T |  | 480 (1) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAC12034-T |  | 480 (3) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAC01506-T |  | 600 (1) | 1500 | 5123 | 28,0 | 12,7 | single |
| OAC02006-T |  | 600 (1) | 2000 | 6830 | 28,0 | 12,7 | single |
| OAC03006-T |  | 600 (1) | 3000 | 10246 | 28,0 | 12,7 | single |
| OAC03036-T |  | 600 (3) | 3000 | 10246 | 30,0 | 14,0 | single |
| OAC04006-T |  | 600 (1) | 4000 | 13661 | 28,0 | 12,7 | single |
| OAC04036-T |  | 600 (3) | 4000 | 13661 | 30,0 | 14,0 | single |
| OAC04806-T |  | 600 (1) | 4800 | 16393 | 28,0 | 12,7 | single |
| OAC05036-T |  | 600 (3) | 6000 | 17076 | 30,0 | 14,0 | single |
| OAC06006-T |  | 600 (1) | 6000 | 20491 | 55,0 | 25,0 | double |
| OAC06036-T |  | 600 (3) | 6000 | 20491 | 58,0 | 26,3 | double |
| OAC08006-T |  | 600 (1) | 8000 | 27321 | 55,0 | 25,0 | double |
| OAC09036-T |  | 600 (3) | 9000 | 30737 | 90,0 | 40,9 | triple |
| OAC12006-T |  | 600 (1) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAC12036-T |  | 600 (3) | 12000 | 40982 | 90,0 | 40,9 | triple |

### Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| KIT-OAC-BS1 | - | Surface mounting box, single unit |  |
| KIT-OAC-BS1-1 | - | Semi recessed 1 in. trim kit, single unit |  |
| KIT-OAC-BS1-2 | - | Semi recessed 2 in. trim kit, single unit |  |
| KIT-OAC-BS2 | - | Surface mounting box, double unit |  |
| KIT-OAC-BS2-1 | - | Semi recessed 1 in. trim kit, double unit |  |
| KIT-OAC-BS2-2 | - | Semi recessed 2 in. trim kit, double unit |  |
| KIT-OAC-BS3 | - | Surface mounting box, triple unit |  |
| KIT-OAC-BS3-1 | - | Semi recessed 1 in. trim kit, triple unit |  |
| KIT-OAC-BS3-2 | - | Semi recessed 2 in. trim kit, triple unit |  |
| KIT-OAC-DIS203 | DIS203 | Disconnect switch, double-pole, 20A at 277V and less |  |
| KIT-OAC-DIS403 | DIS403 | Disconnect switch, triple-pole, 40A at 600V max. |  |
| KIT-OAC-DIS803 | DIS803 | Disconnect switch, triple-pole, 80A at 600V max. |  |
| - | IV | Fan switch |  |
| - | OO | On/Standby heating switch |  |
| - | R4 | 24V control relay, without transformer |  |
| - | RT4 | 24V control relay, with transformer |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 Built-in thermostat included with control knob or tamperproof.

2 208V = 75% of wattage at 240V.

3 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

4 If the built-in thermostat is not required with the 24V control relay, remove the “-T” from the product number (E.g.: OAC04006-RT).

Add "BL" for white, "AM" for almond. Upcharge available for optional colors shown in the color chart section.

### 277V available upon request.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |