Series

OAC	Commercial Wall Fan Heater

Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal, aluminum, 
semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 480V, 600V, 1 or 3-phase.

Construction	-	18-gauge steel front cover.

-	Bottom air outlet.

-	High-limit temperature control with automatic reset.

Fan	-	Closed, factory-lubricated motor.

-	160 cfm fan (single unit: 55 dBA); 2 X 160 cfm fan (double unit: 58 dBA);

3 X 160 cfm fan (triple unit: 61 dBA).

-	Fan delay purges heater of residual heat.

Heating element	-	Durable tubular heating element with fins.

Control	-	Built-in thermostat included with control knob or tamperproof.

- 24V relay, with or without transformer available.

Note: Thermostat is not included in heater with factory installed relay.

Any remote thermostat or relay must be connected to the heater control terminal block.

Installation	-	Minimum clearance from the floor and adjacent walls: 10 in. (25.4 cm).

-	Surface mounting box available.

- 	Semi recessed trim kit available.

Warranty	-	5-year warranty against defects.

Application	-	Apartment building, commercial building, entrance way, stairwell, garage.

.

Models

| Product #1 | Qty | Volts (ph) | Watts | BTU/hr | Weight |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | lb | kg |  |
| OACU01502-T |  | 120 (1) | 1500 | 5123 | 24.0 | 10.9 | # single |
| OACU02008-T |  | 208 (1) | 2000 | 6830 | 24.0 | 10.9 | # single |
| OACU04008-T |  | 208 (1) | 4000 | 13661 | 24.0 | 10.9 | # single |
| OACU04038-T |  | 208 (3) | 4000 | 13661 | 26.0 | 12.0 | # single |
| OACU04808-T |  | 208 (1) | 4800 | 16393 | 24.0 | 10.9 | # single |
| OACU05038-T |  | 208 (3) | 5000 | 17076 | 26.0 | 12.0 | # single |
| OACU06038-T |  | 208 (3) | 6000 | 20491 | 54.0 | 24.5 | double |
| OACU08008-T |  | 208 (1) | 8000 | 27321 | 50.0 | 22.7 | double |
| OACU09038-T |  | 208 (3) | 9000 | 30737 | 90.0 | 40.9 | triple |
| OACU12008-T |  | 208 (1) | 12000 | 40982 | 90.0 | 40.9 | triple |
| OACU12038-T |  | 208 (3) | 12000 | 40982 | 90.0 | 40.9 | triple |
| OACU01500-T1 |  | 240/208 (1) | 1500/1125 | 5123/3842 | 24.0 | 10.9 | # single |
| OACU02000-T1 |  | 240/208 (1) | 2000/1500 | 6830/5123 | 24.0 | 10.9 | # single |
| OACU03000-T1 |  | 240/208 (1) | 3000/2250 | 10246/7684 | 24.0 | 10.9 | # single |
| OACU03030-T1 |  | 240/208 (3) | 3000/2250 | 10246/7684 | 26.0 | 12.0 | # single |
| OACU04000-T1 |  | 240/208 (1) | 4000/3000 | 13661/10246 | 24.0 | 10.9 | # single |
| OACU04030-T1 |  | 240/208 (3) | 4000/3000 | 13661/10246 | 26.0 | 12.0 | # single |
| OACU04800-T1 |  | 240/208 (1) | 4800/3600 | 16393/12295 | 24.0 | 10.9 | # single |
| OACU05030-T1 |  | 240/208 (3) | 5000/3750 | 17076/12807 | 26.0 | 12.0 | # single |
| OACU06000-T1 |  | 240/208 (1) | 6000/4500 | 20491/15368 | 50.0 | 22.7 | double |
| OACU08000-T1 |  | 240/208 (1) | 8000/6000 | 27321/20491 | 50.0 | 22.7 | double |
| OACU12000-T1 |  | 240/208 (1) | 12000/9000 | 40982/30737 | 90.0 | 40.9 | # triple |
| OACU01577-T |  | 277 (1) | 1500 | 5123 | 28.0 | 12.7 | # single |
| OACU02077-T |  | 277 (1) | 2000 | 6830 | 28.0 | 12.7 | # single |
| OACU03077-T |  | 277 (1) | 3000 | 10246 | 28.0 | 12.7 | # single |
| OACU04077-T |  | 277 (1) | 4000 | 13661 | 28.0 | 12.7 | # single |
| OACU04877-T |  | 277 (1) | 4800 | 16393 | 28.0 | 12.7 | # single |
| OACU06077-T |  | 277 (1) | 6000 | 20491 | 55.0 | 25.0 | double |
| OACU08077-T |  | 277 (1) | 8000 | 27321 | 55.0 | 25.0 | double |
| OACU12077-T |  | 277 (1) | 12000 | 40982 | 90.0 | 40.9 | triple |
| OACU01504-T |  | 480 (1) | 1500 | 5123 | 28.0 | 12.7 | # single |
| OACU02004-T |  | 480 (1) | 2000 | 6830 | 28.0 | 12.7 | # single |
| OACU03004-T |  | 480 (1) | 3000 | 10246 | 28.0 | 12.7 | # single |
| OACU03034-T |  | 480 (3) | 3000 | 10246 | 30.0 | 14.0 | # single |
| OACU04004-T |  | 480 (1) | 4000 | 13661 | 28.0 | 12.7 | # single |
| OACU04034-T |  | 480 (3) | 4000 | 13661 | 30.0 | 14.0 | # single |
| OACU04804-T |  | 480 (1) | 4800 | 16393 | 28.0 | 12.7 | # single |
| OACU05034-T |  | 480 (3) | 5000 | 17076 | 30.0 | 14.0 | # single |
| OACU06004-T |  | 480 (1) | 6000 | 20491 | 55.0 | 25.0 | double |
| OACU08004-T |  | 480 (1) | 8000 | 27321 | 55.0 | 25.0 | double |
| OACU12004-T |  | 480 (1) | 12000 | 40982 | 90.0 | 40.9 | # triple |
| OACU12034-T |  | 480 (3) | 12000 | 40982 | 90.0 | 40.9 | # triple |

### Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| KIT-OAC-BS1 | - | Surface mounting box, single unit |  |
| KIT-OAC-BS1-1 | - | Semi recessed 1 in. trim kit, single unit |  |
| KIT-OAC-BS1-2 | - | Semi recessed 2 in. trim kit, single unit |  |
| KIT-OAC-BS2 | - | Surface mounting box, double unit |  |
| KIT-OAC-BS2-1 | - | Semi recessed 1 in. trim kit, double unit |  |
| KIT-OAC-BS2-2 | - | Semi recessed 2 in. trim kit, double unit |  |
| KIT-OAC-BS3 | - | Surface mounting box, triple unit |  |
| KIT-OAC-BS3-1 | - | Semi recessed 1 in. trim kit, triple unit |  |
| KIT-OAC-BS3-2 | - | Semi recessed 2 in. trim kit, triple unit |  |
| KIT-OAC-DIS203 | DIS203 | Disconnect switch, double-pole, 20A at 277V and less |  |
| KIT-OAC-DIS403 | DIS403 | Disconnect switch, triple-pole, 40A at 600V max. |  |
| KIT-OAC-DIS803 | DIS803 | Disconnect switch, triple-pole, 80A at 600V max. |  |
| - | IV | Fan switch |  |
| - | OO | On/Standby heating switch |  |
| - | R4 | 24V control relay, without transformer |  |
| - | RT4 | 24V control relay, with transformer |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 Built-in thermostat included with control knob or tamperproof.

2 208V = 75% of wattage at 240V.

3 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

4 If the built-in thermostat is not required with the 24V control relay, remove the “-T” from the product number (E.g.: OACU04004-RT).

Add "BL" for white, "AM" for almond. Upcharge available for optional colors shown in the color chart section.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |