Series

OACP	Commercial Ceiling Fan Heater

Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal, aluminum, 
semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 480V, 600V, 1-phase.

Construction	-	18-gauge steel front cover.

-	High-limit temperature control with automatic reset.

Fan	-	Factory-lubricated motor (open and ventilated).

-	160 cfm fan (single unit: 55.5 dBA), 2 X 160 cfm fan (double unit: 58.5 dBA).

-	Fan delay purges heater of residual heat.

Heating element	-	Durable tubular heating element with fins.

Control	-	240/208V control circuit standard with contactor when needed 
(except 120V model).

- 24V relay, with or without transformer available.

- Built-in thermostat available.

Note: If an electronic thermostat is used, it must be compatible with forced-air heaters.

Any remote thermostat or relay must be connected to the heater control terminal block.

Installation	-	Maximum recommended installation height: 10 ft. (3 m).

-	Minimum clearance from adjacent walls: 10 in. (25.4 cm).

-	Surface mounting box available.

-	Adaptor for T-bar ceiling available.

Warranty	-	3-year warranty against defects.

Application	-	Apartment building, commercial building, drop ceiling, stairwell, garage.

Models

| Product # | Qty | Volts | Watts | BTU/hr | Weight |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | lb | kg |  |
| OACPU1502 |  | 120 | 1500 | 5123 | 24.0 | 10.9 | single |
| OACPU4008 |  | 208 | 4000 | 13661 | 24.0 | 10.9 | single |
| OACPU4808 |  | 208 | 4800 | 16393 | 24.0 | 10.9 | single |
| OACPU8008 |  | 208 | 8000 | 27321 | 50.0 | 22.7 | double |
| OACPU1500 |  | 240/208 | 1500/11251 | 5123/3842 | 24.0 | 10.9 | single |
| OACPU2000 |  | 240/208 | 2000/15001 | 6830/5123 | 24.0 | 10.9 | single |
| OACPU3000 |  | 240/208 | 3000/22501 | 10246/7684 | 24.0 | 10.9 | single |
| OACPU4000 |  | 240/208 | 4000/30001 | 13661/10246 | 24.0 | 10.9 | single |
| OACPU4800 |  | 240/208 | 4800/36001 | 16393/12295 | 24.0 | 10.9 | single |
| OACPU6000 |  | 240/208 | 6000/45001 | 20491/15368 | 50.0 | 22.7 | double |
| OACPU8000 |  | 240/208 | 8000/60001 | 27321/20491 | 50.0 | 22.7 | double |
| OACPU1577 |  | 277 | 1500 | 5123 | 28.0 | 12.7 | single |
| OACPU2077 |  | 277 | 2000 | 6830 | 28.0 | 12.7 | single |
| OACPU3077 |  | 277 | 3000 | 10246 | 28.0 | 12.7 | single |
| OACPU4077 |  | 277 | 4000 | 13661 | 28.0 | 12.7 | single |
| OACPU6077 |  | 277 | 6000 | 20491 | 50.0 | 22.7 | double |
| OACPU8077 |  | 277 | 8000 | 27321 | 50.0 | 22.7 | double |
| OACPU1504 |  | 480 | 1500 | 5123 | 28.0 | 12.7 | single |
| OACPU2004 |  | 480 | 2000 | 6830 | 28.0 | 12.7 | single |
| OACPU3004 |  | 480 | 3000 | 10246 | 28.0 | 12.7 | single |
| OACPU4004 |  | 480 | 4000 | 13661 | 28.0 | 12.7 | single |
| OACPU6004 |  | 480 | 6000 | 20491 | 50.0 | 22.7 | double |
| OACPU8004 |  | 480 | 8000 | 27321 | 50.0 | 22.7 | double |

### Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| KIT-OAWH-BSA | - | Surface mounting box for an extruded aluminum front grille, single unit |  |
| KIT-OAWH-BSA2 | - | Surface mounting box for an extruded aluminum front grille, double unit |  |
| KIT-OAC-BS1 | - | Surface mounting box, single unit |  |
| KIT-OAC-BS2 | - | Surface mounting box, double unit |  |
| KIT-OAC-DVA | - | Extruded aluminum front grille, single unit |  |
| KIT-OAC-DVA2 | - | Extruded aluminum front grille, double unit |  |
| KIT-OACP-DIS202 | DIS202 | Disconnect switch, double-pole, 20A at 240V |  |
| KIT-OACP-DIS402 | DIS402 | Disconnect switch, triple-pole, 40A at 600V |  |
| KIT-OACP-DIS802 | DIS802 | Disconnect switch, triple-pole, 80A at 600V |  |
| KIT-OACP-R | R | 24V control relay, without transformer |  |
| - | RT | 24V control relay, with transformer |  |
| KIT-OACP-T | T | Thermostat |  |
| KIT-OACP-T-BAR | - | Adaptor for T-bar ceiling installation, 23 3/4 in. X 23 3/4 in. (602 mm X 602 mm), single unit |  |
| KIT-OACP-T-BAR2 | - | Adaptor for T-bar ceiling installation, 23 3/4 in. X 47 11/16 in. (602 mm X 1211 mm), double unit |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 208V = 75% of wattage at 240V.

2  For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

Add "BL" for white, "AM" for almond. Upcharge available for optional colors shown in the color chart section.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |