Series

OAWH	Architectural Aluminum Wall Fan Heater

Features

Color	-	Standard: white, almond, metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

-	Optional: anodized aluminum.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 480V, 600V, 1 or 3-phase.

Construction	-	Extruded aluminum front grille, 1/8 in. (3.2 mm) thick.

-	Bottom air outlet.

-	High-limit temperature control with automatic reset.

Fan	-	Closed, factory-lubricated motor.

- 160 cfm fan (single unit: 55 dBA); 2 X 160 cfm fan (double unit: 58 dBA);

3 X 160 cfm (triple unit: 61 dBA).

-	Fan delay purges heater of residual heat.

Heating element	-	Durable tubular heating element with fins.

Control	-	Built-in tamperproof thermostat included.

- 24V relay, with or without transformer available.

Note: Thermostat is not included in heater with factory installed relay.

Any remote thermostat or relay must be connected to the heater control terminal block.

Installation	-	Minimum clearance from the floor and adjacent walls: 10 in. (25.4 cm).

-	Surface mounting box available.

- 	Semi recessed trim kit available.

Warranty	-	3-year warranty against defects.

Application 	-	Apartment building, commercial building, entrance way, stairwell, garage.

Models

| Product #1 | Qty | Volts (ph) | Watts | BTU/hr | Weight |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | lb | kg |  |
| OAWH01502-TAV |  | 120 (1) | 1500 | 5123 | 24,0 | 10,9 | single |
| OAWH02008-TAV |  | 208 (1) | 2000 | 6830 | 24,0 | 10,9 | single |
| OAWH04008-TAV |  | 208 (1) | 4000 | 13661 | 24,0 | 10,9 | single |
| OAWH04038-TAV |  | 208 (3) | 4000 | 13661 | 28,0 | 13,0 | single |
| OAWH04808-TAV |  | 208 (1) | 4800 | 16393 | 24,0 | 10,9 | single |
| OAWH05038-TAV |  | 208 (3) | 5000 | 17076 | 29,0 | 12,7 | single |
| OAWH06038-TAV |  | 208 (3) | 6000 | 20491 | 56,0 | 25,4 | double |
| OAWH08008-TAV |  | 208 (1) | 8000 | 27321 | 50,0 | 22,7 | double |
| OAWH09038-TAV |  | 208 (3) | 9000 | 30737 | 90,0 | 40,9 | triple |
| OAWH12008-TAV |  | 208 (1) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAWH12038-TAV |  | 208 (3) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAWH01500-TAV |  | 240/208 (1) | 1500/11252 | 5123/3842 | 24,0 | 10,9 | single |
| OAWH02000-TAV |  | 240/208 (1) | 2000/15002 | 6830/5123 | 24,0 | 10,9 | single |
| OAWH03000-TAV |  | 240/208 (1) | 3000/22502 | 10246/7684 | 24,0 | 10,9 | single |
| OAWH03030-TAV |  | 240/208 (3) | 3000/22502 | 10246/7684 | 28,0 | 13,0 | single |
| OAWH04000-TAV |  | 240/208 (1) | 4000/30002 | 13661/10246 | 24,0 | 10,9 | single |
| OAWH04030-TAV |  | 240/208 (3) | 4000/30002 | 13661/10246 | 28,0 | 13,0 | single |
| OAWH04800-TAV |  | 240/208 (1) | 4800/36002 | 16393/12295 | 24,0 | 10,9 | single |
| OAWH05030-TAV |  | 240/208 (3) | 5000/37502 | 17076/12807 | 28,0 | 12,7 | single |
| OAWH06000-TAV |  | 240/208 (1) | 6000/45002 | 20491/15368 | 50,0 | 22,7 | double |
| OAWH08000-TAV |  | 240/208 (1) | 8000/60002 | 27321/20491 | 50,0 | 22,7 | double |
| OAWH12000-TAV |  | 240/208 (1) | 12000/90002 | 40982/30737 | 90,0 | 40,9 | triple |
| OAWH01507-TAV |  | 347 (1) | 1500 | 5123 | 24,0 | 10,9 | single |
| OAWH02007-TAV |  | 347 (1) | 2000 | 6830 | 24,0 | 10,9 | single |
| OAWH03007-TAV |  | 347 (1) | 3000 | 10246 | 24,0 | 10,9 | single |
| OAWH04007-TAV |  | 347 (1) | 4000 | 13661 | 24,0 | 10,9 | single |
| OAWH04807-TAV |  | 347 (1) | 4800 | 16393 | 24,0 | 10,9 | single |
| OAWH06007-TAV |  | 347 (1) | 6000 | 20491 | 55,0 | 25,0 | double |
| OAWH08007-TAV |  | 347 (1) | 8000 | 27321 | 55,0 | 25,0 | double |
| OAWH12007-TAV |  | 347 (1) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAWH01504-TAV |  | 480 (1) | 1500 | 5123 | 28,0 | 12,7 | single |
| OAWH02004-TAV |  | 480 (1) | 2000 | 6830 | 28,0 | 12,7 | single |
| OAWH03004-TAV |  | 480 (1) | 3000 | 10246 | 28,0 | 12,7 | single |
| OAWH03034-TAV |  | 480 (3) | 3000 | 10246 | 32,0 | 15,0 | single |
| OAWH04004-TAV |  | 480 (1) | 4000 | 13661 | 28,0 | 12,7 | single |
| OAWH04034-TAV |  | 480 (3) | 4000 | 13661 | 32,0 | 15,0 | single |
| OAWH04804-TAV |  | 480 (1) | 4800 | 16393 | 28,0 | 12,7 | single |
| OAWH05034-TAV |  | 480 (3) | 5000 | 17076 | 32,0 | 15,0 | single |
| OAWH06004-TAV |  | 480 (1) | 6000 | 20491 | 55,0 | 25,0 | double |
| OAWH08004-TAV |  | 480 (1) | 8000 | 27321 | 55,0 | 25,0 | double |
| OAWH12004-TAV |  | 480 (1) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAWH12034-TAV |  | 480 (3) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAWH01506-TAV |  | 600 (1) | 1500 | 5123 | 28,0 | 12,7 | single |
| OAWH02006-TAV |  | 600 (1) | 2000 | 6830 | 28,0 | 12,7 | single |
| OAWH03006-TAV |  | 600 (1) | 3000 | 10246 | 28,0 | 12,7 | single |
| OAWH03036-TAV |  | 600 (3) | 3000 | 10246 | 32,0 | 15,0 | single |
| OAWH04806-TAV |  | 600 (1) | 4000 | 13661 | 28,0 | 12,7 | single |
| OAWH04036-TAV |  | 600 (3) | 4000 | 13661 | 32,0 | 15,0 | single |
| OAWH04806-TAV |  | 600 (1) | 4800 | 16393 | 28,0 | 12,7 | single |
| OAWH05036-TAV |  | 600 (3) | 5000 | 17076 | 32,0 | 15,0 | single |
| OAWH06006-TAV |  | 600 (1) | 6000 | 20491 | 55,0 | 25,0 | double |
| OAWH06036-TAV |  | 600 (3) | 6000 | 20491 | 60,0 | 27,2 | double |
| OAWH08006-TAV |  | 600 (1) | 8000 | 27321 | 55,0 | 25,0 | double |
| OAWH09036-TAV |  | 600 (3) | 9000 | 30737 | 90,0 | 40,9 | triple |
| OAWH12006-TAV1 |  | 600 (1) | 12000 | 40982 | 90,0 | 40,9 | triple |
| OAWH12036-TAV1 |  | 600 (3) | 12000 | 40982 | 90,0 | 40,9 | triple |

Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| KIT-OAWH-BSA | - | Surface mounting box, single unit |  |
| KIT-OAWH-BSA1-1 | - | Semi recessed 1 in. trim kit, single unit |  |
| KIT-OAWH-BSA1-2 | - | Semi recessed 2 in. trim kit, single unit |  |
| KIT-OAWH-BSA2 | - | Surface mounting box, double unit |  |
| KIT-OAWH-BSA2-1 | - | Semi recessed 1 in. trim kit, double unit |  |
| KIT-OAWH-BSA2-2 | - | Semi recessed 2 in. trim kit, double unit |  |
| KIT-OAWH-BSA3 | - | Surface mounting box, triple unit |  |
| KIT-OAWH-BSA3-1 | - | Semi recessed 1 in. trim kit, triple unit |  |
| KIT-OAWH-BSA3-2 | - | Semi recessed 2 in. trim kit, triple unit |  |
| KIT-OAWH-DIS203 | DIS203 | Disconnect switch, double-pole, 20A at 277V and less |  |
| KIT-OAWH-DIS403 | DIS403 | Disconnect switch, triple-pole, 40A at 600V max. |  |
| KIT-OAWH-DIS803 | DIS803 | Disconnect switch, triple-pole, 80A at 600V max. |  |
| - | IV | Fan switch |  |
| - | R4 | 24V control relay, without transformer |  |
| - | RT4 | 24V control relay, with transformer |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 Built-in tamperproof thermostat included.

3 208V = 75% of wattage at 240V.

2 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

4 If the built-in thermostat is not required with the 24V control relay, remove the “-TAV” from the product number (E.g.: OAWH04006-RT).

Add "BL" for white, "AM" for almond, "AE" for metallic silver, "BZ" for bronze, "CM" for mettallic charcoal, "AL" for aluminum, "NL" for semi-gloss black, «SA» for sand.

277V available upon request.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |