Series

OCA	Cabinet Unit Heater

Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal, aluminum, 
semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	208V, 240V, 277V, 347V, 480V, 600V, 1 or 3-phase.

Construction	-	18-gauge steel.

-	Insulated casing for noise reduction.

-	Extruded aluminum bar grilles prevent insertion of foreign objects.

-	Field adaptable air inlet and outlet.

-	High-limit temperature control with automatic reset.

-	240V control circuit included.

-	Optional fresh air inlet damper.

-	Optional air ﬁlters.

Fan	-	Direct drive, variable speed squirrel-cage fan operating.

-	Fan delay.

-	240/208V factory-lubricated motor (1/15 HP).

-	2 speed-motor available (1600/1500 rpm).

Heating element	-	Durable tubular heating element with fins.

Length	-	Four cabinet lengths are available: 28 in. (711 mm), 48 in. (1219 mm),

66 in. (1676 mm), 80 in. (2032 mm).

Control	-	Wide range of control available.

Ouellet highly recommends use of a wall thermostat for greater comfort.

Installation	-	Multi-position mounting (surface, semi-recessed, recessed).

-	Maximum recommended installation height: 10 ft. (3 m).

-	Recess trim kit available for recessed or semi-recessed units.

-	Sub-base and special back plate available.

Warranty	-	5-year warranty against defects.

Application	-	Hallway, entry way, exhibition center.

Models

| Watts |  | Product # |  |  |  |  |  |  |  |  |  | BTU/hr |  | cfm |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hi | Lo | 208V (3 ph) | Qty | 240V (1 ph) | Qty | 240V (3 ph) | Qty | 277V (1 ph) | Qty | 480V (3 ph) | Qty | Hi | Lo | Hi | Lo | in. | mm | lb | kg |
| 2000 | 1333 | OCAU020381 |  | OCAU02000 |  | OCAU02030 |  | OCAU02077 |  | OCAU020341 |  | 6830 | 4552 | 250 | 200 | 28 | 711 | 115 | 52.3 |
| 3000 | 1000 | OCAU030381 |  | OCAU03000 |  | OCAU03030 |  | OCAU03077 |  | OCAU030341 |  | 10246 | 3415 | 250 | 200 | 28 | 711 | 115 | 52.3 |
| 4000 | 2667 | OCAU040381 |  | OCAU04000 |  | OCAU04030 |  | OCAU04077 |  | OCAU040341 |  | 13661 | 9108 | 250 | 200 | 28 | 711 | 115 | 52.3 |
| 5000 | 3333 | OCAU050381 |  | OCAU05000 |  | OCAU05030 |  | OCAU05077 |  | OCAU050341 |  | 17076 | 11383 | 250 | 200 | 28 | 711 | 115 | 52.3 |
| 6000 | 4000 | OCAU060381 |  | OCAU06000 |  | OCAU06030 |  | OCAU06077 |  | OCAU060341 |  | 20491 | 13661 | 250 | 200 | 28 | 711 | 115 | 52.3 |
| 8000 | 4000 | OCAU080381 |  | OCAU08000 |  | OCAU08030 |  | OCAU08077 |  | OCAU080341 |  | 27321 | 13661 | 500 | 400 | 48 | 1219 | 146 | 66.4 |
| 10000 | 5000 | OCAU100381 |  | OCAU10000 |  | OCAU10030 |  | OCAU10077 |  | OCAU100341 |  | 34152 | 17076 | 500 | 400 | 48 | 1219 | 146 | 66.4 |
| 12000 | 6000 | OCAU120381 |  | OCAU12000 |  | OCAU12030 |  | OCAU12077 |  | OCAU120341 |  | 40982 | 20491 | 500 | 400 | 48 | 1219 | 146 | 66.4 |
| 15000 | 10000 | OCAU150381 |  | OCAU15000 |  | OCAU15030 |  | - | - | OCAU150341 |  | 51228 | 34152 | 750 | 600 | 66 | 1676 | 210 | 95.5 |
| 18000 | 12000 | OCAU180381 |  | OCAU18000 |  | OCAU18030 |  | - | - | OCAU180341 |  | 61473 | 40982 | 750 | 600 | 66 | 1676 | 210 | 95.5 |
| 20000 | 10000 | OCAU200381 |  | OCAU20000 |  | OCAU20030 |  | - | - | OCAU200341 |  | 68304 | 34152 | 1000 | 800 | 80 | 2032 | 240 | 109.1 |
| 24000 | 12000 | OCAU240381 |  | OCAU24000 |  | OCAU24030 |  | - | - | OCAU240341 |  | 81964 | 40982 | 1000 | 800 | 80 | 2032 | 240 | 109.1 |

1 1-phase available upon request (change the "3" for an "0" to the product number).

Add "BL" for white, "AM" for almond. Upcharge available for optional colors shown in the color chart section.

- Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| - | 120 | 120V control circuit |  |
| - | 3P | Triple-pole contactor |  |
| - | AF1 | Fresh air inlet |  |
| - | AFM 1 | Power-operated fresh air inlet |  |
| - | DIS40 4 | Disconnect switch, 40A |  |
| - | DIS80 4 | Disconnect switch, 80A |  |
| - | HL 2 | Hi/Lo power switch |  |
| KIT-OCA-IV | IV | On/Auto fan switch |  |
| KIT-OCA-OO | OO | On/Standby heating switch |  |
| KIT-OCA-R | R | 24V control relay, without transformer |  |
| - | RHL3 | 24V Hi/Lo power relay, with transformer |  |
| - | ROO3 | 24V On/Standby heating relay, with transformer |  |
| KIT-OCA-RT | RT | 24V control relay, with transformer |  |
| - | RV3 | 24V fan relay, with transformer |  |
| KIT-OCA-T-AV | TAV | Single-pole thermostat, tamperproof |  |
| - | T2S | 2-stage thermostat, tamperproof |  |
| Recess trim kit, 2 in. (51 mm) |  |  |  |
| KIT-OCA-CR-28 | - | 28 in. (711 mm) Models |  |
| KIT-OCA-CR-48 | - | 48 in. (1219 mm) Models |  |
| KIT-OCA-CR-66 | - | 66 in. (1676 mm) Models |  |
| KIT-OCA-CR-80 | - | 80 in. (2032 mm) Models |  |
| Special back plate |  |  |  |
| KIT-OCA-DU-28 | - | 28 in. (711 mm) Models |  |
| KIT-OCA-DU-48 | - | 48 in. (1219 mm) Models |  |
| KIT-OCA-DU-66 | - | 66 in. (1676 mm) Models |  |
| KIT-OCA-DU-80 | - | 80 in. (2032 mm) Models |  |
| Air filter, wall installation only |  |  |  |
| KIT-OCA-FI-28 | FI | 28 in. (711 mm) Models |  |
| KIT-OCA-FI-48 | FI | 48 in. (1219 mm) Models |  |
| KIT-OCA-FI-66 | FI | 66 in. (1676 mm) Models |  |
| KIT-OCA-FI-80 | FI | 80 in. (2032 mm) Models |  |
| Washable air filter, wall installation only |  |  |  |
| KIT-OCA-FIW-28 | FIW | 28 in. (711 mm) Models |  |
| KIT-OCA-FIW-48 | FIW | 48 in. (1219 mm) Models |  |
| KIT-OCA-FIW-66 | FIW | 66 in. (1676 mm) Models |  |
| KIT-OCA-FIW-80 | FIW | 80 in. (2032 mm) Models |  |
| Sub-base, 2 in. (51 mm) |  |  |  |
| KIT-OCA-SO-28 | - | 28 in. (711 mm) Models |  |
| KIT-OCA-SO-48 | - | 48 in. (1219 mm) Models |  |
| KIT-OCA-SO-66 | - | 66 in. (1676 mm) Models |  |
| KIT-OCA-SO-80 | - | 80 in. (2032 mm) Models |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 Do not install a built-in thermostat with these options.

2 OCAU06006, is not available with optional HL.

3 When one of these options is ordered, the heater control will automatically be shipped at 24V.

4 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

### Others

| Type | Quantity | Model et description Complementary information | Volts | Phase | Watts | Length mm/in. | Information Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |