- Series

OCB	Heavy-Duty Aluminum Draft Barrier

- Features

Color	-	Standard: white, almond.

- Optional (upcharge applicable): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.
-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

-	Optional: anodized aluminum.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 600V, 50/60 Hz, 1-phase.

Construction	-	14-gauge (0.0787 in. [2 mm]) aluminum front cover.

-	18-gauge satin coat steel cabinet.

-	Full length built-in wireway.

-	Separate low voltage wireway (-WW) available.

-	Pencil-proof openings less than 1/4 in. (6.4 mm).

-	Front fresh air inlet; top warm air outlet.

- Bottom fresh air inlet available (-BAI).

-	Clean back available (-BD).

-	Linear high-limit temperature control with automatic reset.

-	End caps included.

Heating element	-	Stainless steel tubular heating element with aluminum ﬁns.

-	Floating heating elements on high-temperature nylon bushings

reducing expansion noises.

Watt density	-	Average standard watt density of 275W/ft.

-	Average low watt density of 100W/ft., 150W/ft., 200W/ft.

Control	-	Built-in thermostat available with a control knob or tamperproof (installed on the right end only).

-	Mechanical, electronic or pneumatic relays available

(installed on the right end only).

Ouellet highly recommends to use an electronic wall thermostat for greater comfort.

Installation	-	Heater must be installed at least 3 in. (76 mm) from ﬁnished ﬂoor (for -BAI option).

-	Mounting holes spaced on 1 in. (25.4 mm) intervals for ease of installation.

-	Wall or floor mounted installation.

-	Variety of accessories available for wall-to-wall installation.

-	Pedestal kit available.

Warranty	-	3-year warranty against defects.

Application	-	Commercial building, office building, hospital, daycare center, private residence.

- Average Standard Watt Density Models 275W/ft.

| Watts | Product # |  |  |  |  |  |  |  | Watts 240/208V2 | Product # |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 120V | Qty | 208V | Qty | 347V | Qty | 600V | Qty |  | 240/208V | Qty | in. | mm | lb | kg |
| 500 | OCB0502 |  | OCB0508 |  | OCB0507 |  | - | - | 500/375 | OCB0500 |  | 28 | 711 | 8.0 | 3.6 |
| 750 | OCB0752 |  | OCB0758 |  | OCB0757 |  | OCB0756 |  | 750/563 | OCB0750 |  | 38 | 965 | 10.8 | 4.9 |
| 1000 | OCB1002 |  | OCB1008 |  | OCB1007 |  | OCB1006 |  | 1000/750 | OCB1000 |  | 48 3/16 | 1224 | 13.7 | 6.2 |
| 1250 | OCB1252 |  | OCB1258 |  | OCB1257 |  | OCB1256 |  | 1250/938 | OCB1250 |  | 57 11/16 | 1465 | 16.4 | 7.5 |
| 1500 | OCB1502 |  | OCB1508 |  | OCB1507 |  | OCB1506 |  | 1500/1125 | OCB1500 |  | 66 7/16 | 1687 | 18.8 | 8.6 |
| 1750 | - |  | OCB1758 |  | OCB1757 |  | OCB1756 |  | 1750/1313 | OCB1750 |  | 75 13/16 | 1925 | 21.5 | 9.8 |
| 2000 | - |  | OCB2008 |  | OCB2007 |  | OCB2006 |  | 2000/1500 | OCB2000 |  | 84 9/16 | 2148 | 24.0 | 10.9 |
| 2250 | - |  | OCB2258 |  | OCB2257 |  | OCB2256 |  | 2250/1688 | OCB2250 |  | 93 1/2 | 2375 | 26.5 | 12.1 |
| 2500 | - |  | OCB2508 |  | OCB2507 |  | OCB2506 |  | 2500/1875 | OCB2500 |  | 102 5/8 | 2606 | 29.1 | 13.2 |

- Average Low Watt Density Models

| Product #1 | Length | Weight | Average Density W/ft. | Watts | Watts 240/208V2 | 120V | Qty | 208V | Qty | 240/ 208V | Qty | 347V | Qty | 600V | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OCB020X-711 | 28" (711 mm) | 8.0 lb (3.6 kg) | 100 | 200 | 200/150 |  |  |  |  |  |  | ----- | ----- | ----- | ----- |
| OCB030X-711 |  |  | 150 | 300 | 300/225 |  |  |  |  |  |  | ----- | ----- | ----- | ----- |
| OCB040X-711 |  |  | 200 | 400 | 400/300 |  |  |  |  |  |  |  |  | ----- |  |
| OCB030X-965 | 38" (965 mm) | 10.8 lb (4.9 kg) | 100 | 300 | 300/225 |  |  |  |  |  |  |  |  | ----- | ----- |
| OCB045X-965 |  |  | 150 | 450 | 450/338 |  |  |  |  |  |  |  |  | ----- | ----- |
| OCB060X-965 |  |  | 200 | 600 | 600/450 |  |  |  |  |  |  |  |  | ----- | ----- |
| OCB040X-1224 | 48 3/16" (1224 mm) | 13.7 lb (6.2 kg) | 100 | 400 | 400/300 |  |  |  |  |  |  |  |  | ----- | ----- |
| OCB060X-1224 |  |  | 150 | 600 | 600/450 |  |  |  |  |  |  |  |  | ----- | ----- |
| OCB080X-1224 |  |  | 200 | 800 | 800/600 |  |  |  |  |  |  |  |  |  |  |
| OCB050X-1465 | 57 11/16" (1465 mm) | 16.4 lb (7.5 kg) | 100 | 500 | 500/375 |  |  |  |  |  |  |  |  | ----- | ----- |
| OCB075X-1465 |  |  | 150 | 750 | 750/563 |  |  |  |  |  |  |  |  |  |  |
| OCB100X-1465 |  |  | 200 | 1000 | 1000/750 |  |  |  |  |  |  |  |  |  |  |
| OCB060X-1687 | 66 7/16" (1687 mm) | 18.8 lb (8.6 kg) | 100 | 600 | 600/450 |  |  |  |  |  |  |  |  |  |  |
| OCB090X-1687 |  |  | 150 | 900 | 900/675 |  |  |  |  |  |  |  |  |  |  |
| OCB120X-1687 |  |  | 200 | 1200 | 1200/900 |  |  |  |  |  |  |  |  |  |  |
| OCB070X-1925 | 75 13/16" (1925 mm) | 21.5 lb (9.8 kg) | 100 | 700 | 700/525 |  |  |  |  |  |  |  |  |  |  |
| OCB105X-1925 |  |  | 150 | 1050 | 1050/788 |  |  |  |  |  |  |  |  |  |  |
| OCB140X-1925 |  |  | 200 | 1400 | 1400/1050 |  |  |  |  |  |  |  |  |  |  |
| OCB080X-2148 | 84 9/16" (2148 mm) | 24.0 lb (10.9 kg) | 100 | 800 | 800/600 |  |  |  |  |  |  |  |  |  |  |
| OCB120X-2148 |  |  | 150 | 1200 | 1200/900 |  |  |  |  |  |  |  |  |  |  |
| OCB160X-2148 |  |  | 200 | 1600 | 1600/1200 |  |  |  |  |  |  |  |  |  |  |
| OCB090X-2375 | 93 1/2" (2375 mm) | 26.5 lb (12.1 kg) | 100 | 900 | 900/675 |  |  |  |  |  |  |  |  |  |  |
| OCB135X-2375 |  |  | 150 | 1350 | 1350/1013 | ----- | ----- |  |  |  |  |  |  |  |  |
| OCB180X-2375 |  |  | 200 | 1800 | 1800/1350 | ----- | ----- |  |  |  |  |  |  |  |  |
| OCB100X-2606 | 102 5/8" (2606 mm) | 29.1 lb (13.2 kg) | 100 | 1000 | 1000/750 |  |  |  |  |  |  |  |  |  |  |
| OCB150X-2606 |  |  | 150 | 1500 | 1500/1125 | ----- | ----- |  |  |  |  |  |  |  |  |
| OCB200X-2606 |  |  | 200 | 2000 | 2000/1500 | ----- | ----- |  |  |  |  |  |  |  |  |

1 X = 2 (120V), 8 (208V), 0 (240/208V), 7 (347V), 6 (600V).

2 208V = 75% of wattage at 240V.

 = Available.

Add "BL" for white, "AM" for almond.

Other colors, accessories and finishes are available upon request. Please contact your customer service representative.

Other voltages available upon request.

- Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| Finish |  |  |  |
| - | BAI | Bottom fresh air inlet |  |
| - | BD1 | Clean back |  |
| KIT-OCB-CE | - | Outside corner, 6 in. (152 mm) |  |
| KIT-OCB-CI | - | Inside corner, 6 in. (152 mm) |  |
| KIT-OCB-JT | - | Splice plate, 1 3/16 in. (30 mm) |  |
| KIT-OCB-JT-PE | - | Contour splice plate for BD option and/or heater with pedestals, 1 3/16 po (30 mm) |  |
| KIT-OCB-PE | - | Round pedestal kit (2), 3 in. (76 mm) |  |
| KIT-OCB-PEC | - | Square-shaped pedestal kit (2), 3 in. (76 mm) |  |
| OCB-SV-XX | - | Empty cabinet with length of XX mm (minimum: 154 mm, maximum: 2606 mm) |  |
| Controls |  |  |  |
| KIT-OCB-DIS203 | DIS203 | Disconnect switch, 20A à 277V max. |  |
| - | PM | Pneumatic relay, 20A at 600V max. |  |
| KIT-OCB-R-347 | R | Single-pole relay, coil 24V, without transformer: 22A at 120V, 208V, 240V - 19A at 277V - 18A at 347V |  |
| KIT-OCB-R-600 | R | Single-pole relay, coil 24V, without transformer, 10A at 600V |  |
| KIT-OCB-RT-120 | RT | Single-pole relay, coil 24V, with transformer 120V/24V, 22A at 120V |  |
| KIT-OCB-RT-208 | RT | Single-pole relay, coil 24V, with transformer 208V/24V, 22A at 208V |  |
| KIT-OCB-RT-240 | RT | Single-pole relay, coil 24V, with transformer 240V/24V, 22A at 240V |  |
| KIT-OCB-RT-277 | RT | Single-pole relay, coil 24V, with transformer 277V/24V, 19A at 277V |  |
| KIT-OCB-RT-347 | RT | Single-pole relay, coil 24V, with transformer 347V/24V, 18A at 347V |  |
| KIT-OCB-RT-6002 | RT2 | Single-pole relay, coil 24V, with transformer 600V/24V, 10A at 600V (2-wire only) |  |
| KIT-OCB-T | T | Single-pole thermostat: 30A at 120V, 208V, 240V - 15A at 277V, 347V, 600V |  |
| KIT-OCB-T24 | T24 | Double-pole thermostat: 30A at 120, 208V, 240V - 15A at 277V, 347V, 600V |  |
| KIT-OCB-T-AV | TAV | Single-pole tamperproof thermostat: 30A at 120V, 208V, 240V - 15A at 277V, 347V, 600V |  |
| KIT-OCB-T2-AV4 | T2AV4 | Double-pole tamperproof thermostat: 30A at 120, 208V, 240V - 15A at 277V, 347V, 600V |  |
| KIT-OCB-TRIAC-347 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 15A at 347V max. |  |
| KIT-OCB-TRIAC-347T | TRIACT | Single-pole electronic relay, coil 24V, with transformer: 15A at 208V, 240V, 347V |  |
| KIT-OCB-TRIAC-600 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 6A at 600V max. |  |
| - | WW | 24V wireway |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 For anodized aluminum heaters, the clean back option will be fixed with visible rivets.

2 These 2-wire relays with transformer can't supply of 3-wire low voltage thermostats.

3 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

4 "Off" position.

### Others

| Type | Quantity | Model and description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |