Series

OCC	Portable Construction Heater

Features

Color	-	Standard: almond.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	240/208V, 1-phase.

Construction	-	Handle may be used as a reel for the electric cable.

-	6 ft. (2 m) molded electric cable for extra durability.

-	Safely recessed thermostat knob reduces heater size to a minimum.

-	High-limit temperature control with automatic reset (first level) and single operation thermal fuse (second level).

Fan	-	Open motor permanently lubricated.

Note: The open motor is not intended for use in a dusty environment.

-	300 cfm fan.

Heating element	-	Helical coil nickel-chromium alloy wire protected by ceramic insulators.

- Heating element provides instant heat.

Temperature rise	-	28 °C (51 °F).

Control	-	Built-in thermostat included.

Installation	-	4000W must be plugged into a grounded 3-pronged outlet, 20A, 250VAC.

-	4800W must be plugged into a grounded 3-pronged outlet, 30A, 250VAC.

Note: Always unplug heater when not is use.

Warranty	-	2-year warranty against defects.

Application	-	Construction area, workshop, garage.

Models

| Product #1 | Qty | Watts | Volts | Grounded 3-pronged outlet | Weight |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | lb | kg |
| OCCU4000AM-MO-TB6 |  | 4000/3000 | 240/208 | 20A, 250VAC | 16.0 | 7.3 |
| OCCU4800AM-MO-TB6 |  | 4800/3600 | 240/208 | 30A, 250VAC | 16.0 | 7.3 |

1Built-in thermostat included.

Standard color is almond.

### Others

| Type | Quantity | Model and description Complementary information | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact at Ouellet: |  |
| Comments: |  |