Series

OCF	Patio Infrared Heater

Features

Finish	-	Black anodized aluminum or stainless steel.

Voltage	-	120V, 240/208V, 60 Hz, 1-phase.

Construction	-	Weatherproof IP65 certified for outdoor use. 
	-	2 mm wall thickness aluminum extrusion casing.

-	High quality aluminum optical reflector.

-	High quality power cord:

120V model: 7.5 ft. (2.25 m) with NEMA 5-15P plug.

240/208V models: 6 ft. (1.8 m) hard wire.

Heating element	-	Woven carbon filament infrared lamp inside a 15 mm Quartz tube.

-	Element service life ± 5000h.

Wavelength	-	Mid-wave (2 µm).

Control	-	1500W and 3000W models: On/Off integrated regulator and a remote control

with 6 settings.

- 4000W and 6000W models: Two heating level, 50% and 100%. Heater can be

wired to 	bypass the remote if requires.

Certification	-	Comply with the standard UL2021:2015 Ed.4 and C22.2 No.46: 2013 Ed.9.

Installation	-	Horizontal installation only.

-	Pivoting wall mounting brackets included (120V and 240/208V models).

-  Mounting brackets for ceiling suspension included (240/208V models).

-	Installation minimum clearances:

- 1500W model: 6.7 ft. (2.0 m) from floor.

- 300W, 4000W and 6000W models: 8 ft. (2.4 m) from floor.

- 	All models: 12 in. (30 cm) from ceiling, 18 in. (45 cm) between the end of the unit    
  to the wall and 24 in. (61 cm) between the longer side of the unit to the wall.

-	120V Model: Wall mounted only.

Must be plugged into a grounded 3-pronged outlet, 15A, 120VAC.

-	240/208V models: Wall or ceiling mounted.

Hard-wired into an electrical junction box (not included).

Warranty	-	1-year warranty against defects.

-	90-day warranty on the element (for industrial applications only).

Application	-	Resdiential: Solarium, pool & spa, balcony, patio.

- Commercial: Workshop, garage, restaurant patio or bar, hotel entranceway, churches.

Models

| Product #1 | Volts | Watts | Grounded 3-pronged outlet | Dimensions |  |  |  |  |  | Weight |  | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | L |  | H |  | D |  |  |  |  |
|  |  |  |  | in. | mm | in. | mm | in. | mm | lb | kg |  |
| CF15120B | 120 | 1500 | 15A, 120VAC | 27 | 685.8 | 6 1/2 | 165 | 6 5/16 | 161 | 10 | 4.5 |  |
| CF30240SS2 | 240/208 | 3000/2250 | - | 38 | 965.2 | 6 1/2 | 165 | 6 5/16 | 161 | 14.3 | 6.4 |  |
| CF40240SS2 | 240/208 | 4000/3000 | - | 42 | 1066.8 | 9 | 228.6 | 9 | 228.6 | 40 | 18.2 |  |
| CF60240SS2 | 240/208 | 6000/4500 | - | 42 | 1066.8 | 9 | 228.6 | 9 | 228.6 | 40 | 18.2 |  |

1Mounting brackets included. Finish: “B” for black anodized aluminum, “SS” for stainless steel.

2Indoor residential installation forbidden.

### Others

| Type | Quantity | Model and description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |