Series

OCT	Contemporary Convector

Features

Color	-	Standard: white, metallic charcoal.

- Optional (upcharge applicable): almond, metallic silver, bronze,

aluminum, semi-gloss black, sable.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	240/208V, 50/60 Hz, 1-phase.

Construction	-	20-gauge steel.

-	Warm air outlet that directs the air to the centre of the room.

-	Bar grilles prevent insertion of foreign objects; 
smooth-finish paint is washable and anti-yellowing.

-	High-limit temperature control with automatic reset.

Heating element	-	2 quiet aluminum monoblock elements.

Watt density	-	700W/ft. (2320W/m).

Control	-	Built-in non programmable electronic thermostat (-THO models).

Installation	-	4 in. (102 mm) minimum clearance to ﬂoor and from an adjacent wall.

-	Quick surface-mounted installation, with the included wall mounting bracket.

Warranty	-	5-year warranty against defects.

(The warranty is valid only if the heater is controlled by a proportional pulsing

electronic thermostat with a cycle of 15 seconds.)

Application	-	Kitchen, living room, family room, bedroom, basement.

Models

| Product # |  |  |  | Volts | Watts | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Without Thermostat1 | Qty | # Non Programmable # Integrated # Thermostat | Qty |  |  | in. | mm | lb | kg |
| OCT0500 |  | OCT0500-THO |  | 240/208 | 500/375 | 18 | 457 | 13 | 6 |
| OCT0750 |  | OCT0750-THO |  | 240/208 | 750/563 | 20 3/4 | 527.5 | 15.5 | 7 |
| OCT1000 |  | OCT1000-THO |  | 240/208 | 1000/750 | 23 9/16 | 598 | 16.7 | 7.6 |
| OCT1500 |  | OCT1500-THO |  | 240/208 | 1500/1125 | 29 1/16 | 739 | 20 | 9.1 |
| OCT2000 |  | OCT2000-THO |  | 240/208 | 2000/1500 | 34 5/8 | 880 | 24 | 10.9 |

1The OCT warranty is valid only if the heater is controlled by a proportional electronic pulsing line thermostat with a cycle of 15 seconds maximum.

Add "BL" for white, "CM" metallic charcoal.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |