- Series

ODB	Aluminum Draft Barrier

With bottom

fresh air inlet

- Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

-	Optional: anodized aluminum.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 600V, 50/60 Hz, 1-phase.

Construction	-	14-gauge (0.0787 in. [2 mm]) aluminum front cover.

-	18-gauge satin coat steel cabinet.

-	Screw assembly for added strength.

-	Full length built-in wireway.

-	Pencil-proof openings less than 1/4 in. (6.4 mm).

-	Front fresh air inlet; top warm air outlet.

- Bottom fresh air inlet available (-BAI).

-	Clean back available (-BD).

-	Linear high-limit temperature control with automatic reset. 
High altitude version available (-HA).

-	End caps included.

Heating element	-	Stainless steel tubular heating element with aluminum ﬁns.

-	Floating heating element on high-temperature nylon bushings

reducing expansion noises.

Watt density	-	Average standard watt density of 275W/ft.

-	Average low watt density of 100W/ft., 150W/ft., 200W/ft.

Control	-	Built-in thermostat available with a control knob

or tamperproof.

- Mechanical, electronic or pneumatic relays available

(installed on the right end only).

Ouellet highly recommends to use an electronic wall thermostat for greater comfort.

Installation	-	Heater must be installed at least 2 in. (51 mm) from finished floor (for -BAI option).

-	Mounting holes spaced on 4 in. (102 mm) intervals for ease of installation.

-	Wall or floor mounted installation.

-	Variety of accessories available for wall-to-wall installation.

-	Pedestal kit available.

Warranty	-	3-year warranty against defects.

Application	-	Commercial building, office building, hospital, daycare center, private residence.

Average Standard Watt Density Models 275W/ft.

| Watts | Watts 240/208V1 | Product # |  |  |  |  |  |  |  |  |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120V | Qty | 208V | Qty | 240/208V | Qty | 347V | Qty | 600V | Qty | in. | mm | lb | kg |
| 500 | 500/375 | ODB0502 |  | ODB0508 |  | ODB0500 |  | ODB0507 |  | - | - | 29 1/16 | 738 | 8.0 | 3.6 |
| 750 | 750/563 | ODB0752 |  | ODB0758 |  | ODB0750 |  | ODB0757 |  | ODB0756 |  | 39 1/16 | 992 | 10.4 | 4.7 |
| 1000 | 1000/750 | ODB1002 |  | ODB1008 |  | ODB1000 |  | ODB1007 |  | ODB1006 |  | 49 5/16 | 1253 | 12.8 | 5.8 |
| 1250 | 1250/938 | ODB1252 |  | ODB1258 |  | ODB1250 |  | ODB1257 |  | ODB1256 |  | 58 13/16 | 1494 | 15.1 | 6.8 |
| 1500 | 1500/1125 | ODB1502 |  | ODB1508 |  | ODB1500 |  | ODB1507 |  | ODB1506 |  | 67 1/2 | 1715 | 17.2 | 7.8 |
| 1750 | 1750/1313 | - | - | ODB1758 |  | ODB1750 |  | ODB1757 |  | ODB1756 |  | 76 7/8 | 1952 | 19.4 | 8.8 |
| 2000 | 2000/1500 | - | - | ODB2008 |  | ODB2000 |  | ODB2007 |  | ODB2006 |  | 85 5/8 | 2175 | 21.5 | 9.7 |
| 2250 | 2250/1688 | - | - | ODB2258 |  | ODB2250 |  | ODB2257 |  | ODB2256 |  | 94 1/2 | 2400 | 23.6 | 10.7 |
| 2500 | 2500/1875 | - | - | ODB2508 |  | ODB2500 |  | ODB2507 |  | ODB2506 |  | 103 5/8 | 2632 | 25.8 | 11.7 |

- Average Low Watt Density Models

| Product #2 | Length | Weight | Average Density W/ft. | Watts | Watts 240/208V1 | 120V | Qty | 208V | Qty | 240/208V | Qty | 347V | Qty | 600V | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ODB020X-738 | 29 1/16" (738 mm) | 8.0 lb (3.6 kg) | 100 | 200 | 200/150 |  |  |  |  |  |  | ----- | ----- | ----- | ----- |
| ODB030X-738 |  |  | 150 | 300 | 300/225 |  |  |  |  |  |  | ----- | ----- | ----- | ----- |
| ODB040X-738 |  |  | 200 | 400 | 400/300 |  |  |  |  |  |  |  |  | ----- | ----- |
| ODB030X-992 | 39 1/16" (992 mm) | 10.4 lb (4.7 kg) | 100 | 300 | 300/225 |  |  |  |  |  |  |  |  | ----- | ----- |
| ODB045X-992 |  |  | 150 | 450 | 450/338 |  |  |  |  |  |  |  |  | ----- | ----- |
| ODB060X-992 |  |  | 200 | 600 | 600/450 |  |  |  |  |  |  |  |  | ----- | ----- |
| ODB040X-1253 | 49 5/16" (1253 mm) | 12.8 lb (5.8 kg) | 100 | 400 | 400/300 |  |  |  |  |  |  |  |  | ----- | ----- |
| ODB060X-1253 |  |  | 150 | 600 | 600/450 |  |  |  |  |  |  |  |  | ----- | ----- |
| ODB080X-1253 |  |  | 200 | 800 | 800/600 |  |  |  |  |  |  |  |  |  |  |
| ODB050X-1494 | 58 13/16" (1494 mm) | 15.1 lb (6.8 kg) | 100 | 500 | 500/375 |  |  |  |  |  |  |  |  | ----- | ----- |
| ODB075X-1494 |  |  | 150 | 750 | 750/563 |  |  |  |  |  |  |  |  |  |  |
| ODB100X-1494 |  |  | 200 | 1000 | 1000/750 |  |  |  |  |  |  |  |  |  |  |
| ODB060X-1715 | 67 1/2" (1715 mm) | 17.2 lb (7.8 kg) | 100 | 600 | 600/450 |  |  |  |  |  |  |  |  |  |  |
| ODB090X-1715 |  |  | 150 | 900 | 900/675 |  |  |  |  |  |  |  |  |  |  |
| ODB120X-1715 |  |  | 200 | 1200 | 1200/900 |  |  |  |  |  |  |  |  |  |  |
| ODB070X-1952 | 76 7/8" (1952 mm) | 19.6 lb (8.8 kg) | 100 | 700 | 700/525 |  |  |  |  |  |  |  |  |  |  |
| ODB105X-1952 |  |  | 150 | 1050 | 1050/788 |  |  |  |  |  |  |  |  |  |  |
| ODB140X-1952 |  |  | 200 | 1400 | 1400/1050 |  |  |  |  |  |  |  |  |  |  |
| ODB080X-2175 | 85 5/8" (2175 mm) | 21.5 lb (9.7 kg) | 100 | 800 | 800/600 |  |  |  |  |  |  |  |  |  |  |
| ODB120X-2175 |  |  | 150 | 1200 | 1200/900 |  |  |  |  |  |  |  |  |  |  |
| ODB160X-2175 |  |  | 200 | 1600 | 1600/1200 | ----- | ----- |  |  |  |  |  |  |  |  |
| ODB090X-2400 | 94 1/2" (2400 mm) | 23.6 lb (10.7 kg) | 100 | 900 | 900/675 |  |  |  |  |  |  |  |  |  |  |
| ODB135X-2400 |  |  | 150 | 1350 | 1350/1013 | ----- | ----- |  |  |  |  |  |  |  |  |
| ODB180X-2400 |  |  | 200 | 1800 | 1800/1350 | ----- | ----- |  |  |  |  |  |  |  |  |
| ODB100X-2632 | 103 5/8" (2632 mm) | 26.8 lb (11.7 kg) | 100 | 1000 | 1000/750 |  |  |  |  |  |  |  |  |  |  |
| ODB150X-2632 |  |  | 150 | 1500 | 1500/1125 | ----- | ----- |  |  |  |  |  |  |  |  |
| ODB200X-2632 |  |  | 200 | 2000 | 2000/1500 | ----- | ----- |  |  |  |  |  |  |  |  |
| ODB225X-2632 |  |  | 200 | 2250 | 2250/1688 | ----- | ----- |  |  |  |  |  |  |  |  |

- 1 208V = 75% of wattage at 240V.

- 2 X = 2 (120V), 8 (208V), 0 (240/208V), 7 (347V), 6 (600V).

-  = Available.

- Add "BL" for white, "AM" for almond.

Other colors, accessories and finishes are available upon request.

Other voltages available upon request.

- Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| Finish |  |  |  |
| - | BAI | Bottom fresh air inlet |  |
| - | BD1 | Clean back |  |
| KIT-ODB-CE | - | Outside corner, 4 in. (102 mm) |  |
| KIT-ODB-CI | - | Inside corner, 4 in. (102 mm) |  |
| KIT-ODB-JT | - | Splice plate, 2 in. (51 mm) |  |
| KIT-ODB-JT-PE | - | Contour splice plate for BD option and/or heater with pedestals, 2 in. (51 mm) |  |
| KIT-ODB-PE | - | Round pedestal kit (2), 3 in. (76 mm) |  |
| KIT-ODB-PEC | - | Square-shaped pedestal kit (2), 2 in. (51 mm) |  |
| ODB-SV-XX | - | Empty cabinet with length of XX mm (minimum: 154 mm, maximum: 2632 mm) |  |
| Controls |  |  |  |
| KIT-ODB-DIS204 | DIS204 | Disconnect switch, 20A at 277V max. |  |
| KIT-ODB-DIS404 | DIS404 | Disconnect switch, 40A at 600V max. |  |
| - | HA | Linear high-limit temperature control for high altitude (3000 ft. to 8200 ft.) |  |
| - | PM | Pneumatic relay, 20A at 600V max. |  |
| KIT-ODB-R-347 | R | Single-pole relay, coil 24V, without transformer: 22A at 120V, 208V, 240V - 19A at 277V - 18A at 347V |  |
| KIT-ODB-R-600 | R | Single-pole relay, coil 24V, without transformer, 10A at 600V |  |
| KIT-ODB-RT-1202 | RT2 | Single-pole relay, coil 24V, with transformer 120V/24V, 22A at 120V |  |
| KIT-ODB-RT-208 | RT | Single-pole relay, coil 24V, with transformer 208V/24V, 22A at 208V |  |
| KIT-ODB-RT-240 | RT | Single-pole relay, coil 24V, with transformer 240V/24V, 22A at 240V |  |
| KIT-ODB-RT-277 | RT | Single-pole relay, coil 24V, with transformer 277V/24V, 19A at 277V |  |
| KIT-ODB-RT-347 | RT | Single-pole relay, coil 24V, with transformer 347V/24V, 18A at 347V |  |
| KIT-ODB-RT-6002 | RT2 | Single-pole relay, coil 24V, with transformer 600V/24V, 10A at 600V (2-wire only) |  |
| KIT-ODB-TB6 | TB6 | Single-pole bi-metal thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-ODB-TB263 | TB263 | Double-pole bi-metal thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-ODB-TB6-AV | TB6AV | Single-pole bi-metal tamperproof thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-ODB-TB26-AV3 | TB26AV3 | Double-pole bi-metal tamperproof thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-ODB-TRIAC-347 | TRIAC | Single-pole electronic relay, coil 24V. without transformer, 15A at 347V max. |  |
| KIT-ODB-TRIAC-347T | TRIACT | Single-pole electronic relay, coil 24V. with transformer: 15A at 208V, 240V, 347V |  |
| KIT-ODB-TRIAC-600 | TRIAC | Single-pole electronic relay, coil 24V. without transformer, 6A at 600V max. |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 For anodized aluminum heaters, the clean back option will be fixed with visible rivets.

2 These 2-wire relays with transformer can't supply of 3-wire low voltage thermostats.

3 "Off" position.

4 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |