- Series

ODBA	Steel Draft Barrier

With bottom

fresh air inlet

- Features

Color 	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 600V, 50/60 Hz, 1-phase.

Construction	-	16-gauge steel front cover.

-	18-gauge satin coat steel cabinet.

-	Screw assembly for added strength.

-	Full length built-in wireway.

-	Front fresh air inlet; top warm air outlet.

- Bottom fresh air inlet available (-BAI).

-	Clean back available (-BD).

-	Linear high-limit temperature control with automatic reset. 
High altitude version available (-HA).

-	End caps included.

Heating element	-	Stainless steel tubular heating element with aluminum ﬁns.

-	Floating heating element on high-temperature nylon bushings

reducing expansion noises.

Watt density	-	Average standard watt density of 275W/ft.

-	Average low watt density of 100W/ft., 150W/ft., 200W/ft.

Control	-	Built-in thermostat available with a control knob

or tamperproof.

- Mechanical, electronic or pneumatic relays available

(installed on the right end only).

Ouellet highly recommends to use an electronic wall thermostat for greater comfort.

Installation	-	Heater must be installed at least 2 in. (51 mm) from finished floor

(for -BAI option).

-		Mounting holes spaced on 4 in. (102 mm) intervals for ease of installation.

-	Wall or floor mounted installation.

-	Variety of accessories available for wall-to-wall installation.

-	Pedestal kit available.

Warranty	-	1-year warranty against defects.

Application	-	Commercial building, office building, hospital, daycare center, private residence.

- Standard Watt Density Models 1

| Watts | Watts 240/208V2 | Product # |  |  |  |  |  |  |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120V | Qty | 208V | Qty | 240/208V | Qty | 277V | Qty | in. | mm | lb | kg |
| 500 | 500/375 | ODBA0502 |  | ODBA0508 |  | ODBA0500 |  | ODBA0577 |  | 28 7/8 | 733 | 10.5 | 4.8 |
| 750 | 750/563 | ODBA0752 |  | ODBA0758 |  | ODBA0750 |  | ODBA0777 |  | 38 7/8 | 987 | 12.7 | 5.8 |
| 1000 | 1000/750 | ODBA1002 |  | ODBA1008 |  | ODBA1000 |  | ODBA1077 |  | 49 1/8 | 1248 | 14.9 | 6.8 |
| 1250 | 1250/938 | ODBA1252 |  | ODBA1258 |  | ODBA1250 |  | ODBA1277 |  | 58 5/8 | 1489 | 17.0 | 7.7 |
| 1500 | 1500/1125 | ODBA1502 |  | ODBA1508 |  | ODBA1500 |  | ODBA1577 |  | 67 5/16 | 1710 | 18.8 | 8.5 |
| 1750 | 1750/1313 | - | - | ODBA1758 |  | ODBA1750 |  | ODBA1777 |  | 76 5/8 | 1947 | 20.8 | 9.4 |
| 2000 | 2000/1500 | - | - | ODBA2008 |  | ODBA2000 |  | ODBA2077 |  | 85 7/16 | 2170 | 22.7 | 10.3 |
| 2250 | 2250/1688 | - | - | ODBA2258 |  | ODBA2250 |  | ODBA2277 |  | 94 5/16 | 2395 | 24.6 | 11.2 |
| 2500 | 2500/1875 | - | - | ODBA2508 |  | ODBA2500 |  | ODBA2577 |  | 103 7/16 | 2627 | 26.6 | 12.1 |

- Low Watt Density Models

| Product #3 | Length | Weight | Average Density W/ft. | Watts | Watts 240/208V2 | 120V | Qty | 208V | Qty | 240/208V | Qty | 277V | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ODBA020X-733 | 28 7/8" (733 mm) | 11.6 lb (5.3 kg) | 100 | 200 | 200/150 |  |  |  |  |  |  | ----- | ----- |
| ODBA030X-733 |  |  | 150 | 300 | 300/225 |  |  |  |  |  |  | ----- | ----- |
| ODBA040X-7331 |  |  | 200 | 400 | 400/300 |  |  |  |  |  |  |  |  |
| ODBA030X-987 | 38 7/8" (987 mm) | 15.1 lb (6.9 kg) | 100 | 300 | 300/225 |  |  |  |  |  |  |  |  |
| ODBA045X-987 |  |  | 150 | 450 | 450/338 |  |  |  |  |  |  |  |  |
| ODBA060X-9871 |  |  | 200 | 600 | 600/450 |  |  |  |  |  |  |  |  |
| ODBA040X-1248 | 49 1/8" (1248 mm) | 18.7 lb (8.5 kg) | 100 | 400 | 400/300 |  |  |  |  |  |  |  |  |
| ODBA060X-1248 |  |  | 150 | 600 | 600/450 |  |  |  |  |  |  |  |  |
| ODBA080X-12481 |  |  | 200 | 800 | 800/600 |  |  |  |  |  |  |  |  |
| ODBA050X-1489 | 58 5/8" (1489 mm) | 22.0 lb (10.0 kg) | 100 | 500 | 500/375 |  |  |  |  |  |  |  |  |
| ODBA075X-1489 |  |  | 150 | 750 | 750/563 |  |  |  |  |  |  |  |  |
| ODBA100X-14891 |  |  | 200 | 1000 | 1000/750 |  |  |  |  |  |  |  |  |
| ODBA060X-1710 | 67 5/16" (1710 mm) | 25.0 lb (11.4 kg) | 100 | 600 | 600/450 |  |  |  |  |  |  |  |  |
| ODBA090X-1710 |  |  | 150 | 900 | 900/675 |  |  |  |  |  |  |  |  |
| ODBA120X-17101 |  |  | 200 | 1200 | 1200/900 |  |  |  |  |  |  |  |  |
| ODBA070X-1947 | 76 5/8" (1947 mm) | 28.3 lb (12.8 kg) | 100 | 700 | 700/525 |  |  |  |  |  |  |  |  |
| ODBA105X-1947 |  |  | 150 | 1050 | 1050/788 |  |  |  |  |  |  |  |  |
| ODBA140X-19471 |  |  | 200 | 1400 | 1400/1050 |  |  |  |  |  |  |  |  |
| ODBA080X-2170 | 85 7/16" (2170 mm) | 31.3 lb (14.2 kg) | 100 | 800 | 800/600 |  |  |  |  |  |  |  |  |
| ODBA120X-2170 |  |  | 150 | 1200 | 1200/900 |  |  |  |  |  |  |  |  |
| ODBA160X-21701 |  |  | 200 | 1600 | 1600/1200 |  |  |  |  |  |  |  |  |
| ODBA090X-2395 | 94 5/16" (2395 mm) | 34.4 lb (15.6 kg) | 100 | 900 | 900/675 |  |  |  |  |  |  |  |  |
| ODBA135X-2395 |  |  | 150 | 1350 | 1350/1013 | ----- | ----- |  |  |  |  |  |  |
| ODBA180X-23951 |  |  | 200 | 1800 | 1800/1350 | ----- | ----- |  |  |  |  |  |  |
| ODBA100X-2627 | 103 7/16" (2627 mm) | 37.6 lb (17.6 kg) | 100 | 1000 | 1000/750 |  |  |  |  |  |  |  |  |
| ODBA150X-2627 |  |  | 150 | 1500 | 1500/1125 | ----- | ----- |  |  |  |  |  |  |
| ODBA200X-26271 |  |  | 200 | 2000 | 2000/1500 | ----- | ----- |  |  |  |  |  |  |
| ODBA225X-26271 |  |  | 200 | 2250 | 2250/1688 | ----- | ----- |  |  |  |  |  |  |

- 1 Not for residential use in the United States.

- 2 X = 2 (120V), 8 (208V), 0 (240/208V), 7 (347V), 6 (600V).

-  = Available.

Add "BL" for white, "AM" for almond. Optional colors shown in the color chart section are available upon request.

Other colors, accessories and finishes are available upon request. Please contact your customer service representative.

Other voltages available upon request.

- Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| Finish |  |  |  |
| - | BAI | Bottom fresh air inlet |  |
| - | BD | Clean back |  |
| KIT-ODBA-CE | - | Outside corner, 4 in. (102 mm) |  |
| KIT-ODBA-CI | - | Inside corner, 4 in. (102 mm) |  |
| KIT-ODBA-JT | - | Splice plate, 2 in. (51 mm) |  |
| KIT-ODBA-JT-PE | - | Contour splice plate for BD option and/or heater with pedestals, 2 in. (51 mm) |  |
| KIT-ODBA-PE | - | Round pedestal kit (2), 3 in. (76 mm) |  |
| KIT-ODBA-PEC | - | Square-shaped pedestal kit (2), 2 in. (51 mm) |  |
| ODBA-SV-XX | - | Empty cabinet length of XX mm (minimum: 154 mm, maximum: 2625 mm) |  |
| Controls |  |  |  |
| KIT-ODBA-DIS203 | DIS203 | Disconnect switch, 20A at 277V max. |  |
| KIT-ODBA-DIS403 | DIS403 | Disconnect switch, 40A at 600V max. |  |
| - | HA | Linear high-limit temperature control for high altitude (3000 ft. to 8200 ft.) |  |
| - | PM | Pneumatic relay, 20A at 600V max. |  |
| KIT-ODBA-R-347 | R | Single-pole relay, coil 24V, without transformer: 22A at 120V, 208V, 240V - 19A at 277V - 18A at 347V |  |
| KIT-ODBA-R-600 | R | Single-pole relay, coil 24V, without transformer, 10A at 600V |  |
| KIT-ODBA-RT-120 | RT | Single-pole relay, coil 24V, with transformer 120V/24V, 22A at 120V |  |
| KIT-ODBA-RT-208 | RT | Single-pole relay, coil 24V, with transformer 208V/24V, 22A at 208V |  |
| KIT-ODBA-RT-240 | RT | Single-pole relay, coil 24V, with transformer 240V/24V, 22A at 240V |  |
| KIT-ODBA-RT-277 | RT | Single-pole relay, coil 24V, with transformer 277V/24V, 19A at 277V |  |
| KIT-ODBA-RT-347 | RT | Single-pole relay, coil 24V, with transformer 347V/24V, 18A at 347V |  |
| KIT-ODBA-RT-6001 | RT1 | Single-pole relay, coil 24V, with transformer 600V/24V, 10A at 600V (2-wire only) |  |
| KIT-ODBA-TB6 | TB6 | Single-pole bi-metal thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-ODBA-TB262 | TB262 | Double-pole bi-metal thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-ODBA-TB6-AV | TB6AV | Single-pole bi-metal tamperproof thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-ODBA-TB26-AV2 | TB26AV2 | Double-pole bi-metal tamperproof thermostat: 25A at 120V, 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-ODBA-TRIAC-347 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 15A max. at 347V max. |  |
| KIT-ODBA-TRIAC-347T | TRIACT | Single-pole electronic relay, coil 24V, with transformer: 15A at 208V, 240V, 347V |  |
| KIT-ODBA-TRIAC-600 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 6A at 600V max. |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 These 2-wire relays with transformer can't supply of 3-wire low voltage thermostats.

2 "Off" position.

3 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |