Series

ODS	Ceiling Fan Heater

Features

Color	-	Standard: white, almond.

- Optional (10% upcharge): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black.

- Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	208V, 240V, 277V, 347V, 480V, 600V, 1 or-3 phase.

Construction	-	Outlet grille made of flat ribbon wire.

-	High-limit temperature control with automatic reset.

Fan	-	Totally enclosed and lifetime-lubricated ball bearing motor.

-	Fan delay purges heater of residual heat.

Heating element	-	Three stainless steel tubular heating elements with aluminum fins.

Control	-	240/208V control circuit standard (except for 2 to 5kW at 277V have 
277V control standard).

-	Built-in 240V contactor for some models (see table).

-	24V and 120V control circuit available.

-	Built-in tamperproof thermostat available.

-	24V relay (with or without transformer) available.

-	Continuous circulation controlled by a built-in switch with power-on indicator

light available (-CV).

Any remote thermostat or relay must be connected to the heater control terminal block.

Installation	-	Recommended height: between 8 ft. (2.4 m) and 12 ft. (3.6 m).

-	Recessed into insulated or T-bar ceiling (ODSR).

-	Surface mounting (ODSA and ODSB), T-BAR adaptor optional.

Note: We recommend installing this product on an outside ceiling to avoid vibrations spreading

to an adjacent room.

Warranty	-	3-year warranty against defects.

-	10-year warranty on the heating element.

Application	-	Apartment building, commercial building, drop ceiling, stairwell, garage.

Models

| Watts | Volts (ph) | Product # | Qty | Volts (ph) | Product # | Qty | cfm | Temperature Rise |  | Motor |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | °C | °F | HP | rpm |
| ODSA Surface |  |  |  |  |  |  |  |  |  |  |  |
| 2000 | 240 (1) | ODSA02000 |  | 240 (3) | ODSA020301 |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 240 (1) | ODSA03000 |  | 240 (3) | ODSA030301 |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 240 (1) | ODSA04000 |  | 240 (3) | ODSA040301 |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 240 (1) | ODSA050001 |  | 240 (3) | ODSA050301 |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 208 (1) | ODSA02008 |  | 208 (3) | ODSA020381 |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 208 (1) | ODSA03008 |  | 208 (3) | ODSA030381 |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 208 (1) | ODSA04008 |  | 208 (3) | ODSA040381 |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 208 (1) | ODSA050081 |  | 208 (3) | ODSA050381 |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 277 (1) | ODSA02077 |  | - | - |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 277 (1) | ODSA03077 |  | - | - |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 277 (1) | ODSA04077 |  | - | - |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 277 (1) | ODSA05077 |  | - | - |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 347 (1) | ODSA020071 |  | - | - |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 347 (1) | ODSA030071 |  | - | - |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 347 (1) | ODSA040071 |  | - | - |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 347 (1) | ODSA050071 |  | - | - |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 480 (1) | ODSA020041 |  | 480 (3) | ODSA020341 |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 480 (1) | ODSA030041 |  | 480 (3) | ODSA030341 |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 480 (1) | ODSA040041 |  | 480 (3) | ODSA040341 |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 480 (1) | ODSA050041 |  | 480 (3) | ODSA050341 |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 600 (1) | ODSA020061 |  | 600 (3) | ODSA020361 |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 600 (1) | ODSA030061 |  | 600 (3) | ODSA030361 |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 600 (1) | ODSA040061 |  | 600 (3) | ODSA040361 |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 600 (1) | ODSA050061 |  | 600 (3) | ODSA050361 |  | 310 | 28 | 51 | 1/50 | 1550 |
| ODSB Surface |  |  |  |  |  |  |  |  |  |  |  |
| 7500 | 240 (1) | ODSB075001 |  | 240 (3) | ODSB075301 |  | 600 | 22 | 39 | 1/33 | 1550 |
| 10000 | 240 (1) | ODSB100001 |  | 240 (3) | ODSB100301 |  | 600 | 29 | 53 | 1/33 | 1550 |
| 7500 | 208 (1) | ODSB075081 |  | 208 (3) | ODSB075381 |  | 600 | 22 | 39 | 1/33 | 1550 |
| 9900 | 208 (1) | ODSB099081 |  | 208 (3) | ODSB099381 |  | 600 | 29 | 53 | 1/33 | 1550 |
| 7500 | 277 (1) | ODSB075771 |  | 347 (1) | ODSB075071 |  | 600 | 22 | 39 | 1/33 | 1550 |
| 10000 | 277 (1) | ODSB100771 |  | 347 (1) | ODSB100071 |  | 600 | 29 | 53 | 1/33 | 1550 |
| 7500 | 480 (1) | ODSB075041 |  | 480 (3) | ODSB075341 |  | 600 | 22 | 39 | 1/33 | 1550 |
| 10000 | 480 (1) | ODSB100041 |  | 480 (3) | ODSB100341 |  | 600 | 29 | 53 | 1/33 | 1550 |
| 7500 | 600 (1) | ODSB075061 |  | 600 (3) | ODSB075361 |  | 600 | 22 | 39 | 1/33 | 1550 |
| 10000 | 600 (1) | ODSB100061 |  | 600 (3) | ODSB100361 |  | 600 | 29 | 53 | 1/33 | 1550 |
| ODSR Recessed |  |  |  |  |  |  |  |  |  |  |  |
| 2000 | 240 (1) | ODSR02000 |  | 240 (3) | ODSR020301 |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 240 (1) | ODSR03000 |  | 240 (3) | ODSR030301 |  | 310 | 17 | 39 | 1/50 | 1550 |
| 4000 | 240 (1) | ODSR04000 |  | 240 (3) | ODSR040301 |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 240 (1) | ODSR050001 |  | 240 (3) | ODSR050301 |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 208 (1) | ODSR02008 |  | 208 (3) | ODSR020381 |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 208 (1) | ODSR03008 |  | 208 (3) | ODSR030381 |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 208 (1) | ODSR04008 |  | 208 (3) | ODSR040381 |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 208 (1) | ODSR050081 |  | 208 (3) | ODSR050381 |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 277 (1) | ODSR02077 |  | - | - |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 277 (1) | ODSR03077 |  | - | - |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 277 (1) | ODSR04077 |  | - | - |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 277 (1) | ODSR05077 |  | - | - |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 347 (1) | ODSR020071 |  | - | - |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 347 (1) | ODSR030071 |  | - | - |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 347 (1) | ODSR040071 |  | - | - |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 347 (1) | ODSR050071 |  | - | - |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 480 (1) | ODSR020041 |  | 480 (3) | ODSR020341 |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 480 (1) | ODSR030041 |  | 480 (3) | ODSR030341 |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 480 (1) | ODSR040041 |  | 480 (3) | ODSR040341 |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 480 (1) | ODSR050041 |  | 480 (3) | ODSR050341 |  | 310 | 28 | 51 | 1/50 | 1550 |
| 2000 | 600 (1) | ODSR020061 |  | 600 (3) | ODSR020361 |  | 310 | 11 | 20 | 1/50 | 1550 |
| 3000 | 600 (1) | ODSR030061 |  | 600 (3) | ODSR030361 |  | 310 | 17 | 30 | 1/50 | 1550 |
| 4000 | 600 (1) | ODSR040061 |  | 600 (3) | ODSR040361 |  | 310 | 23 | 41 | 1/50 | 1550 |
| 5000 | 600 (1) | ODSR050061 |  | 600 (3) | ODSR050361 |  | 310 | 28 | 51 | 1/50 | 1550 |

### Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| - | 120 | 120V control circuit |  |
| - | CV | Summer fan switch and pilot light |  |
| KIT-ODS-DIS202 | DIS202 | Disconnect switch, double-pole, 20A at 277V and less |  |
| KIT-ODS-DIS402 | DIS402 | Disconnect switch, triple-pole, 40A at 600V max. |  |
| - | DIS802 | Disconnect switch, triple-pole, 80A at 600V |  |
| KIT-ODS-R | R | 24V control relay, without transformer |  |
| - | RCV24 | Summer fan relay 24V, remote control and pilot light (switch not included) |  |
| - | RCV240 | Summer fan relay 240V, remote control and pilot light (switch not included) |  |
| - | RT | 24V control relay, with transformer |  |
| KIT-ODS-T-AV | TAV | Thermostat tamperproof |  |
| KIT-ODSA-T-BAR | - | Adaptor for T-bar ceiling, ODSA only |  |
| KIT-ODSB-T-BAR | - | Adaptor for T-bar ceiling, ODSB only |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 Built-in 240V contactor.

2 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |