Series

OFB	Compact Wall Fan Heater

Features

Color	-	Standard: white.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 240/208V, 1-phase.

Construction	-	20-gauge steel cabinet.

-	Warm air outlet that directs the air downwards, towards the center of the room.

-	Bar grille prevents insertion of foreign objects; smooth-finish paint is washable 
and anti-yellowing.

-	Manual de-energize high-limit thermal protection.

Fan	-	Whisper-quiet squirrel-cage fan.
-	Factory-lubricated motor.

Heating element	-	Nickel-chromium alloy stitched wire protected by mica insulators.
	-	Heating element provides instant heat.

Control	-	Line voltage control circuit for thermostat, timer or switch (not included, must be compatible with the selected voltage and wattage of the heater).

Note: If an electronic thermostat is used, it must be compatible with forced-air heaters.

Installation	-	Recommended installation height: max. 10 ft. (3 m), min. 6 ft. (1.8 m).

-	Wattage must be selected at the time of wiring (750W or 1500W).

-	Quick surface-mounted installation, with the included mounting bracket.

-	Flexible cable with electrical wall box cover plate included.

-	Requires installation of a wall electrical box not supplied.

Warranty	-	2-year warranty against defects.

-	5-year warranty on the heating element.

Application 	-	Bathroom, entrance way, stairwell.

Models

| Product # | Qty | Volts | Watts1 | Weight |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | lb | kg |
| # OFB1502BL |  | 120 | 750 1500 | 8.1 | 3.7 |
| OFB1500BL |  | 240 | 750/563 1500/1125 | 8.1 | 3.7 |

1Wattage can be selected on site (750W or 1500W).

Standard color is white.

### Others

| Type | Quantity | Model et description Complementary information | Volts | Phase | Watts | Length mm/in. | Information Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrical Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |