Series

OFH	Floor Insert Fan Heater

Features

Color	-	Standard: bronze, white, almond.

-	Optional (upcharge applicable): metallic silver, metallic charcoal, aluminum,

semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 240/208V, 1-phase.

Construction	-	Welded 20-gauge steel.

-	Sturdy floor grille (14-gauge).

-	Protective screen keeps foreign objects from falling into the unit.

-	High-limit temperature control with manual reset.

Fan	-	Large diameter, quiet fan operating at 43.9 dBA at 80 cfm.

-	Factory-lubricated motor.

Heating element	-	Durable tubular heating element with fins.

Installation	-	Minimum clearance from adjacent walls: 6 in. (15.2 cm).

-	Large wiring compartment.

Warranty	-	5-year warranty against defects.

Application	-	Bay window, near patio door.

Note: If an electronic thermostat is used, it must be compatible with forced-air heaters.

Models

| Product # | Qty | Volts | Watts | Weight |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | lb | kg |
| OFH0502 |  | 120 | 500 | 16.0 | 7.3 |
| OFH1002 |  | 120 | 1000 | 16.0 | 7.3 |
| OFH1502 |  | 120 | 1500 | 16.0 | 7.3 |
| OFH0500 |  | 240/208 | 500/375 | 16.0 | 7.3 |
| OFH1000 |  | 240/208 | 1000/750 | 16.0 | 7.3 |
| OFH1500 |  | 240/208 | 1500/1125 | 16.0 | 7.3 |

Add "BZ" for bronze, "BL" for white, "AM" for almond.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |