- Series

OHB	High Density Aluminum Draft Barrier

- Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

-	Optional: anodized aluminum.

Voltage	-	208V, 240/208V, 277V, 347V, 600V, 50/60 Hz, 1-phase.

Construction	-	14-gauge (0.0787 in. [2 mm]) aluminum front cover.

-	18-gauge satin coat steel cabinet.

-	Screw assembly for added strength.

-	Full length built-in wireway.

-	Separate low voltage wireway (-WW) available.

-	Pencil-proof openings less than 1/4 in. (6.4 mm).

-	Bottom fresh air inlet, top warm air outlet.

-	Clean back available (-BD).

- Linear high-limit temperature control with automatic reset.

- High altitude version available (-HA).

-	End caps included.

Heating element	-	Two stainless steel tubular heating elements with aluminum fins.

-	Floating heating elements on high-temperature nylon bushings

reducing expansion noises.

Watt density	-	Average standard watt density 355W/ft.

- Average low watt density 175W/ft, 250W/ft.

- Average high watt density 500W/ft.

Control	-	Built-in thermostat available with a control knob or tamperproof.

- Mechanical, electronic or pneumatic relays available.

Ouellet highly recommends to use an electronic wall thermostat for greater comfort.

Installation	-	Not for residential use in the United States.

-	Heater must be installed at least 3 in. (76 mm) from ﬁnished ﬂoor.

-	Mounting holes spaced on 2 in. (51 mm) intervals for ease of installation.

-	Installation on wall or ﬂoor mounted with pedestals.

-	Pedestal kit available.

-	Variety of accessories available for wall-to-wall installation.

Warranty	-	3-year warranty against defects.

Applications	-	Commercial building, office building, hospital, daycare center, private residence.

- Average Standard Watt Density Models 355W/ft.

| Watts | Watts | Product # |  |  |  |  |  |  |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 240/208V1 | 208V | Qty | 240/208V | Qty | 347V | Qty | 600V | Qty | in. | mm | lb | kg |
| 750 | 750/563 | OHB0758 |  | OHB0750 |  | OHB0757 |  | - | - | 23 5/8 | 600 | 10.7 | 4.8 |
| 1000 | 1000/750 | OHB1008 |  | OHB1000 |  | OHB1007 |  | - | - | 35 7/16 | 900 | 14.1 | 6.4 |
| 1400 | 1400/1050 | OHB1408 |  | OHB1400 |  | OHB1407 |  | OHB1406 |  | 47 1/4 | 1200 | 17.6 | 8.0 |
| 1750 | 1750/1313 | OHB1758 |  | OHB1750 |  | OHB1757 |  | OHB1756 |  | 59 1/16 | 1500 | 21.1 | 9.6 |
| 2100 | 2100/1577 | OHB2108 |  | OHB2100 |  | OHB2107 |  | OHB2106 |  | 70 7/8 | 1800 | 24.6 | 11.1 |
| 2450 | 2450/1840 | OHB2458 |  | OHB2450 |  | OHB2457 |  | OHB2456 |  | 82 11/16 | 2100 | 28.0 | 12.7 |
| 2800 | 2800/2103 | OHB2808 |  | OHB2800 |  | OHB2807 |  | OHB2806 |  | 94 1/2 | 2400 | 31.5 | 14.3 |

- Average Low Watt Density Models

| Product # | Length | Weight | Average Density W/ft. | Watts | Watts 240/208V | 208V | Qty | 240/208V1 | Qty | 347V | Qty | 600V | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OHB037X-600 | 23 5/8" (600 mm) | 10.7 lb (4.8 kg) | 175 | 375 | 375/281 |  |  |  |  | - | - | - | - |
| OHB050X-600 |  |  | 250 | 500 | 500/375 |  |  |  |  | - | - | - | - |
| OHB050X-900 | 35 7/16" (900 mm) | 14.1 lb (6.4 kg) | 175 | 500 | 500/375 |  |  |  |  |  |  | - | - |
| OHB075X-900 |  |  | 250 | 750 | 750/563 |  |  |  |  |  |  | - | - |
| OHB070X-1200 | 47 1/4" (1200 mm) | 17.6 lb (8.0 kg) | 175 | 700 | 700/525 |  |  |  |  |  |  |  |  |
| OHB100X-1200 |  |  | 250 | 1000 | 1000/750 |  |  |  |  |  |  |  |  |
| OHB087X-1500 | 59 1/16" (1500 mm) | 21.1 lb (9.6 kg) | 175 | 875 | 875/656 |  |  |  |  |  |  |  |  |
| OHB125X-1500 |  |  | 250 | 1250 | 1250/938 |  |  |  |  |  |  |  |  |
| OHB105X-1800 | 70 7/8" (1800 mm) | 24.6 lb (11.1 kg) | 175 | 1050 | 1050/788 |  |  |  |  |  |  |  |  |
| OHB150X-1800 |  |  | 250 | 1500 | 1500/1125 |  |  |  |  |  |  |  |  |
| OHB122X-2100 | 82 11/16" (2100 mm) | 28.0 lb (12.7 kg) | 175 | 1225 | 1225/919 |  |  |  |  |  |  |  |  |
| OHB175X-2100 |  |  | 250 | 1750 | 1750/1313 |  |  |  |  |  |  |  |  |
| OHB140X-2400 | 94 1/2" (2400 mm) | 31.5 lb (14.3 kg) | 175 | 1400 | 1400/1050 |  |  |  |  |  |  |  |  |
| OHB200X-2400 |  |  | 250 | 2000 | 2000/1500 |  |  |  |  |  |  |  |  |

- Average High Watt Density Models 500W/ft.

| Watts | Watts 240/208V1 | Product # |  |  |  |  |  |  |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 208V | Qty | 240/208V | Qty | 347V | Qty | 600V | Qty | in. | mm | lb | kg |
| 1000 | 1000/750 | OHBH1008 |  | OHBH1000 |  | OHBH1007 |  | OHBH1006 |  | 23 5/8 | 600 | 10.7 | 4.8 |
| 1500 | 1500/1125 | OHBH1508 |  | OHBH1500 |  | OHBH1507 |  | OHBH1506 |  | 35 7/16 | 900 | 14.1 | 6.4 |
| 2000 | 2000/1500 | OHBH2008 |  | OHBH2000 |  | OHBH2007 |  | OHBH2006 |  | 47 1/4 | 1200 | 17.6 | 8.0 |
| 2500 | 2500/1875 | OHBH2508 |  | OHBH2500 |  | OHBH2507 |  | OHBH2506 |  | 59 1/16 | 1500 | 21.1 | 9.6 |
| 3000 | 3000/2250 | OHBH3008 |  | OHBH3000 |  | OHBH3007 |  | OHBH3006 |  | 70 7/8 | 1800 | 24.6 | 11.1 |
| 3500 | 3500/2625 | OHBH35083 |  | OHBH3500 |  | OHBH3507 |  | OHBH3506 |  | 82 11/16 | 2100 | 28.0 | 12.7 |
| 4000 | 4000/3000 | OHBH40083 |  | OHBH40003 |  | OHBH4007 |  | OHBH4006 |  | 94 1/2 | 2400 | 31.5 | 14.3 |

1 208V = 75% of wattage at 240V.

2 X = 8 (208V), 0 (240/208V), 7 (347V), 6 (600V).

3 If the control option "24V electronic relay" is required, these products need two electronic relays.

 = Available.

Add "BL" for white, "AM" for almond.

Other colors, accessories and finishes are available upon request. Please contact your customer service representative.

Other voltages available upon request.

- Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| Finish |  |  |  |
| - | BD1 | Clean back |  |
| KIT-OHB-CE | - | Outside corner, 8 in. (203 mm) |  |
| KIT-OHB-CI | - | Inside corner, 8 in. (203 mm) |  |
| KIT-OHB-JT | - | Splice plate, 2 in. (51 mm) |  |
| KIT-OHB-JT-PE | - | Contour splice plate for BD option and/or heater with pedestals, 2 in. (51 mm) |  |
| KIT-OHB-PEC | - | Square-shaped pedestal kit (2), 3 in. (76 mm) |  |
| OHB-SV-XX | - | Empty cabinet with length of XX mm (minimum: 154 mm, maximum: 2400 mm) |  |
| Controls |  |  |  |
| KIT-OHB-DIS203 | DIS203 | Disconnect switch, 20A at 277V max. |  |
| KIT-OHB-DIS403 | DIS403 | Disconnect switch, 40A at 600V max. |  |
| - | HA | Linear high-limit temperature control for high altitude (3000 ft. to 8200 ft.) |  |
| - | PM | Pneumatic relay, 20A at 600V max. |  |
| KIT-OHB-R-347 | R | Single-pole relay, coil 24V, without transformer: 22A at 208V, 240V - 19A at 277V - 18A at 347V |  |
| KIT-OHB-R-600 | R | Single-pole relay, coil 24V, without transformer, 10A at 600V |  |
| KIT-OHB-RT-208 | RT | Single-pole relay, coil 24V, with transformer 208V/24V, 22A at 208V |  |
| KIT-OHB-RT-240 | RT | Single-pole relay, coil 24V, with transformer 240V/24V, 22A at 240V |  |
| KIT-OHB-RT-277 | RT | Single-pole relay, coil 24V, with transformer 277V/24V, 19A at 277V |  |
| KIT-OHB-RT-347 | RT | Single-pole relay, coil 24V, with transformer 347V/24V, 18A at 347V |  |
| KIT-OHB-RT-6002 | RT2 | Single-pole relay, coil 24V, with transformer 600V/24V, 10A at 600V (2-wire only) |  |
| KIT-OHB-T | T | Single-pole thermostat: 25A at 208V, 240V - 10A at 277V, 347V, 480V, 600V |  |
| KIT-OHB-T24 | T24 | Double-pole thermostat: 30A at 208V, 240V - 15A at 277V, 347V, 600V |  |
| KIT-OHB-T-AV | TAV | Single-pole tamperproof thermostat: 25A at 208V, 240V - 10A at 277V, 347V, 480V, 600V |  |
| KIT-OHB-T2-AV4 | T2AV4 | Double-pole tamperproof thermostat: 30A at 208V, 240V - 15A at 277V, 347V, 600V |  |
| KIT-OHB-TRIAC-347 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 15A at 347V max. |  |
| KIT-OHB-TRIAC-347T | TRIACT | Single-pole electronic relay, coil 24V, with transformer: 15A at 208V, 240V, 347V |  |
| KIT-OHB-TRIAC-600 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 6A at 600V max. |  |
| - | WW | 24V wireway includes high temperature wiring |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 For anodized aluminum heaters, the clean back option will be fixed with visible rivets.

2  These 2-wire relays with transformer can't supply a 3-wire low voltage thermostats.

3 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

4 "Off" position.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |