Series

OHV-RF	Hard Wire Unit Heater

Features

Color	-	Standard: almond and black.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	240/208V, 1 phase.

Construction	-	Positive off, double-pole, single-throw thermostat.

-	Fan on/auto switch.

-	Two level finger-tip high/low heat selector switch.

-	Automatic reset over temperature protector.

-	Specially designed outlet diffuser provides maximum air velocity

and prevents foreign objects from entering heater.

-	18 and 20 gauge steel.

-	Motor - totally enclosed - thermally protected.

-	Draw-through design permits greater heat transfer.

Installation	-	Minimum mounting height of 6 ft. (1.8 m).

-	Maximum mounting height of 10 ft. (3 m).

-	Versatile bracket for ceiling or wall mounting.

-	Adjustable bracket allows all angle rotation.

-	Large and easily accessible wiring compartment.

Note: We recommend installing this product on an outside ceiling or wall to avoid vibrations spreading to an adjacent room.

Warranty	-	1-year warranty against defects.

Application 	-	Factory, garage, school.

Model

| Product #1 | Volts | Phase | kW | Amp. | Air Flow |  | Temperature Rise |  | Weight |  | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | fpm | cfm | °C | °F | lb | kg |  |
| OHVU05000AMRF-F1-HL-T5 | 240/208 | 1 | 5/3.75 | 20.8/18 | 800 | 310 | 28 | 51 | 42 | 19 |  |

Standard color is almond and black.

1 Units come standard with hi-lo heat setting, fan only switch,integral thermostat and combination wall/ceiling mounting bracket.

Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date : |  |
| Contact in Ouellet: |  |
| Comments: |  |