- Series

OHY	Convection and Fan Heater

- Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	240V, 1-phase.

Construction	-	Cast aluminum front cover.

-	Steel bar grilles prevent the insertion of foreign objects;

smooth-ﬁnish paint is washable and anti-yellowing.

-	Automatic reset safety device.

-	Built-in white LED night light.

Fan	-	Whisper-quiet squirrel-cage fan.

-	Factory-lubricated motor.

Heating element	-	Helical coil nickel-chromium alloy wire protected by ceramic insulators.

-	Heating element provides instant heat.

Control	-	Built-in electronic digital thermostat/timer.

- Thermostat mode: pulse modulation: automatic convection

to fan-forced switch-over at 1000W.

-	Timer mode: 5-30 minutes full power fan-forced boost.

Installation	-	4 in. (102 mm) minimum clearance to ﬂoor.

-	May be surface-mounted or recessed in 2 in. X 3 in. stud partitions.

-	Mounting holes for ease of installation.

-	Large wiring compartment.

Warranty	-	5-year warranty against defects.

Application	-	Bathroom, kitchen, family room, entrance way.

- Models

| Product # | Qty | Volts | Watts | Weight |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | lb | kg |
| OHY2000-TH1 |  | 240 | 2000 | 22.75 | 10.3 |

- Option

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| KIT-OHY-DIS2 | DIS2 | Disconnect switch, 20A at 250V |  |

* For factory installed options, add the option number to the product number.  See the Product Code section inside the front cover.

1BEWARE: Not available for 208V.

2 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity

of the disconnect.

Add"BL" for white, "AM" for almond.

Electronic thermostat available in grey only.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |