Series

OKB	Infrared Radiant Heater

Features

Finish	-	Anodized aluminum.

Voltage	-	120V, 208V, 240V, 480V, 600V, 1-phase.

-	Other voltages available upon request.

Construction	-	No visible light is emitted by the heater.

-	Built-in adjustable directional system.

Heating element	-	Incoloy stainless steel tubular heating element with 60° standard radiation angle (optional 45°).

-	Quartz element with 60° standard radiation angle.

Installation	-	Vertical or horizontal mounting for stainless steel element models.

-	Horizontal mounting for Quartz element models.

-	Surface-mounted to connection box (on non-combustible surface) 
or suspended by chains and eyelets (chains not included).

-	Must be connected only to a circuit protected by suitable ground fault protection.

-	90 °C (194 °F) wiring is sufficient.

-	Recommended installation height: 8 to 10 ft. (2.4 to 3 m).

Warranty	-	1-year warranty against defects.

Application	-	Factory, warehouse, workshop, chruch.

Models

| Watts |  | Product # |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Over-All Length |  |  |  | Heating Section |  |  |  | Weight |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120V | Qty |  |  | 208V | Qty |  | 240V |  | Qty |  | 600V |  | Qty |  | in. |  | mm |  | in. |  | mm |  | lb |  | kg |  |
| Incoloy stainless steel tubular heating element with 60° standard radiation angle (optional 45°)1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 950 | OKB299C611 |  |  |  |  | OKB299C621 |  |  |  | OKB299C631 |  |  |  | - |  | - |  | 31 1/2 |  | 800 |  | 20 |  | 508 |  | 5,1 |  | 2,3 |
| 1500 | OKB411C611 |  |  |  |  | OKB411C621 |  |  |  | OKB411C631 |  |  |  | - |  | - |  | 43 1/2 |  | 1105 |  | 32 |  | 813 |  | 6,8 |  | 3,1 |
| 1900 | OKB471C611 |  |  |  |  | OKB471C621 |  |  |  | OKB471C631 |  |  |  | - |  | - |  | 49 1/2 |  | 1257 |  | 38 |  | 965 |  | 8,0 |  | 3,6 |
| 2000 | - |  |  |  | - | OKB412C621 |  |  |  | OKB412C631 |  |  |  | OKB412C681 |  |  |  | 43 1/2 |  | 1105 |  | 32 |  | 813 |  | 7,2 |  | 3,3 |
| 2300 | - |  |  |  | - | OKB472C621 |  |  |  | OKB472C631 |  |  |  | OKB472C681 |  |  |  | 49 1/2 |  | 1257 |  | 38 |  | 965 |  | 8,1 |  | 3,7 |
| 3000 | - |  |  |  | - | OKB593C621 |  |  |  | OKB593C631 |  |  |  | OKB593C681 |  |  |  | 61 1/2 |  | 1562 |  | 50 |  | 1270 |  | 9,9 |  | 4,5 |
| 3800 | - |  |  |  | - | OKB713C621 |  |  |  | OKB713C631 |  |  |  | OKB713C681 |  |  |  | 73 1/2 |  | 1867 |  | 62 |  | 1575 |  | 11,7 |  | 5,3 |
| Quartz element with 60° standard radiation angle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 800 | OKB258T611 |  |  |  |  | OKB258T621 |  |  |  | OKB258T631 |  |  |  | - |  | - |  | 271/2 |  | 800 |  | 16 |  | 406 |  | 4,9 |  | 2,2 |
| 1600 | OKB411T611 |  |  |  |  | OKB411T621 |  |  |  | OKB411T631 |  |  |  | - |  | - |  | 43 ½ |  | 1105 |  | 32 |  | 813 |  | 7,1 |  | 3,2 |
| 2500 | - |  |  | - |  | OKB592T621 |  |  |  | OKB592T631 |  |  |  | OKB592T681 |  |  |  | 611/2 |  | 1562 |  | 50 |  | 1270 |  | 9,7 |  | 4,4 |
| 3100 | - |  |  | - |  | OKB713T621 |  |  |  | OKB713T631 |  |  |  | OKB713T681 |  |  |  | 721/2 |  | 1867 |  | 62 |  | 1575 |  | 11,3 |  | 5,1 |

Options

| Product # Kit | ## Description | Qty |
| --- | --- | --- |
| WGA040 | Wire guard for heated section of 16’’ (406 mm) |  |
| WGA050 | Wire guard for heated section of 20’’ (508 mm) |  |
| WGA081 | Wire guard for heated section of 32’’ (813 mm) |  |
| WGA096 | Wire guard for heated section of 38’’ (965 mm) |  |
| WGA127 | Wire guard for heated section of 50’’ (1270 mm) |  |
| WGA157 | Wire guard for heated section of 62’’ (1575 mm) |  |

145° spread available upon request. Remember to change the 6 for 4 when ordering. Example : OKB299C621 becomes OKB299C421.

Other voltages available upon request.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |