Series

OLA	High Density Architectural Cabinet Convector

Features

Color	-	Standard: white, almond.

- Optional (upcharge available): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

- Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	208V, 240V, 277V, 347V, 600V, 1 or 3-phase.

Construction	-	20-gauge steel cabinet.

-	16-gauge steel front cover.

-	Extruded aluminum bar grilles prevent insertion of foreign objects.

-	Front fresh air inlet; top warm air outlet.

- Front warm air outlet  available (-FR).

-	Clean back available (-BD).

-	Linear high-limit temperature control with automatic reset.

Heating element	-	Three stainless steel tubular heating elements with aluminum ﬁns.

-	Floating heating elements on high-temperature bushings,

reducing expansion noises.

Watt density	-	Average standard watt density of 1000W/ft.

-	Average low watt density of 500W/ft., 750W/ft.

Control	-	Built-in tamperproof thermostat available.

- Mechanical, electronic or pneumatic relays available.

Note: Single-pole thermostat is for 1 phase model only.

Ouellet highly recommends to use an electronic wall thermostat for greater comfort.

Installation	-	Not for residential use in the United States.

-	Wall or ﬂoor mounted installation.

-	Front outlet -FR models may be recessed.

-	Mounting holes spaced on 2 in. (51 mm) intervals for ease of installation.

-	Variety of accessories available for wall-to-wall installation.

-	Pedestal kit or sub-base available.

Warranty	-	1-year warranty against defects.

Application	-	Commercial building, hospital, daycare center.

Standard Watt Density Models

| Watts | Watts 240/208V | Product # |  |  |  |  |  |  |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 208V (1 ph) | Qty | 208V (3 ph) | Qty | 240/208V | Qty | 277V | Qty | in. | mm | lb | kg |
| 2000 | 2000/1500 | OLA2008 |  | OLI20381 |  | OLA2000 |  | OLA2077 |  | 27 3/16 | 690 | 37.0 | 16.8 |
| 3000 | 3000/2250 | OLA3008 |  | OLI30381 |  | OLA3000 |  | OLA3077 |  | 37 3/16 | 944 | 47.0 | 21.3 |
| 4000 | 4000/3000 | OLI40081 |  | OLI40381 |  | OLI40001 |  | OLA4077 |  | 47 3/16 | 1198 | 57.0 | 25.9 |
| 5000 | 5000/3750 | OLI50082 |  | OLI50381 |  | OLI50001 |  | OLI50771 |  | 57 3/16 | 1452 | 67.0 | 30.4 |
| 6000 | 6000/4500 | OLI60082 |  | OLI60382 |  | OLI60002 |  | OLI60771 |  | 65 3/16 | 1655 | 79.0 | 35.8 |

Average Low Watt Density Models

| Product #3 | Length | Weight | AverageDensity W/ft. | Watts | 208V (1 ph) | Qty | 208V (3 ph) | Qty | 240V | Qty | 277V | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OLA10XX-690 | 27 3/16" (690 mm) | 37.0 lb (16.8 kg) | 500 | 1000 |  |  |  |  |  |  |  |  |
| OLA15XX-690 |  |  | 750 | 1500 |  |  |  |  |  |  |  |  |
| OLA15XX-944 | 37 3/16" (944 mm) | 47.0 lb (21.3 kg) | 500 | 1500 |  |  |  |  |  |  |  |  |
| OLA22XX-944 |  |  | 750 | 2250 |  |  |  |  |  |  |  |  |
| OLA20XX-1198 | 47 3/16" (1198 mm) | 57.0 lb (25.9 kg) | 500 | 2000 |  |  |  |  |  |  |  |  |
| OLA30XX-1198 |  |  | 750 | 3000 |  |  |  |  |  |  |  |  |
| OLA25XX-1452 | 57 3/16" (1452 mm) | 67.0 lb (30.4 kg) | 500 | 2500 |  |  |  |  |  |  |  |  |
| OLA37XX-1452 |  |  | 750 | 3750 |  |  |  |  |  |  |  |  |
| OLA30XX-1655 | 65 3/16" (1655 mm) | 79.0 lb (35.8 kg) | 500 | 3000 |  |  |  |  |  |  |  |  |
| OLA45XX-1655 |  |  | 750 | 4500 |  |  |  |  |  |  |  |  |

1 If the control option "24V electronic relay" is required, these products need two electronic relays.

2 If the control option "24V electronic relay" is required, these products need three electronic relays.

3 XX = 08 (208V, 1 ph), 38 (208V, 3 ph), 00 (240V), 30 (240V, 3PH), 77 (277V), 07 (347V), 04 (480V, 1PH), 34 (480V, 3PH), 06 (600V, 1 ph), 36 (600V, 3 ph).

 = Available.

Add "BL" for white, "AM" for almond.

Other colors, accessories and finishes are available upon request. Please contact your customer service representative.

Other voltages available upon request.

- Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| Finish |  |  |  |
| - | BD | Clean back |  |
| KIT-OLA-CI | - | Corner (Inside and outside) |  |
| KIT-OLA-CR-27 | - | Recess trim kit, 27 3/16 in. (690 mm) |  |
| KIT-OLA-CR-37 | - | Recess trim kit, 37 3/16 in. (944 mm) |  |
| KIT-OLA-CR-47 | - | Recess trim kit, 47 3/16 in. (1198 mm) |  |
| KIT-OLA-CR-57 | - | Recess trim kit, 57 3/16 in. (1452 mm) |  |
| KIT-OLA-CR-65 | - | Recess trim kit, 65 3/16 in. (1655 mm) |  |
| KIT-OLA-CTR | - | Relay box, 12 in. (305 mm) |  |
| - | FR | Front warm air outlet |  |
| KIT-OLA-PEC | - | Square-shaped pedestal kit (2), 3 in. (76 mm) |  |
| KIT-OLA-SO-27 | - | Sub-base, 27 3/16 in. (690 mm) |  |
| KIT-OLA-SO-37 | - | Sub-base, 37 3/16 in. (944 mm) |  |
| KIT-OLA-SO-47 | - | Sub-base, 47 3/16 in. (1198 mm) |  |
| KIT-OLA-SO-57 | - | Sub-base, 57 3/16 in. (1452 mm) |  |
| KIT-OLA-SO-65 | - | Sub-base, 65 3/16 in. (1655 mm) |  |
| KIT-OLA-SV-27 | - | Empty cabinet, 27 3/16 in. (690 mm) |  |
| KIT-OLA-SV-37 | - | Empty cabinet, 37 3/16 in. (944 mm) |  |
| KIT-OLA-SV-47 | - | Empty cabinet, 47 3/16 in. (1198 mm) |  |
| KIT-OLA-SV-57 | - | Empty cabinet, 57 3/16 in. (1452 mm) |  |
| KIT-OLA-SV-65 | - | Empty cabinet, 65 3/16 in. (1655 mm) |  |
| Controls |  |  |  |
| - | CTT | Single-pole relay, coil 240V: 25A at 240V, 10A at 600V |  |
| - | CTT2 | Double-pole relay, coil 240V: 25A at 240V, 10A at 600V |  |
| - | DIS403 | Disconnect switch, triple-pole, 40A at 600V |  |
| - | PM | Pneumatic relay, 20A at 600V |  |
| KIT-OLA-R | R | Single-pole relay, coil 24V, without transformer: 22A at 208V, 240V - 18A at 347V |  |
| KIT-OLA-R2 | R2 | Double-pole relay, coil 24V, without tranformer: 25A at 240V - 10A at 600V |  |
| KIT-OLA-R2-30 | R230 | Double-pole relay, coil 24V, without transformer, 30A at 600V max. |  |
| KIT-OLA-RT-208 | RT | Single-pole relay, coil 24V, with transformer 208V/24V, 4500W, 22A at 208V |  |
| KIT-OLA-RT-240 | RT | Single-pole relay, coil 24V, with transformer 240V/24V, 5200W, 22A at 240V |  |
| KIT-OLA-RT-277 | RT | Single-pole relay, coil 24V, with transformer 277V/24V, 5200W, 19A at 277V |  |
| KIT-OLA-RT-347 | RT | Single-pole relay, coil 24V, with transformer 347V/24V, 6000W, 18A at 347V |  |
| KIT-OLA-RT-6001 | RT1 | Single-pole relay, coil 24V, with transformer 600V/24V, 6000W, 10A at 600V (2-wire only) |  |
| KIT-OLA-T-AV2 | TAV2 | Single-pole tamperproof thermostat: 30A at 250V max. - 15A at 600V max. |  |
| KIT-OLA-T2-AV | T2AV | Double-pole tamperproof thermostat: 30A at 250V max. - 15A at 600V max. |  |
| - | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 15A at 347V max. |  |
| - | TRIACT | Single-pole electronic relay, coil 24V, with transformer, 15A at 208V, 240V, 347V |  |
| - | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 6A at 600V max. |  |
| Transformers |  |  |  |
| - | TRF24 | Transformer 24V, 50VA |  |
| - | TRF240 | Transformer 240V, 50VA |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 These 2-wire relays with transformer can't supply of 3-wire low voltage thermostats.

2 For 1-phase model only.

3 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

Electronic relay (1 or 3-phase) available upon request.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |