- Series

OMB	Mini Aluminum Draft Barrier

- Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal, aluminum, 
semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

-	Optional: anodized aluminum.

Voltage	-	208V, 240/208V, 277V, 347V, 600V, 50/60 Hz, 1-phase.

Construction	-	14-gauge (0.0787 in. [2 mm]) aluminum front cover.

-	18-gauge satin coat steel cabinet.

-	Screw assembly for added strength.

-	Full length built-in wireway.

-	Separate low voltage wireway (-WW) available.

-	Pencil-proof openings less than 1/4 in. (6.4 mm).

-	Bottom fresh air inlet; top warm air outlet.

-	Clean back available (-BD).

-	Linear high-limit temperature control with automatic reset.

High altitude version available (-HA).

-	End caps included.

Heating element	-	Stainless steel tubular heating element with aluminum fins.

-	Floating heating element on high-temperature nylon bushings

reducing expansion noises.

Watt density	-	Average standard watt density of 175W/ft.

-	Average low watt density of 100W/ft., 150W/ft.

Control	-	Built-in thermostat available with a control knob or tamperproof.

- Mechanical, electronic or pneumatic relays available (installed on the right end only).

Ouellet highly recommends to use an electronic wall thermostat for greater comfort.

Installation	-	Not for residential use in the United States.

-	Heater must be installed at least 2 in. (51 mm) from finished floor.

-	Mounting holes spaced on 4 in. (102 mm) intervals for ease of installation.

-	Installation on wall or ﬂoor mounted with pedestals.

-	Variety of accessories available for wall-to-wall installation.

-	Pedestal kit available.

Warranty	-		1-year warranty against defects.

Application	-	Commercial building, office building, hospital, daycare center private residence.

- Standard Watt Density Models

| Watts | Watts 240/208V1 | Product # |  |  |  |  |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 208V | Qty | 240/208V | Qty | 277V | Qty | in. | mm | lb | kg |
| 350 | 350/263 | OMB0358 |  | OMB0350 |  | OMB0377 | - | 27 9/16 | 700 | 7.0 | 3.2 |
| 525 | 525/394 | OMB0528 |  | OMB0520 |  | OMB0527 |  | 35 7/16 | 900 | 8.3 | 3.8 |
| 700 | 700/526 | OMB0708 |  | OMB0700 |  | OMB0777 |  | 47 1/4 | 1200 | 10.2 | 4.6 |
| 875 | 875/657 | OMB0878 |  | OMB0870 |  | OMB0877 |  | 59 1/16 | 1500 | 12.0 | 5.5 |
| 1050 | 1050/789 | OMB1058 |  | OMB1050 |  | OMB1077 |  | 70 7/8 | 1800 | 13.9 | 6.3 |
| 1225 | 1225/920 | OMB1228 |  | OMB1220 |  | OMB1277 |  | 82 11/16 | 2100 | 15.8 | 7.2 |
| 1400 | 1400/1052 | OMB1408 |  | OMB1400 |  | OMB1477 |  | 94 1/2 | 2400 | 17.6 | 8.0 |

- Low Watt Density Models

| Product #1 | Length | Weight | Average Density W/ft. | Watts | Watts 240/208V1 | 208V | Qty | 240/208V | Qty | 277V | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OMB030X-700 | 27 9/16" (700 mm) | 7.0 lb (3.2 kg) | 150 | 300 | 300/225 |  |  |  |  | ----- | ----- |
| OMB030X-900 | 35 7/16" (900 mm) | 8.3 lb (3.8 kg) | 100 | 300 | 300/225 |  |  |  |  | ----- | ----- |
| OMB045X-900 |  |  | 150 | 450 | 450/338 |  |  |  |  |  |  |
| OMB040X-1200 | 47 1/4" (1200 mm) | 10.2 lb (4.6 kg) | 100 | 400 | 400/300 |  |  |  |  |  |  |
| OMB060X-1200 |  |  | 150 | 600 | 600/450 |  |  |  |  |  |  |
| OMB050X-1500 | 59 1/16" (1500 mm) | 12.0 lb (5.5 kg) | 100 | 500 | 500/375 |  |  |  |  |  |  |
| OMB075X-1500 |  |  | 150 | 750 | 750/563 |  |  |  |  |  |  |
| OMB060X-1800 | 70 7/8" (1800 mm) | 13.9 lb (6.3 kg) | 100 | 600 | 600/450 |  |  |  |  |  |  |
| OMB090X-1800 |  |  | 150 | 900 | 900/675 |  |  |  |  |  |  |
| OMB070X-2100 | 82 11/16" (2100 mm) | 15.8 lb (7.2 kg) | 100 | 700 | 700/525 |  |  |  |  |  |  |
| OMB105X-2100 |  |  | 150 | 1050 | 1050/788 |  |  |  |  |  |  |
| OMB080X-2400 | 94 1/2" (2400 mm) | 17.6 lb (8.0 kg) | 100 | 800 | 800/600 |  |  |  |  |  |  |
| OMB120X-2400 |  |  | 150 | 1200 | 1200/900 |  |  |  |  |  |  |

1 X = 8 (208V), 0 (240/208V), 77 (277V).

 = Available.

Other colors, accessories and finishes are available upon request. Please contact your customer service representative.

Other voltages available upon request.

Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| Finish |  |  |  |
| - | BD3 | Clean back |  |
| KIT-OMB-CE | - | Outside corner, 4 in. (102 mm) |  |
| KIT-OMB-CI | - | Inside corner, 4 in. (102 mm) |  |
| KIT-OMB-JT | - | Splice plate, 2 in. (51 mm) |  |
| KIT-OMB-JT-PE | - | Contour splice plate for BD option and1or heater with pedestals, 2 in. (51 mm) |  |
| KIT-OMB-PE | - | Round pedestal kit (2), 3 in. (76 mm) |  |
| KIT-OMB-PEC | - | Square-shaped pedestal kit (2), 2 in. (51 mm) |  |
| OMB-SV-XX | - | Empty cabinet with length of XX mm (minimum: 154 mm, maximum: 2400 mm) |  |
| Controls |  |  |  |
| KIT-OMB-DIS204 | DIS204 | Disconnect switch, 20A at 277V max. |  |
| - | HA | Linear high-limit temperature control for high altitude (3000 ft. to 8200 ft.) |  |
| - | PM | Pneumatic relay, 20A at 600V max. |  |
| KIT-OMB-R-347 | R | Single-pole relay, coil 24V, without transformer: 22A at 208V, 240V - 19A at 277V - 18A at 347V |  |
| KIT-OMB-R-600 | R | Single-pole relay, coil 24V, without transformer, 10A at 600V |  |
| KIT-OMB-RT-208 | RT | Single-pole relay, coil 24V, with transformer 208V/24V, 22A at 208V |  |
| KIT-OMB-RT-240 | RT | Single-pole relay, coil 24V, with transformer 240V/24V, 22A at 240V |  |
| KIT-OMB-RT-277 | RT | Single-pole relay, coil 24V, with transformer 277V/24V, 19A at 277V |  |
| KIT-OMB-RT-347 | RT | Single-pole relay, coil 24V, with transformer 347V/24V, 18A at 347V |  |
| KIT-OMB-RT-6002 | RT2 | Single-pole relay, coil 24V, with transformer 600V/24V, 10A at 600V (2-wire only) |  |
| KIT-OMB-TB6 | TB6 | Single-pole bi-metal thermostat: 25A at 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-OMB-TB261 | TB261 | Double-pole bi-metal thermostat: 25A at 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-OMB-TB6-AV | TB6AV | Single-pole bi-metal tamperproof thermostat: 25A at 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-OMB-TB26-AV1 | TB26AV1 | Double-pole bi-metal tamperproof thermostat: 25A at 208V, 240V, 277V - 10A at 347V, 480V, 600V |  |
| KIT-OMB-TRIAC-347 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 10A max. at 347V max. |  |
| KIT-OMB-TRIAC-347T | TRIACT | Single-pole electronic relay, coil 24V, with transformer: 10A max. at 208V, 240V, 347V |  |
| KIT-OMB-TRIAC-600 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 6A at 600V max. |  |
| - | WW | 24V wireway |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 "Off" position.

2 These 2-wire relays with transformer can't supply of 3-wire low voltage thermostats.

3 For anodized aluminum heaters, the clean back option will be fixed with visible rivets.

4 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |