Series

ONC	Standard Convector

Features

Color	-	Standard: white.

-	Optional (upcharge applicable): almond, metallic silver, bronze, 	aluminum,

semi-gloss black, metallic charcoal, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 480V, 600V, 60 Hz, 1-phase.

Construction	-	20-gauge steel.

-	Warm air outlet that directs the air to the centre of the room.

-	Bar grilles prevent insertion of foreign objects.

-	High-limit temperature control with automatic reset.

Heating element	-	2 stainless steel tubular heating elements with aluminum fins.

- Floating heating element on high-temperature nylon bushings reducing expansion noises.

Watt density	-	700W/ft. (2320W/m).

Control	-	Built-in non programmable electronic thermostat for the 120V to 240V models (-TH models).

- Wall thermostat (not included).

Installation	-	4 in. (102 mm) minimum clearance to ﬂoor and from an adjacent wall.

-	Quick surface-mounted installation, with the included wall mounting bracket.

Warranty	-	5-year warranty against defects.

Ouellet highly recommends the use of an electronic wall thermostat for greater comfort.

Application	-	Kitchen, living room, family room, bedroom, basement.

Models

| Product # |  |  |  | Volts | Watts | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Without thermostat | Qty | Built-in thermostat | Qty |  |  | po | mm | lb | kg |
| ONC0502BL |  | # ONC0502BL-TH |  | 120 | 500 | 18 | 457 | 13 | 6 |
| ONC1002BL |  | # ONC1002BL-TH |  | 120 | 1000 | 23 9/16 | 598 | 16.7 | 7.6 |
| ONC1502BL |  | # ONC1502BL-TH |  | 120 | 1500 | 29 1/16 | 739 | 20 | 9.12 |
| ONC0508BL |  | # ONC0508BL-TH |  | 208 | 500 | 18 | 457 | 13 | 6 |
| ONC1008BL |  | # ONC1008BL-TH |  | 208 | 1000 | 23 9/16 | 598 | 16.7 | 7.6 |
| ONC1508BL |  | # ONC1508BL-TH |  | 208 | 1500 | 29 1/16 | 739 | 20 | 9.12 |
| ONC2008BL |  | # ONC2008BL-TH |  | 208 | 2000 | 34 5/8 | 880 | 24 | 10.9 |
| ONC0500BL |  | # ONC0500BL-TH |  | 240/208 | 500/375 | 18 | 457 | 13 | 6 |
| ONC1000BL |  | # ONC1000BL-TH |  | 240/208 | 1000/750 | 23 9/16 | 598 | 16.7 | 7.6 |
| ONC1500BL |  | # ONC1500BL-TH |  | 240/208 | 1500/1125 | 29 1/16 | 739 | 20 | 9.12 |
| ONC2000BL |  | # ONC2000BL-TH |  | 240/208 | 2000/1500 | 34 5/8 | 880 | 24 | 10.9 |
| ONC0507BL |  | # - |  | 347 | 500 | 18 | 457 | 13 | 6 |
| ONC1007BL |  | # - |  | 347 | 1000 | 23 9/16 | 598 | 16.7 | 7.6 |
| ONC1507BL |  | # - |  | 347 | 1500 | 29 1/16 | 739 | 20 | 9.12 |
| ONC2007BL |  | # - |  | 347 | 2000 | 34 5/8 | 880 | 24 | 10.9 |
| ONC1006BL |  | # - |  | 600 | 1000 | 23 9/16 | 598 | 16.7 | 7.6 |
| ONC1506BL |  | # - |  | 600 | 1500 | 29 1/16 | 739 | 20 | 9.12 |
| ONC2006BL |  | # - |  | 600 | 2000 | 34 5/8 | 880 | 24 | 10.9 |

Standard color is white.

### Options

| Produit # Kit | Produit # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| KIT-ONC-TRIAC-347 | TRIAC | Single-pole electronic relay, coil 24V, without transformer: 15A at 347V max. |  |
| KIT-ONC-TRIAC-347T | TRIACT | Single-pole electronic relay, coil 24V, with transformer: 15A at 208V, 240V, 277V, 347V |  |
| KIT-ONC-TRIAC-600 | TRIAC | Single-pole electronic relay, coil 24V, without transformer: 6A at 600V max. |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

Other voltages available upon request.

### Others

| Type | Quantity | Model and description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |