Series

OPP	Patio Door Baseboard Heater

Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	208V, 240/208V, 347V, 600V, 1-phase.

Construction	-	So sturdy, it can be used as a step (18-gauge steel).

-	Linear high-limit temperature control with automatic reset.

Heating element	-	Stainless steel tubular heating element with aluminum fins.

-	Floating heating element on high-temperature nylon bushings 
reducing expansion noises.

Watt density	-	Average standard watt density of 209W/ft.

Control	-	Built-in thermostat available with control knob or tamperproof

(installed on the right end only).

- Mechanical or electronic relays available (installed on the right end only).

Ouellet highly recommends using an electronic wall thermostat for greater confort.

Installation	-	Mounting holes spaced on 2 in. (51 mm) intervals for ease of installation.

-	Variety of accessories available for wall-to-wall installation.

-	Pedestal kit available.

Warranty	-	3-year warranty against defects.

-	10-year warranty on the heating element.

Application 	-	Patio door, bay window, commercial building, office building, 
	condo, restaurant.

Average Standard Watt Density Models 209W/ft.

| Watts | Watts 240/208V | Product # |  |  |  |  |  |  |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 208V | Qty | 240/208V | Qty | 347V | Qty | 600V | Qty | in. | mm | lb | kg |
| 500 | 500/375 | OPP0508 |  | OPP0500 |  | OPP0507 |  | - | - | 33 1/2 | 855 | 13.0 | 5.9 |
| 750 | 750/563 | OPP0758 |  | OPP0750 |  | OPP0757 |  | OPP0756 |  | 48 | 1219 | 17.0 | 7.7 |
| 1000 | 1000/750 | OPP1008 |  | OPP1000 |  | OPP1007 |  | OPP1006 |  | 60 | 1525 | 20.0 | 9.1 |
| 1250 | 1250/938 | OPP1258 |  | OPP1250 |  | OPP1257 |  | OPP1256 |  | 72 | 1830 | 26.0 | 11.8 |
| 1500 | 1500/1125 | OPP1508 |  | OPP1500 |  | OPP1507 |  | OPP1506 |  | 86 3/4 | 2204 | 31.0 | 14.1 |
| 1750 | 1750/1313 | OPP1758 |  | OPP1750 |  | OPP1757 |  | OPP1756 |  | 100 | 2542 | 38.0 | 17.3 |
| 2000 | 2000/1500 | OPP2008 |  | OPP2000 |  | OPP2007 |  | OPP2006 |  | 113 5/16 | 2879 | 41.0 | 18.6 |

Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| - | BD | Clean back |  |
| KIT-OPP-CIBR | - | Corner (inside and outside), 4 in. (102 mm). Relay section. |  |
| KIT-OPP-JT | - | Splice plate, 2 in. (51 mm) |  |
| KIT-OPP-JT-PE | - | Contour splice plate for heaters with pedestals, 2 in. (51 mm) |  |
| KIT-OPP-PE | - | Round pedestal kit (2), 3 in. (76 mm) |  |
| KIT-OPP-PEC | - | Square-shaped pedestal kit (2), 2 in. (51 mm) |  |
| KIT-OPP-R-3471 | R1 | Single-pole relay, coil 24V, without transformer: 22A at 208V, 240V - 18A at 347V |  |
| KIT-OPP-R-6001 | R1 | Single-pole relay, coil 24V, without transformer, 10A at 600V |  |
| KIT-OPP-RT-2081 | RT1 | Single-pole relay, coil 24V, with transformer 208V/24V, 22A at 208V |  |
| KIT-OPP-RT-2401 | RT1 | Single-pole relay, coil 24V, with transformer 240V/24V, 22A at 240V |  |
| KIT-OPP-RT-3471 | RT1 | Single-pole relay, coil 24V, with transformer 347V/24V, 18A at 347V |  |
| KIT-OPP-RT-6001,2 | RT1,2 | Single-pole relay, coil 24V, with transformer 600V/24V, 10A at 600V |  |
| OPP-SV-XX | - | Empty cabinet length of XX mm (minimum: 154 mm, maximum: 2879 mm) |  |
| KIT-OPP-TB11 | TB11 | Single-pole thermostat: 30A at 120V, 208V, 240V - 15A at 347V, 600V |  |
| KIT-OPP-TB21,3 | TB21,3 | Double-pole thermostat: 30A at 120V, 208V, 240V - 15A at 347V, 600V |  |
| KIT-OPP-TB1-AV1 | TB1AV1 | Single-pole thermostat, tamperproof: 30A at 120V, 208V, 240V - 15A at 347V, 600V |  |
| KIT-OPP-TB2-AV1,3 | TB2AV1,3 | Double-pole thermostat, tamperproof: 30A at 120V, 208V, 240V - 15A at 347V, 600V |  |
| KIT-OPP-TRIAC-3471 | TRIAC1 | Single-pole electronic relay, coil 24V, without transformer: 10A max. at 347V max. |  |
| KIT-OPP-TRIAC-347T1 | TRIACT1 | Single-pole electronic relay, coil 24V, with transformer: 10A max. at 208V, 240V, 347V |  |
| KIT-OPP-TRIAC-6001 | TRIAC1 | Single-pole electronic relay, coil 24V, without transformer,6A at 600V max. |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 Options installed at right end only.

2 These 2-wire relays with transformer can't supply low voltage thermostats.

3 "Off" position.

Add "BL" for white, "AM" for almond. Upcharge available for optional colors shown in the color chart section.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |