Series

OPPM	Mini Steel Draft Barrier

Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 480V, 600V, 1-phase.

Construction	-	So sturdy, it can be used as a step (18-gauge steel).

-	Linear high-limit temperature control with automatic reset.

Heating element	-	Stainless steel tubular heating element with aluminum fins.

-	Floating heating element on high-temperature nylon bushings 
reducing expansion noises.

Watt density	-	Standard watt density.

Control	-	Built-in thermostat available with control knob or tamperproof

(installed on the left end only).

- Mechanical or electronic relays available (installed on the right end only).

Ouellet highly recommends using an electronic wall thermostat for greater comfort.

Installation	-	Not for residential use in the United States.

- 2 rows of mounting holes, every 2 in. (51 mm), for ease of installation.

- Variety of accessories available for wall-to-wall installation.

- Pedestal kit available.

Warranty	-	1-year warranty against defects.

Application 	-	Patio door, bay window, commercial building, office building, 
	condo, restaurant.

Standard Watt Density Models

| Watts | Watts 240/208V | Product # |  |  |  |  |  |  |  |  |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120V | Qty | 208V | Qty | 240/208V | Qty | 277V | Qty | 480V | Qty | in. | mm | lb | kg |
| 300 | 300/225 | OPPM0302 |  | OPPM0308 |  | OPPM0300 |  | - | - | - | - | 23 5/8 | 600 | 5.3 | 2.4 |
| 450 | 450/338 | OPPM0452 |  | OPPM0458 |  | OPPM0450 |  | OPPM0477 |  | - | - | 35 7/16 | 900 | 9.5 | 4.3 |
| 600 | 600/450 | OPPM0602 |  | OPPM0608 |  | OPPM0600 |  | OPPM0677 |  | - | - | 47 1/4 | 1200 | 12.2 | 5.5 |
| 750 | 750/563 | OPPM0752 |  | OPPM0758 |  | OPPM0750 |  | OPPM0777 |  | OPPM0754 |  | 59 1/16 | 1500 | 15.0 | 6.8 |
| 900 | 900/675 | OPPM0902 |  | OPPM0908 |  | OPPM0900 |  | OPPM0977 |  | OPPM0904 |  | 70 7/8 | 1800 | 17.8 | 8.1 |
| 1050 | 1050/788 | OPPM1052 |  | OPPM1058 |  | OPPM1050 |  | OPPM1077 |  | OPPM1054 |  | 82 11/16 | 2100 | 20.6 | 9.3 |
| 1200 | 1200/900 | OPPM1202 |  | OPPM1208 |  | OPPM1200 |  | OPPM1277 |  | OPPM1204 |  | 94 1/2 | 2400 | 23.4 | 10.6 |
| 1350 | 1350/1013 | OPPM1352 |  | OPPM1358 |  | OPPM1350 |  | OPPM1377 |  | OPPM1354 |  | 106 5/16 | 2700 | 26.2 | 11.9 |

Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| - | BD | Clean back |  |
| KIT-OPPM-CE | - | Outside corner, 3 in. (76 mm) |  |
| KIT-OPPM-CI | - | Inside corner, 3 in. (76 mm) |  |
| KIT-OPPM-JT | - | Splice plate, 2 in. (51 mm) |  |
| KIT-OPPM-JT-PE | - | Contour splice plate for BD option and/or heater with pedestals, 2 in. (51 mm) |  |
| KIT-OPPM-PE | - | Round pedestal kit (2), 3 in. (76 mm) |  |
| KIT-OPPM-PEC | - | Square-shaped pedestal kit (2), 2 in. (51 mm) |  |
| KIT-OPPM-R-3471 | R1 | Single-pole relay, coil 24V, without transformer: 22A at 120V, 208V, 240V - 19A at 277V - 18A at 347V |  |
| KIT-OPPM-R-6001 | R1 | Single-pole relay, coil 24V, without transformer: 10A at 480V, 600V |  |
| KIT-OPPM-RT-2081 | RT1 | Single-pole relay, coil 24V, with transformer 208V/24V, 22A at 208V |  |
| KIT-OPPM-RT-2401 | RT1 | Single-pole relay, coil 24V, with transformer 240V/24V, 22A at 240V |  |
| KIT-OPPM-RT-2771 | RT1 | Single-pole relay, coil 24V, with transformer 277V/24V, 19A at 277V |  |
| KIT-OPPM-RT-3471 | RT1 | Single-pole relay, coil 24V, with transformer 347V/24V, 18A at 347V |  |
| OPPM-SV-XX | - | Empty cabinet length of XX mm (minimum : 154 mm, maximum : 2700 mm) |  |
| KIT-OPPM-TB62 | TB62 | Single-pole bi-metal thermostat: 25A at 120V, 208V, 240V - 17A at 277V - 10A at 347V, 480V, 600V |  |
| KIT-OPPM-TB262,3 | TB262,3 | Double-pole bi-metal thermostat: 25A at 120V, 208V, 240V - 17A at 277V - 10A at 347V, 480V, 600V |  |
| KIT-OPPM-TB6-AV2 | TB6AV2 | Single-pole bi-metal tamperproof thermostat: 25A at 120V, 208V, 240V - 17A at 277V - 10A at 347V, 480V, 600V |  |
| KIT-OPPM-TB26-AV2,3 | TB26AV2,3 | Double-pole bi-metal tamperproof thermostat: 25A at 120V, 208V, 240V - 17A at 277V - 10A at 347V, 480V, 600V |  |
| KIT-OPPM-TRIAC-3471 | TRIAC1 | Single-pole electronic relay, coil 24V, without transformer: 10A max. (except for 1350W model: 11.5A max.) at 347V max. |  |
| KIT-OPPM-TRIAC-347T1 | TRIACT1 | Single-pole electronic relay, coil 24V, with transformer: 10A max. (except for 1350W model: 11.5A max.) at 208V, 240V, 347V |  |
| KIT-OPPM-TRIAC-6001 | TRIAC1 | Single-pole electronic relay, coil 24V, with transformer: 6A at 600V max. |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 Options installed at right end only.

2 Options installed at left end only.

3 "Off" position.

Add "BL" for white, "AM" for almond. Upcharge available for optional colors shown in the color chart section.

Other voltages available upon request.

### Others

| Type | Quantity | Model et description Complementary information | Volts | Phase | Watts | Length mm/in. | Information Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |