Series

OPR	Heavy-Duty Steel Baseboard Heater

Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal, 
aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 208V, 240/208V, 277V, 347V, 600V, 1-phase.

Construction	-	16-gauge steel front.

-	18-gauge steel cabinet.

-	Full length built-in wireway.

-	Interior parts protected by a louvre grille.

-	Pencil-proof openings less than 1/4 in. (6.4 mm).

-	Clean back available (-BD).

-	Linear high-limit temperature control with automatic reset.

Heating element	-	Stainless steel tubular heating element with aluminum ﬁns.

-	Floating heating element on high-temperature nylon bushings 
reducing expansion noises.

Watt density	-	Average standard watt density of 275W/ft.

-	Average low watt density of 100W/ft., 150W/ft., 200W/ft.

Control	-	Built-in thermostat available with a control knob

or tamperproof (installed on the left end only).

-	Mechanical or electronic relays available (installed on the left end only).

Ouellet highly recommends to use an electronic wall thermostat for greater comfort.

Installation	-	Wall or ﬂoor mounted installation.

-	Mounting holes spaced at 2 in. (51 mm) intervals for ease of installation.

-	Variety of accessories available for wall-to-wall installation.

-	Pedestal kit available.

Warranty	-	3-year warranty against defects.

Application	-	Commercial building, office building, hospital, daycare center,

private residence, penal institution.

Average Standard Watt Density Models 275W/ft.

| Watts | Watts 240/208V1 | Product # |  |  |  |  |  |  |  |  |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120V | Qty | 208V | Qty | 240/208V | Qty | 347V | Qty | 600V | Qty | in. | mm | lb | kg |
| 300 | 300/225 | OPR0302 |  | OPR0308 |  | OPR0300 |  | - | - | - | - | 20 9/16 | 522 | 10.4 | 4.7 |
| 500 | 500/375 | OPR0502 |  | OPR0508 |  | OPR0500 |  | OPR0507 |  | - | - | 25 1/2 | 647 | 14.0 | 6.4 |
| 750 | 750/563 | OPR0752 |  | OPR0758 |  | OPR0750 |  | OPR0757 |  | OPR0756 |  | 35 1/2 | 901 | 20.0 | 9.1 |
| 1000 | 1000/750 | OPR1002 |  | OPR1008 |  | OPR1000 |  | OPR1007 |  | OPR1006 |  | 45 3/4 | 1162 | 26.0 | 11.8 |
| 1250 | 1250/938 | OPR1252 |  | OPR1258 |  | OPR1250 |  | OPR1257 |  | OPR1256 |  | 55 1/4 | 1403 | 32.0 | 14.6 |
| 1500 | 1500/1125 | OPR1502 |  | OPR1508 |  | OPR1500 |  | OPR1507 |  | OPR1506 |  | 63 15/16 | 1624 | 37.0 | 16.8 |
| 1750 | 1750/1313 | - | - | OPR1758 |  | OPR1750 |  | OPR1757 |  | OPR1756 |  | 73 1/4 | 1861 | 43.0 | 19.6 |
| 2000 | 2000/1500 | - | - | OPR2008 |  | OPR2000 |  | OPR2007 |  | OPR2006 |  | 82 1/16 | 2084 | 47.5 | 21.6 |
| 2250 | 2250/1688 | - | - | OPR2258 |  | OPR2250 |  | OPR2257 |  | OPR2256 |  | 90 15/16 | 2309 | 53.5 | 24.3 |
| 2500 | 2500/1875 | - | - | OPR2508 |  | OPR2500 |  | OPR2507 |  | OPR2506 |  | 100 | 2541 | 58.4 | 26.6 |

Average Low Watt Density Models

| Product #2 | Length | Weight | Average Density W/pi | Watts | Watts 240/208V1 | 120V | Qty | 208V | Qty | 240/ 208V | Qty | 347V | Qty | 600V | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OPR020X-647 | 25 1/2" (647 mm) | 14.0 lb (6.4 kg) | 100 | 200 | 200/150 |  |  |  |  |  |  | ----- | ----- | ----- | ----- |
| OPR030X-647 |  |  | 150 | 300 | 300/225 |  |  |  |  |  |  | ----- | ----- | ----- | ----- |
| OPR040X-647 |  |  | 200 | 400 | 400/300 |  |  |  |  |  |  |  |  | ----- |  |
| OPR030X-901 | 35 1/2" (901 mm) | 20.0 lb (9.1 kg) | 100 | 300 | 300/225 |  |  |  |  |  |  |  |  | ----- | ----- |
| OPR045X-901 |  |  | 150 | 450 | 450/338 |  |  |  |  |  |  |  |  | ----- | ----- |
| OPR060X-901 |  |  | 200 | 600 | 600/450 |  |  |  |  |  |  |  |  | ----- | ----- |
| OPR040X-1162 | 45 3/4" (1162 mm) | 26.0 lb (11.8 kg) | 100 | 400 | 400/300 |  |  |  |  |  |  |  |  | ----- | ----- |
| OPR060X-1162 |  |  | 150 | 600 | 600/450 |  |  |  |  |  |  |  |  | ----- | ----- |
| OPR080X-1162 |  |  | 200 | 800 | 800/600 |  |  |  |  |  |  |  |  |  |  |
| OPR050X-1403 | 55 1/4" (1403 mm) | 32.0 lb (14.6 kg) | 100 | 500 | 500/375 |  |  |  |  |  |  |  |  | ----- | ----- |
| OPR075X-1403 |  |  | 150 | 750 | 750/563 |  |  |  |  |  |  |  |  |  |  |
| OPR100X-1403 |  |  | 200 | 1000 | 1000/750 |  |  |  |  |  |  |  |  |  |  |
| OPR060X-1624 | 63 15/16" (1624 mm) | 37.0 lb (16.8 kg) | 100 | 600 | 600/450 |  |  |  |  |  |  |  |  |  |  |
| OPR090X-1624 |  |  | 150 | 900 | 900/675 |  |  |  |  |  |  |  |  |  |  |
| OPR120X-1624 |  |  | 200 | 1200 | 1200/900 |  |  |  |  |  |  |  |  |  |  |
| OPR070X-1861 | 73 1/4" (1861 mm) | 43.0 lb (19.6 kg) | 100 | 700 | 700/525 |  |  |  |  |  |  |  |  |  |  |
| OPR105X-1861 |  |  | 150 | 1050 | 1050/788 |  |  |  |  |  |  |  |  |  |  |
| OPR140X-1861 |  |  | 200 | 1400 | 1400/1050 |  |  |  |  |  |  |  |  |  |  |
| OPR080X-2084 | 82 1/16" (2084 mm) | 47.5 lb (21.6 kg) | 100 | 800 | 800/600 |  |  |  |  |  |  |  |  |  |  |
| OPR120X-2084 |  |  | 150 | 1200 | 1200/900 |  |  |  |  |  |  |  |  |  |  |
| OPR160X-2084 |  |  | 200 | 1600 | 1600/1200 |  |  |  |  |  |  |  |  |  |  |
| OPR090X-2309 | 90 15/16" (2309 mm) | 53.5 lb (24.3 kg) | 100 | 900 | 900/675 |  |  |  |  |  |  |  |  |  |  |
| OPR135X-2309 |  |  | 150 | 1350 | 1350/1013 | ----- | ----- |  |  |  |  |  |  |  |  |
| OPR180X-2309 |  |  | 200 | 1800 | 1800/1350 | ----- | ----- |  |  |  |  |  |  |  |  |
| OPR100X-2541 | 100" (2541 mm) | 58.4 lb (26.6 kg) | 100 | 1000 | 1000/750 |  |  |  |  |  |  |  |  |  |  |
| OPR150X-2541 |  |  | 150 | 1500 | 1500/1125 | ----- | ----- |  |  |  |  |  |  |  |  |
| OPR200X-2541 |  |  | 200 | 2000 | 2000/1500 | ----- | ----- |  |  |  |  |  |  |  |  |

- 1 208V = 75% of wattage at 240V.

- 2 X = 2 (120V), 8 (208V), 0 (240V/208V), 7 (347V), 6 (600V).

-  = Available.

- Add "BL" for white, "AM" for almond.

Other colors, accessories and finishes are available upon request. Please contact your customer service representative.

Other voltages available upon request.

- Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| Finish |  |  |  |
| - | BD | Clean back |  |
| KIT-OPR-BR | - | Relay section, 5 in. (127 mm) |  |
| KIT-OPR-CE | - | Outside corner, 5 in. (127 mm) |  |
| KIT-OPR-CI | - | Inside corner, 5 in. (127 mm) |  |
| KIT-OPR-JT | - | Splice plate, 3 1/8 in. (80 mm) |  |
| KIT-OPR-PE | - | Round pedestal kit (2), 3 in. (76 mm) |  |
| KIT-OPR-PEC | - | Square-shape pedestal kit (2), 3 in. (76 mm) |  |
| OPR-SV-XX | - | Empty cabinet length of XX mm (minimum: 154 mm, maximum: 2541 mm) |  |
| Controls |  |  |  |
| KIT-OPR-R-347 | R | Single-pole relay, coil 24V, without transformer: 22A at 120V, 208V, 240V - 19A at 277V - 18A at 347V |  |
| KIT-OPR-R-600 | R | Single-pole relay, coil 24V, without transformer, 10A at 600V |  |
| KIT-OPR-RT-120 | RT | Single-pole relay, coil 24V, with transformer 120V/24V, 22A at 120V |  |
| KIT-OPR-RT-208 | RT | Single-pole relay, coil 24V, with transformer 208V/24V, 22A at 208V |  |
| KIT-OPR-RT-240 | RT | Single-pole relay, coil 24V, with transformer 240V/24V, 22A at 240V |  |
| KIT-OPR-RT-277 | RT | Single-pole relay, coil 24V, with transformer 277V/24V, 19A at 277V |  |
| KIT-OPR-RT-347 | RT | Single-pole relay, coil 24V, with transformer 347V/24V, 18A at 347V |  |
| KIT-OPR-RT-6001 | RT1 | Single-pole relay, coil 24V, with transformer 600V/24V, 10A at 600V (2 wire only) |  |
| KIT-OPR-T | T | Single-pole thermostat: 30A at 120V, 208V, 240V - 15A at 277V, 347V, 600V |  |
| KIT-OPR-T22 | T22 | Double-pole thermostat: 30A at 120V, 208V, 240V - 15A at 277V, 347V, 600V |  |
| KIT-OPR-T-AV | TAV | Single-pole tamperproof thermostat: 30A at 120V, 208V, 240V - 15A at 277V, 347V, 600V |  |
| KIT-OPR-T2-AV2 | T2AV2 | Double-pole tamperproof thermostat : 30A à 120V, 208V, 240V - 15A à 277V, 347V, 600V |  |
| KIT-OPR-TRIAC-347 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 15A at 347V max. |  |
| KIT-OPR-TRIAC-347T | TRIACT | Single-pole electronic relay, coil 24V, with transformer: 15A at 208V, 240V, 347V |  |
| KIT-OPR-TRIAC-600 | TRIAC | Single-pole electronic relay, coil 24V, without transformer, 6A at 600V max. |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 These 2-wire relays with transformer can't supply of 3-wire low voltage thermostats.

2 "Off" position.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |