Series

OPX	Explosion-Proof Sloped Top Convector Heater

Features

Color	-	Standard: beige.

Voltage	-	120V, 208V, 240V, 347V, 480V, 600V, 1 or 3-phase.

Construction	-	Sloped top, 18-gauge steel cabinet.

- Low temperature convection.

- Linear high-limit temperature control with automatic reset.

- Sealed junction box.

Heating element	-	Cooper tubular heating element with aluminum fins.

Watt density	-	Varies according to temperature code.

Control	-	Built-in or wall thermostat available.

Installation	-	Built-in wall brackets.

-	Minimum mounting height of 6 in. (152 mm) above the ﬂoor.

Warranty	-	1-year warranty against defects.

Application 	-	Paint room, chemical storage, petrochemical plant, oil refinery, dry cleaning plant,

coal handling facility.

Maximum room temperature: 40 °C (104 °F).

Consult the Electrical Code in your area before installing this unit.

Models

| Watts | Product # |  |  |  |  |  |  |  |  |  |  |  | To Code2 | Unit Size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 120V (1 ph) | Qté | 208V (1 ph) | #### Qté | 208V (3 ph) | Qté | 240V (1 ph) | #### Qté | 347V (1 ph) | Qté | 600V (3 ph) | Qté |  |  |
| Classe I, Groupes B3, C et D, Divisions 1 et 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 500 | OPX2A0502 |  | OPX2A0508 |  | - | - | OPX2A0500 |  | - | - | - | - | T2A | 11 |
| 1000 | OPX2A1002 |  | OPX2A1008 |  | - | - | OPX2A1000 |  | OPX2A1007 |  | - | - | T2A | 11 |
| 1800 | OPX2A1802 |  | OPX2A1808 |  | - | - | OPX2A1800 |  | OPX2A1807-CTT |  | OPX2A1836-CTT |  | T2A | 2 |
| 2500 | - | - | OPX2A2508 |  | OPX2A2538-CTT |  | OPX2A2500 |  | OPX2A2507-CTT |  | OPX2A2536-CTT |  | T2A | 2 |
| 3600 | - | - | OPX2A3608 |  | OPX2A3638-CTT |  | OPX2A3600 |  | OPX2A3607-CTT |  | OPX2A3636-CTT |  | T2A | 3 |
| 4400 | - | - | - | - | OPX2A4438-CTT |  | OPX2A4400 |  | OPX2A4407-CTT |  | OPX2A4436-CTT |  | T2A | 3 |
| 6500 | - | - | - | - | OPX2A6538-CTT |  | OPX2A6500-CTT |  | OPX2A6507-CTT |  | OPX2A6536-CTT |  | T2A | 4 |
| 7500 | - | - | - | - | OPX2A7538-CTT |  | OPX2A7500-CTT |  | OPX2A7507-CTT |  | OPX2A7536-CTT |  | T2A | 4 |
| 9500 | - | - | - | - | OPX2A9538-CTT |  | OPX2A9500-CTT |  | OPX2A9507-CTT |  | OPX2A9536-CTT |  | T2A | 4 |
| Classe I, Groupes B3, C et D, Divisions 1 et 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 500 | OPX3A0502 |  | OPX3A0508 |  | - | - | OPX3A0500 |  | - | - | - | - | T3A | 11 |
| 1400 | OPX3A1402 |  | OPX3A1408 |  | - | - | OPX3A1400 |  | OPX3A1407-CTT |  | - | - | T3A | 2 |
| 2300 | OPX3A2302 |  | OPX3A2308 |  | OPX3A2338-CTT |  | OPX3A2300 |  | OPX3A2307-CTT |  | OPX3A2336-CTT |  | T3A | 3 |
| 3200 | - | - | OPX3A3208 |  | OPX3A3238-CTT |  | OPX3A3200 |  | OPX3A3207-CTT |  | OPX3A3236-CTT |  | T3A | 4 |
| 5000 | - | - | - | - | OPX3A5038-CTT |  | OPX3A5000 |  | OPX3A5007-CTT |  | OPX3A5036-CTT |  | T3A | 4 |
| Classe II, Groupes E, F et G, Divisions 1 et 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 500 | OPX3C0502 |  | OPX3C0508 |  | - | - | OPX3C0500 |  | - | - | - | - | T3C | 11 |
| 1000 | OPX3C1002 |  | OPX3C1008 |  | - | - | OPX3C1000 |  | OPX3C1007-CTT |  | - | - | T3C | 2 |
| 1700 | OPX3C1702 |  | OPX3C1708 |  | - | - | OPX3C1700 |  | OPX3C1707-CTT |  | OPX3C1736-CTT |  | T3C | 3 |
| 3200 | - | - | OPX3C3208 |  | OPX3C3238-CTT |  | OPX3C3200 |  | OPX3C3207-CTT |  | OPX3C3236-CTT |  | T3C | 4 |

Options

| Product # | Description | Factory Installed | Kit | Qty |
| --- | --- | --- | --- | --- |
| C1-14 | Galvanized steel cabinet wet area approved (Type 3), model 1 |  | - |  |
| C1-24 | Galvanized steel cabinet wet area approved (Type 3), model 2 |  | - |  |
| C1-34 | Galvanized steel cabinet wet area approved (Type 3), model 3 |  | - |  |
| C1-44 | Galvanized steel cabinet wet area approved (Type 3), model 4 |  | - |  |
| C2-14 | 304 stainless steel cabinet corrosion and wet area approved (Type 3), model 1 |  | - |  |
| C2-24 | 304 stainless steel cabinet corrosion and wet area approved (Type 3), model 2 |  | - |  |
| C2-34 | 304 stainless steel cabinet corrosion and wet area approved (Type 3), model 3 |  | - |  |
| C2-44 | 304 stainless steel cabinet corrosion and wet area approved (Type 3), model 4 |  | - |  |
| CTT | 24V contactor and control transformer |  | - |  |
| T | Class I and II, Groups C, D, E, F and G single-pole thermostat |  | - |  |
| TGRB | Class I and II, Groups B, C, D, E, F and G single-pole thermostat |  | - |  |
| XB-TM | Explosion-proof wall thermostat, range -18 °C to 40 °C (0 °F to 100 °F): 15A at 600V, 25A at 277V | - |  |  |

1 No factory installed option available for size 1 model.

2 Temperature code: T2A = 280 °C (536 °F), T3A = 180 °C (356 °F), T3C = 160 °C (320 °F).

3 For Group B applications, if the convector requires a built-in thermostat, use the TGRB option.

4 See features below.

For any other ratings and risk areas, contact our customer service.

Standard color is beige.

Features of the C1 Option

Galvanized steel cabinet wet area approved with a baked on powder coating.

Elements with a chromate coating.

A gasketed terminal box cover (O-ring).

The option C1 is not available with the T option and the TGRB option. Allow a 4 weeks production lead time.

Features of the C2 Option

304 stainless steel cabinet corrosion and wet area (Type 3) approved with a baked on powder coating.

Elements with a chromate coating.

A gasketed terminal box cover (O-ring).

ONLY the cabinet is upgraded to stainless steel.

The option C2 is not available with the T option and the TGRB option. Allow a 4 weeks production lead time.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |