- Series

ORC	Radiant Cove Heater

- Features

Color and Finish	-	Standard: silichem off-white, mat black and anodized aluminum finish.

Voltage	-	120V, 208V, 240V, 277V, 347V, 480V, 600V, 50/60 Hz.

Construction	-	Front made of an aluminum extrusion profiled to increase radiating surface.

-	Back of the unit made with 18-gauge steel.

-	Junction box on each end.

-	Wall installation brackets (supplied) that doesn’t require opening the unit’s casing.

-	KOE 7/8 in. (22 mm) for the installation of the BX wires.

-	4 in. (102 mm) ground wires on both ends.

-	Wireway approved for interconnection of units using the factory installed additional wires. This is used in wall-to-wall heater installation.

-	Highly-emissive paint on the heat exchanger for higher efficiency.

Heating element	-	Stainless steel tubular heating element inserted into a solid aluminum extrusion

preventing any thermal expansion noise.

Watt density	-	150W/ft.

Control	-	Single or double-pole built-in thermostat available.

Ouellet highly recommends to use an electronic wall thermostat for greater comfort.

Installation	-	6 ft. (1.83 m) minimum clearence to ﬂoor and 4 1/2 in. (114 mm)

minimum from ceiling.

-	Variety of accessories available for wall-to-wall installation.

Warranty	-	1o-year warranty against defects.

Application	-	Entrance way, bathroom, kitchen, living room, family room, bedroom, patio door,

bay window, daycare, nursery, workshop.

- Models

| Watts | Product # |  |  |  |  |  |  |  |  |  | Weight |  | Length |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 120V | #### Qty | 208V | Qty | 240V | Qty | 347V | Qty | 600V | Qty | lb | kg | in. | mm |
| 450 | ORC0452 |  | ORC0458 |  | ORC0450 |  | ORC0457 |  | - | - | 6.0 | 2.7 | 34 | 864 |
| 600 | ORC0602 |  | ORC0608 |  | ORC0600 |  | ORC0607 |  | - | - | 8.0 | 3.6 | 47 | 1194 |
| 750 | ORC0752 |  | ORC0758 |  | ORC0750 |  | ORC0757 |  | ORC0756 |  | 10.0 | 4.5 | 59 | 1499 |
| 900 | ORC0902 |  | ORC0908 |  | ORC0900 |  | ORC0907 |  | ORC0906 |  | 12.0 | 5.4 | 71 | 1803 |
| 1050 | ORC1052 |  | ORC1058 |  | ORC1050 |  | ORC1057 |  | ORC1056 |  | 13.0 | 5.9 | 83 | 2108 |
| 1200 | ORC1202 |  | ORC1208 |  | ORC1200 |  | ORC1207 |  | ORC1206 |  | 15.0 | 6.8 | 94 | 2388 |
| 15001 | ORC1502 |  | ORC1508 |  | ORC1500 |  | ORC1507 |  | ORC1506 |  | 18.0 | 8.2 | 113 | 2870 |

- Options

| Product # Kit | Product # Factory installed* | Description | ## Qty |
| --- | --- | --- | --- |
| KIT-ORC-JT | - | Splice plate |  |
| KIT-ORC-TB62 | TB62 | Single-pole bi-metal thermostat |  |
| KIT-ORC-TB262, 3 | TB262, 3 | 4 in. (101 mm) section with double-pole bi-metal thermostat |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 These models must be ordered in pairs to reduce shipping damages.

2 Ouellet highly recommends to use an electronic wall thermostat for greater comfort.

3 "Off" position.

### Add "BL" for silichem off-white, "NM" for mat black and "AA" for anodized aluminum finish.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| ELectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |