Series

OSM-AD	AirDuo Hand and Hair Dryer

Features

Color	-	Standard: white.

Finish	-	Standard: epoxy paint.

Voltage	-	120V, 240/208V, 1-phase.

Construction	-	Die cast aluminum housing.

-	360° revolving die cast aluminum nozzle finished in bright chrome plating. Nozzle can be locked to fixed position in the field.

Fan	-	Brushless type motor, thermally protected, operating at 3200 rpm.

- Delivers 148 cfm of air volume and 4700 lfm of air velocity (74 dBA with hands).

Heating element	-	Helical coil nickel-chromium alloy wire protected by mica insulator.

Control	-	Pushbutton with micro-switch activated timer. Time cycle setting can be adjusted in the field from 15 seconds to 3 minutes.

Installation	-	Surface-mounted with security screws to prevent vandalism (tool supplied).

Warranty	-	5-year warranty against defects.

Application	-	School, gymnasium, health center, pool.

Models

| Product # | Qty | Volts | Watts | Weight |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | lb | kg |
| OAD1802BL |  | 120 | 1800 | 18.0 | 8.2 |
| OAD1800BL |  | 240/208 | 1800/1350 | 18.0 | 8.2 |

- Standard color is white.

### Others

| Type | Quantity | Model et description Complementary information | Volts | Phase | Watts | Length mm/in. | Information Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |