FHM	Floor Heating Cable on Mat

Features

Voltage	-	120V, 240/208V, 1-phase.

Cold lead length	-	10’ (3 m).

Construction	-	Heating cable made of a twin conductor fastened to an adhesive fibreglass mat for a simpler and faster installation with negligible magnetic field.

Watt density	-	12W/sq. ft. (130W/sq. m), 3” (76 mm) spacing.

Dimension	-	Mats of 18 in. (0.46 m) in width offered in several lengths.

Control	-	Two types of control method possible (see instruction manual for details):

• Surface heating control with electronic thermostat in floor mode (F)
  	and temperature sensor.

• Ambient heating control with electronic thermostat in ambient mode

with floor limit (A or AF) and temperature sensor.

Note: A ground fault circuit interrupter (GFCI) must be used with this heating device

unless exempted by the applicable national and/or local electrical code for the area of installation.

Included materials	-	15' (4.6 m) ﬂoor sensor.

- Measurements table label (to be placed in electrical panel).

Installation	-	Never cut or shorten the heating cable.

-	For indoor applications only.

-	On concrete slab or plywood subfloor.

Warranty	-	25-year warranty on the heating cable.

Application 	-	Kitchen, bathroom, entrance way, family room, living room.

Models

| Product # 240/208V | Qty | Product # 120V | Qty | Cable Diameter (mm) | Covered Surface1 Spacing 3” (76 mm) |  | Length |  | Weight |  | Watts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | sq. ft. | sq. m | ft. in. | m | lb | kg |  |
| - | - | FHM120-60 |  | 3.2 | 5 | 0.5 | 3’ 4’’ | 0.9 | 2.0 | 0.9 | 60 |
| FHM240-120 | - | FHM120-120 |  | 3.2 | 10 | 0.9 | 6’ 8’’ | 1.8 | 2.0 | 0.9 | 120 |
| - | - | FHM120-180 |  | 3.2 | 15 | 1.4 | 10’ | 3.0 | 3.0 | 1.4 | 180 |
| FHM240-240 |  | FHM120-240 |  | 3.2 | 20 | 1.9 | 13’ 4’’ | 4 | 3.0 | 1.4 | 240 |
| - | - | FHM120-300 |  | 3.2 | 25 | 2.4 | 16’ 8’’ | 4.9 | 4.0 | 1.8 | 300 |
| FHM240-360 |  | FHM120-360 |  | 3.2 | 30 | 2.8 | 20’ | 6.1 | 4.0 | 1.8 | 360 |
| - | - | FHM120-420 |  | 3.2 | 35 | 3.3 | 23’ 4’’ | 7 | 5.0 | 2.3 | 420 |
| FHM240-480 |  | FHM120-480 |  | 3.2 | 40 | 3.8 | 26’ 8’’ | 7.9 | 5.0 | 2.3 | 480 |
| - | - | FHM120-540 |  | 3.2 | 45 | 4.2 | 30’ | 9.2 | 6.0 | 2.7 | 540 |
| FHM240-600 |  | FHM120-600 |  | 3.2 | 50 | 4.7 | 33’ 4’’ | 10.1 | 7.0 | 3.2 | 600 |
| FHM240-720 |  | FHM120-720 |  | 3.2 | 60 | 5.6 | 40’ | 12.2 | 7.0 | 3.2 | 720 |
| FHM240-840 |  | FHM120-840 |  | 3.2 | 70 | 6.5 | 46’ 8’’ | 14.0 | 9.0 | 4.1 | 840 |
| FHM240-960 |  | FHM120-960 |  | 3.2 | 80 | 7.4 | 53’ 4’’ | 16.2 | 10.0 | 4.5 | 960 |
| FHM240-1080 |  | - | - | 3.2 | 90 | 8.4 | 60’ | 18.3 | 11.0 | 5.0 | 1080 |
| FHM240-1200 |  | - | - | 3.2 | 100 | 9.3 | 66’ 8’’ | 20.1 | 11.0 | 5.0 | 1200 |
| FHM240-1440 |  | - | - | 3.2 | 120 | 11.2 | 80’ | 24.4 | 13.0 | 5.9 | 1440 |

1Does not represent the surface of the room but rather the surface covered by the floor heating system, excluding the fixtures and other spaces to consider.

208V = 75% of wattage at 240V.

15' (4.6 m) floor sensor and 10' (3 m) cold lead included.

The color of the mesh may be different.

Options

| Product # Kit | Description | Qty |
| --- | --- | --- |
| OTM-CC | CableCheck - Electrical fault indicator |  |
| OTM-SA | Adhesive spray to secure the mat on concrete slab, 16.75 oz (474 g) |  |
| KIT-SP1 | Repair kit |  |
| KIT-CBL-SN | 15 ft. (4.6 m) floor sensor |  |

- Electronic Thermostats for Floor Heating System

| Product # | Description | Qty | Amp. | Volts | Protection |
| --- | --- | --- | --- | --- | --- |
| OTH3600-GA2 | Non programmable electronic thermostat for floor heating system |  | 15 | 120/208/240 | Class A GFCI 5 mA1 |
| OTH3600P-GA2 | Programmable electronic thermostat for floor heating system |  | 15 | 120/208/240 | Class A GFCI 5 mA1 |
| OTH3600-GA-GT2 | Wireless communication programmable electronic thermostat for floor heating system |  | 15 | 120/208/240 | Class A GFCI 5 mA1 |

- Slave Unit for Floor Heating System

| Product # | Description | Qty | Amp. | Volts | Protection |
| --- | --- | --- | --- | --- | --- |
| TR1310-120-240GA3 | Slave unit for floor heating system |  | 15 | 120/208/240 | Class A GFCI 5 mA1 |

1 GFCI: Ground fault circuit interrupter.

2 15’ (4.6 m) floor sensor included.

3 Compatible with OTH3600P-GA and OTH3600-GA-GT only, not compatible with the non programmable thermostat OTH3600-GA.

Standard color is white.

### Others

| Type | Quantity | Model and description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |