Series

OVC	Compact Electronic Fan Heater

Features

Color	-	Standard: white.

-	Optional (upcharge applicable): almond, metallic silver, bronze, metallic charcoal, aluminum, 
semi-gloss black, sand.

- Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120, 240/208V, 1-phase.

Construction	-	Shock-resistant cast aluminum front cover.

-	20-gauge steel.

-	High-limit temperature control with automatic reset.

Fan	-	Whisper-quiet squirrel-cage fan.

-	Factory-lubricated, two-speed motor.

Heating element	-	Nickel-chromium alloy stitched wire protected by mica insulators.

-	Heating element provides instant heat.

Control	Model (-TH)

-	Operating modes adapted for all requirements (comfort, economy, frost-free mode).

-	Built-in electronic digital thermostat/timer with backlit display that can be used as a

night light.

-	Electronic thermostat with 2 triacs automatically switches from 750W to 1500W when needed.

-	Keypad lock function that eliminates unwanted temperature adjustments and the use of the thermostat modes.

Model (-PW)

- Requires non pulsing wall-mounted thermostat (sold separately).

- Wattage must be selected at the time of wiring (750W or 1500W).

Installation	-	6 in. (152 mm) minimum clearance to ﬂoor.

-	May be recessed or surface-mounted without adaptor in 2 in. X 3 in. stud partitions

with gypsum.

-	Easy installation and connection with mounting screws and knockouts (mounting screws included).

-	Template included.

Warranty	-	5-year warranty against defects.

Application	-	Hallways, bathrooms, shower rooms, dining rooms, kitchens, corridors.

Models

| Product # | Qty | Volts | Watts |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min. | max. | lb | kg |
| OVC1502BL-TH |  | 120 | 750/563 | 1500/1125 | 11.5 | 5.22 |
| OVC1500BL-TH |  | 240/208 | 750/563 | 1500/1125 | 11.5 | 5.22 |
| OVC1500BL-PW1 |  | 240/208 | 750/563 1500/1125 |  | 12.2 | 5.53 |

- Options

| Product # Factory installed* | Description | Qty |
| --- | --- | --- |
| DIS2 | Disconnect switch, 20A at 250V |  |
| R3 | Single-pole relay, coil 24V, without transformer, 22A at 208V, 240V |  |
| RT3 | Single-pole relay, coil 24V, with transformer 208V, 240V/24V, 22A max. |  |
| TRIAC3 | Single-pole electronic relay, coil 24V, without transformer, 15A at 240V max. |  |
| TRIACT3 | Single-pole electronic relay, coil 24V, with transformer, 15A at 240V max. |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 Requires non pulsing wall-mounted thermostat (sold separately). Please refer to the Wall Thermostats Reference Chart.

2 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect. This option is available with built-in thermostat (option -TH) or power selector (option -PW).

3 These options must be chosen with the power selector (option -PW).

Custom colors available upon request.

### Others

| Type | Quantity | Model and description Additional information | Volts | Phase | Watts | Length mm/in. | Information Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Ouellet Contact: |  |
| Comments: |  |