Series

OVO	Residential Wall Fan Heater

Features

Color	-	Standard: white.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 208V, 240/208V, 1-phase.

Construction	-	22-gauge steel cabinet, 20-gauge steel grille.

-	High-limit temperature control with manual reset.

- 	Additional one-time thermal fuse.

Fan	-	Quiet fan.

Heating element	-	Nichrome heating element provides instant heat.

Control	-	Built-in double-pole thermostat (-T2 models).

-	Built-in double-pole thermostat and timer (-T2M models).

-	Built-in thermostat kit available.

- 	Wall thermostat (not included, the use of an electronic thermostat is strongly

recommended).

Note: If an electronic wall-mounted thermostat is used, it must be compatible with forced-air heaters.

- 	Controls located at the top of the unit.

Installation	-	6 in. (150 mm) minimum clearance to ﬂoor or any adjacent surfaces.

-	May be recessed or surface-mounted with optional surface mounting box.

- Quick and easy installation easily retrofits most popular models.

- 	Wattage can be selected onsite.

Warranty	-	5-year warranty.

Application	-	Kitchen, family room, bathroom (model with timer and thermostat), entrance way, office.

Models

| Product # | Qty | Product # | Qty | Product # | Qty | Volts | Watts1 | BTU/hr | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Without thermostat2 |  | Built-in thermostat |  | Built-in thermostat and timer |  |  |  |  | lb | kg |
| # OVOU1002BL |  | OVOU1002BL-T2 |  | OVOU1002BL-T2M |  | 120 | 1000 or 500 | 3413 or 1706 | 7.4 | 3.4 |
| # OVOU1502BL |  | OVOU1502BL-T2 |  | OVOU1502BL-T2M |  | 120 | 1500 or 750 | 5119 or 2560 | 7.4 | 3.4 |
| # OVOU1008BL |  | OVOU1008BL-T2 |  | OVOU1008BL-T2M |  | 208 | 1000 or 500 | 3413 or 1706 | 7.4 | 3.4 |
| # OVOU2008BL |  | OVOU2008BL-T2 |  | OVOU2008BL-T2M |  | 208 | 2000 or 1000 | 6825 or 3413 | 7.4 | 3.4 |
| # OVOU1000BL |  | OVOU1000BL-T2 |  | OVOU1000BL-T2M |  | 240/208 | 1000/750 or 500/375 | 3413/2560 or 1706/1280 | 7.4 | 3.4 |
| # OVOU1500BL |  | OVOU1500BL-T2 |  | OVOU1500BL-T2M |  | 240/208 | 1500/1125 or 750/560 | 5119/3839 or 2560/1911 | 7.4 | 3.4 |
| # OVOU2000BL |  | OVOU2000BL-T2 |  | OVOU2000BL-T2M |  | 240/208 | 2000/1500 or 1000/750 | 6825/5119 or 3413/2560 | 7.4 | 3.4 |

Standard color is white.

Add -SBE after BL for unit without back box. (Available for United States market only.)

1Wattage can be selected onsite.

2Requires non pulsing wall-mounted thermostat. Please refer to the Wall Thermostats Reference Chart at the end of the catalog.

- Options

| Product # Kit | Description | Qty |
| --- | --- | --- |
| KIT-OVO-BB6 | Back box (6 pack) |  |
| KIT-OVO-BSBL | Surface mounting box |  |
| KIT-OVO-T1BL | Single-pole thermostat, 120V-600V |  |
| KIT-OVO-T2BL | Double-pole thermostat, 120V-277V |  |

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |