Series

OVR	Register Wall Fan Heater

Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 208V, 240/208V, 1-phase.

Construction	-	20-gauge steel.

-	High-limit temperature control with automatic reset.

Fan	-	Quiet fan operating.

-	500W to 1000W; 40 cfm fan and 1250W to 2000W; 80 cfm fan.

-	Factory-lubricated motor.

Heating element	-	Durable tubular heating element with fins.

-	Factory sealed element upon request.

Control	-	Built-in thermostat available with a control knob or tamperproof.

Note: If an electronic thermostat is used, it must be compatible with forced-air heaters.

Installation	-	The air outlet should be installed at least 10 in. (25.4 cm) above ﬂoor.

-	Quick installation with recess box mounting holes.

-	Large wiring compartment.

-	May be recessed in 2 in. x 3 in. stud partitions.

-	Surface mounting box available.

-	Tight fit with V-shaped recess box.

Warranty	-	5-year warranty against defects.

Application	-	Entrance way, kitchen, bathroom, family room, basement.

Models

| Product # | Qty | Volts | Watts | Weight |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | lb | kg |
| OVRU0502 |  | 120 | 500 | 9.0 | 4.1 |
| OVRU0752 |  | 120 | 750 | 9.0 | 4.1 |
| OVRU1002 |  | 120 | 1000 | 9.0 | 4.1 |
| OVRU1252 |  | 120 | 1250 | 9.0 | 4.1 |
| OVRU1502 |  | 120 | 1500 | 9.0 | 4.1 |
| OVRU0500 |  | 240/208 | 500/375 | 9.0 | 4.1 |
| OVRU0750 |  | 240/208 | 750/563 | 9.0 | 4.1 |
| OVRU1000 |  | 240/208 | 1000/750 | 9.0 | 4.1 |
| OVRU1250 |  | 240/208 | 1250/938 | 9.0 | 4.1 |
| OVRU1500 |  | 240/208 | 1500/1125 | 9.0 | 4.1 |
| OVRU2000 |  | 240/208 | 2000/1500 | 9.0 | 4.1 |
| OVRU2008 |  | 208 | 2000 | 9.0 | 4.1 |

Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| KIT-OVR-BS | - | Surface mounting box |  |
| KIT-OVR-DIS3 | DIS3 | Disconnect switch, 20A at 277V max. |  |
| - | ES1 | With sealed element factory installed |  |
| KIT-OVR-TB6 | TB6 | Single-pole bi-metal thermostat |  |
| KIT-OVR-TB262 | TB262 | Double-pole bi-metal thermostat |  |
| KIT-OVR-TB6-AV | TB6AV | Single-pole bi-metal tamperproof thermostat |  |
| KIT-OVR-TB26-AV2 | TB26AV2 | Double-pole bi-metal tamperproof thermostat |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 The option "sealed element" increases the reliability of these in humid environments. It does not completely eliminate the leakage currents of metal sheathed heating elements.

2 "Off" position.

3 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

Add "BL" for white, "AM" for almond.

Upcharge available for optional colors shown in the color chart section.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |