Series

OVRP	Compact Ceiling Fan Heater

Features

Color	-	Standard: white, almond.

-	Optional (upcharge applicable): metallic silver, bronze, metallic charcoal,

aluminum, semi-gloss black, sand.

-	Custom colors available upon request.

Finish	-	Standard: epoxy/polyester powder paint.

Voltage	-	120V, 240/208V, 347V, 1-phase.

Construction	-	20-gauge steel cabinet.

-	22-gauge steel front cover.

-	High-limit temperature control with automatic reset.

Fan	-	Quiet fan operating.

-	120V, 208V, 240/208V models: 500W to 1000W; 40 cfm fan and 
1250W to 2000W; 80 cfm fan.

-	347V models: 80 cfm fan.

-	Factory-lubricated motor.

Heating element	-	120V, 208V, 240/208V: durable tubular heating element with fins.

- Factory sealed element upon request.

- 347V: Iron-Chromium-Aluminum alloy.

Control	-	Must be used with a compatible wall thermostat (not included).

-	If an electronic thermostat is used, it must be compatible with forced-air heaters.

-	Electronic relay available.

Installation	-	Ceiling installation only.

-	The air outlet should be installed at least 10 in. (25.4 cm) from the wall.

-	Quick installation with recess box mounting holes.

-	Large wiring compartment.

-	May be recessed in ceiling.

-	Surface mounting box available.

-	Tight fit with V-shaped recess box.

Warranty	-	5-year warranty against defects.

Application	-	Entrance way, kitchen, bathroom, family room, basement.

Models

| # Product | Qty | Volts | Watts | Weight |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | lb | kg |
| OVRP0502 |  | 120 | 500 | 9.0 | 4.1 |
| OVRP0752 |  | 120 | 750 | 9.0 | 4.1 |
| OVRP1002 |  | 120 | 1000 | 9.0 | 4.1 |
| OVRP1252 |  | 120 | 1250 | 9.0 | 4.1 |
| OVRP1502 |  | 120 | 1500 | 9.0 | 4.1 |
| OVRP0500 |  | 240/208 | 500/375 | 9.0 | 4.1 |
| OVRP0750 |  | 240/208 | 750/563 | 9.0 | 4.1 |
| OVRP1000 |  | 240/208 | 1000/750 | 9.0 | 4.1 |
| OVRP1250 |  | 240/208 | 1250/938 | 9.0 | 4.1 |
| OVRP1500 |  | 240/208 | 1500/1125 | 9.0 | 4.1 |
| OVRP0507 |  | 347 | 500 | 9.0 | 4.1 |
| OVRP1007 |  | 347 | 1000 | 9.0 | 4.1 |
| OVRP1507 |  | 347 | 1500 | 9.0 | 4.1 |
| OVRP2007 |  | 347 | 2000 | 9.0 | 4.1 |

Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| KIT-OVR-BS | - | Surface mounting box |  |
| KIT-OVR-DIS1 | DIS1 | Disconnect switch, 20A at 277V max. (120V, 208V, 240/208V only) |  |
| - | ES2 | Factory sealed element (120V, 208V, 240/208V only) |  |
| - | TRIAC3 | Single-pole electronic relay, coil 24V, without transformer, 15A at 347V max. |  |
| - | TRIACT3 | Single-pole electronic relay, coil 24V, with transformer, 15A at 208V, 240V, 277V, 347V |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 For the U.S. only: The nominal current of the unit must not exceed 80% of the capacity of the disconnect.

2 The option "sealed element" increases the reliability of these in humid environments. It does not completely eliminate the leakage currents of metal sheathed heating elements.

3 The TRIAC and TRIACT option is only available for the products OVRP0507, OVRP0757 and OVRP1007.

Add "BL" for white, "AM" for almond.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |