- Series

OWF-R	Floor Heating Cable for Installation with Strapping

- Features

Voltage	-	120V, 240/208V, 1-phase.

Cold lead length	-	10’ (3 m).

Construction	-	Heating cable made of a twin conductor for a simpler and faster installation,

with negligible magnetic field.

Watt density	-	12W/sq. ft. (130W/sq. m), 3” (76 mm) spacing.

- 9W/sq. ft. (100W/sq. m), 4” (102 mm) spacing.

Control	-	Two types of control method possible (see instruction manual for details):

• Surface heating control with electronic thermostat in floor mode (F)
  	and temperature sensor.

• Ambient heating control with electronic thermostat in ambient mode

with floor limit (A or AF) and temperature sensor.

Note: A ground fault circuit interrupter (GFCI) must be used with this heating device

unless exempted by the applicable national and/or local electrical code for the area of installation.

Included materials	-	Plastic strapping.

- 15' (4.6 m) ﬂoor sensor.

- Measurements table label (to be placed in electrical panel).

Installation	-	Never cut or shorten the heating cable.

-	For indoor applications only.

- On concrete slab or plywood subfloor.

Warranty	-	25-year warranty on the heating cable.

Application 	-	Kitchen, bathroom, entrance way, family room, living room.

- Models

| Product # Strapping included1 |  |  |  | Covered Surface Depending on Spacing2 |  |  |  | Length |  | Strapping length included (ft.) | Weight |  | Watts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Spacing 3’’ (76 mm) |  | Spacing 4’’ (102 mm) |  |  |  |  |  |  |  |
| 240/208V | 120V | Cable Diameter (mm) | Qty | 12W/ sq. ft. | 130W/ sq. m | 9W/ sq. ft. | 100W/ sq. m | ft. | m |  | lb | kg |  |
| - | OWF-R0082 | 4.3 |  | 8 | 0.7 | 11 | 1.0 | 32 | 9.75 | 25 | - | - | 85 |
| - | OWF-R0122 | 4.3 |  | 10 | 0.9 | 13 | 1.2 | 40 | 12.19 | 25 | - | - | 120 |
| - | OWF-R0152 | 4.3 |  | 13 | 1.2 | 17 | 1.6 | 53 | 16.15 | 25 | - | - | 150 |
| OWF-R0170 | OWF-R0172 | 4.3 |  | 15 | 1.4 | 20 | 1.9 | 61 | 18.59 | 25 | - | - | 170 |
| OWF-R0240 | OWF-R0242 | 4.3 |  | 20 | 1.9 | 27 | 2.5 | 80 | 24.38 | 25 | - | - | 240 |
| OWF-R0300 | OWF-R0302 | 4.3 |  | 25 | 2.3 | 33 | 3.1 | 101 | 30.78 | 25 | - | - | 300 |
| OWF-R0360 | OWF-R0362 | 4.3 |  | 30 | 2.8 | 40 | 3.7 | 120 | 36.58 | 50 | - | - | 360 |
| OWF-R0420 | OWF-R0422 | 4.3 |  | 35 | 3.3 | 47 | 4.3 | 141 | 42.98 | 50 | - | - | 420 |
| OWF-R0475 | OWF-R0472 | 4.3 |  | 40 | 3.7 | 53 | 5.0 | 160 | 48.76 | 50 | - | - | 475 |
| OWF-R0600 | OWF-R0602 | 4.3 |  | 50 | 4.6 | 67 | 6.2 | 200 | 60.96 | 50 | - | - | 600 |
| OWF-R0720 | OWF-R0722 | 4.3 |  | 60 | 5.6 | 80 | 7.4 | 240 | 73.15 | 75 | - | - | 720 |
| OWF-R0840 | OWF-R0842 | 4.3 |  | 70 | 6.5 | 93 | 8.7 | 280 | 85.34 | 75 | - | - | 840 |
| OWF-R0960 | OWF-R0962 | 4.3 |  | 80 | 7.4 | 107 | 9.9 | 320 | 97.54 | 75 | - | - | 960 |
| OWF-R1080 | - | 4.3 |  | 90 | 8.4 | 120 | 11.2 | 360 | 109.73 | 100 | - | - | 1080 |
| - | OWF-R1142 | 4.3 |  | 95 | 8.8 | 127 | 11.7 | 380 | 115.82 | 100 | - | - | 1140 |
| OWF-R1200 | - | 4.3 |  | 100 | 9.3 | 133 | 12.4 | 400 | 121.92 | 100 | - | - | 1200 |
| - | OWF-R1322 | 4.3 |  | 110 | 10.2 | 147 | 13.6 | 440 | 134.11 | 100 | - | - | 1320 |
| OWF-R1440 | - | 4.3 |  | 120 | 11.1 | 160 | 14.9 | 480 | 146.30 | 100 | - | - | 1440 |
| - | OWF-R1452 | 4.3 |  | 120 | 11.1 | 160 | 14.8 | 480 | 146.30 | 100 | - | - | 1450 |
| - | OWF-R1502 | 4.3 |  | 125 | 11.6 | 167 | 15.5 | 500 | 152.40 | 125 | - | - | 1500 |
| OWF-R1600 | - | 4.3 |  | 140 | 13.0 | 187 | 17.4 | 560 | 170.69 | 125 | - | - | 1600 |
| - | OWF-R1682 | 4.3 |  | 140 | 13.0 | 187 | 17.4 | 560 | 170.69 | 125 | - | - | 1680 |
| OWF-R1920 | - | 4.3 |  | 160 | 14.9 | 213 | 19.8 | 640 | 195.07 | 125 | - | - | 1920 |
| OWF-R2280 | - | 4.3 |  | 190 | 17.7 | 253 | 23.6 | 760 | 231.64 | 150 | - | - | 2280 |
| OWF-R2640 | - | 4.3 |  | 220 | 20.4 | 293 | 27.2 | 880 | 268.22 | 200 | - | - | 2640 |
| OWF-R2900 | - | 4.3 |  | 242 | 22.5 | 323 | 30.0 | 968 | 295.04 | 200 | - | - | 2900 |
| OWF-R3000 | - | 4.3 |  | 250 | 23.2 | 333 | 30.9 | 1000 | 304.80 | 200 | - | - | 3000 |
| OWF-R3360 | - | 4.3 |  | 280 | 26.0 | 374 | 34.7 | 1120 | 341.38 | 200 | - | - | 3360 |

1 Strapping included. The length of plastic strapping included is based on a square room with strapping every 24" (610 mm).

2 Does not represent the surface of the room but rather the surface covered by the floor heating system, excluding fixtures and other spaces to consider.

15’ (4.6 m) floor sensor and 10' (3 m) cold lead included.

- Options

| Product # Kit | Description | Qty |
| --- | --- | --- |
| OTM-CC | CableCheck - Electrical fault indicator |  |
| KIT-SP1 | Repair kit |  |
| KIT-CBL-G25 | 25 ft. (7.6 m) plastic strapping |  |
| KIT-CBL-SN | 15 ft. (4.6 m) floor sensor |  |

- Electronic Thermostats for Floor Heating System

| Product # | Description | Qty | Amp. | Volts | Protection |
| --- | --- | --- | --- | --- | --- |
| OTH3600-GA2 | Non programmable electronic thermostat for floor heating system |  | 15 | 120/208/240 | Class A GFCI 5 mA1 |
| OTH3600P-GA2 | Programmable electronic thermostat for floor heating system |  | 15 | 120/208/240 | Class A GFCI 5 mA1 |
| OTH3600-GA-GT2 | Wireless communication programmable electronic thermostat for floor heating system |  | 15 | 120/208/240 | Class A GFCI 5 mA1 |

- Slave Unit for Floor Heating System

| Product # | Description | Qty | Amp. | Volts | Protection |
| --- | --- | --- | --- | --- | --- |
| TR1310-120-240GA3 | Slave unit for floor heating system |  | 15 | 120/208/240 | Class A GFCI 5 mA1 |

1 GFCI: Ground fault circuit interrupter.

2 15’ (4.6 m) floor sensor included.

3 Compatible with OTH3600P-GA and OTH3600-GA-GT only, not compatible with the non programmable thermostat OTH3600-GA.

Standard color is white.

### Others

| Type | Quantity | Model et description Complementary information | Volts | Phase | Watts | Length mm/in. | Information Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |