Series

OWS-T	Heating Cable for Snow Melting on Mat

Features

Voltage	-	240/208V, 1-phase.

Construction	-	Series heating cable set, twin conductor type.

- Heating cable held as a mat at regular 3’’ (76 mm) spacing with flexible strips.

- Fluoropolymer/XLPE resistance wire insulation 0.019’’ (0.5 mm) thick.

- Copper shielding (0.823 sq. mm), serves as ground.

- Polyolefin (EPR) outer sheath insulation 0.08’’ (2 mm) thick.

Watt density	-	50W/sq. ft. (538W/sq. m) at 240V, 3’’ (76 mm) spacing.

Dimension	-	24’’ (610 mm) and 36’’ (914 mm) wide mats offered in several lengths.

Cold lead	-  50’ (15 m) long.

- 12 AWG or 14 AWG (according to maximum allowable load).

- PVC outer sheath insulation 0.03’’ (0.76 mm) thick.

- 3/8’’ (9.5 mm) outer diameter.

Included materials	-  Measurements table label (to be placed in electrical panel).

Installation	-	Never cut or shorten the heating cable.

- For outdoor applications only.

- The heating cable must be completely embedded in concrete, asphalt

or stone dust under paving.

- Minimum installation temperature -5 °C (23 °F).

- Maximum long-term exposure temperature 105 °C (221 °F).

- Maximum exposure temperature for 10 minutes 220 °C (428 °F).

Warranty	-	10-year warranty on heating cable.

Application	-	Residential driveway, sidewalk, access ramp, underground parking ramp, 
boarding platforms for animals.

OWS-T 24 in. (610 mm) Wide Mat Models

| Watts | Product # |  |  |  |  |  | Covered surface |  | Mat length |  | Cable length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 208V | AMP | Qty | 240V | AMP | Qty | sq. ft. | sq. m | ft. | m | ft. | m | lb | kg |
| 500 | OWS-T0508-24 | 2.40 |  | OWS-T0500-24 | 2.08 |  | 11.0 | 1.0 | 5 | 1.5 | 43 | 13.1 | 3.0 | 1.4 |
| 1000 | OWS-T1008-24 | 4.81 |  | OWS-T1000-24 | 4.17 |  | 22.0 | 2.0 | 10 | 3.0 | 86 | 26.2 | 5.0 | 2.3 |
| 1500 | OWS-T1508-24 | 7.21 |  | OWS-T1500-24 | 6.25 |  | 32.5 | 3.0 | 15 | 4.6 | 128 | 39.0 | 7.0 | 3.2 |
| 2000 | OWS-T2008-24 | 9.62 |  | OWS-T2000-24 | 8.33 |  | 43.5 | 4.0 | 20 | 6.1 | 171 | 52.1 | 10.0 | 4.5 |
| 2500 | OWS-T2508-24 | 12.02 |  | OWS-T2500-24 | 10.42 |  | 54.0 | 5.0 | 25 | 7.6 | 214 | 65.2 | 12.5 | 5.7 |
| 3000 | OWS-T3008-24 | 14.42 |  | OWS-T3000-24 | 12.50 |  | 65.0 | 6.0 | 30 | 9.1 | 257 | 78.4 | 15.0 | 6.8 |
| 4000 | OWS-T4008-24 | 19.23 |  | OWS-T4000-24 | 16.67 |  | 86.5 | 8.1 | 40 | 12.2 | 342 | 104.3 | 20.0 | 9.1 |
| 5000 | OWS-T5008-24 | 24.04 |  | OWS-T5000-24 | 20.83 |  | 108.5 | 10.1 | 50 | 15.2 | 428 | 130.5 | 25.0 | 11.4 |
| 6000 | - | - | - | OWS-T6000-24 | 25.00 |  | 130.0 | 12.1 | 60 | 18.3 | 513 | 156.4 | 30.0 | 13.6 |

OWS-T 36 in. (914 mm) Wide Mat Models

| Watts | Product # |  |  |  |  |  | Covered surface1 |  | Mat length |  | Cable length2 |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 208V | AMP | Qty | 240V | AMP | Qty | sq. ft. | sq. m | ft. | m | ft. | m | lb | kg |
| 450 | OWS-T0458-36 | 2.16 |  | OWS-T0450-36 | 1.88 |  | 9.5 | 0.9 | 3 | 0.9 | 38 | 11.6 | 2.5 | 1.1 |
| 750 | OWS-T0758-36 | 3.61 |  | OWS-T0750-36 | 3.13 |  | 16.0 | 1.5 | 5 | 1.5 | 63 | 19.2 | 3.5 | 1.6 |
| 1050 | OWS-T1058-36 | 5.05 |  | OWS-T1050-36 | 4.38 |  | 22.0 | 2.0 | 7 | 2.1 | 88 | 26.8 | 5.0 | 2.3 |
| 1500 | OWS-T1508-36 | 7.21 |  | OWS-T1500-36 | 6.25 |  | 31.5 | 2.9 | 10 | 3.0 | 126 | 38.4 | 8.0 | 3.6 |
| 3000 | OWS-T3008-36 | 14.42 |  | OWS-T3000-36 | 12.50 |  | 63.5 | 5.9 | 20 | 6.1 | 251 | 76.5 | 15.0 | 6.8 |
| 4500 | OWS-T4508-36 | 21.63 |  | OWS-T4500-36 | 18.75 |  | 95.0 | 8.8 | 30 | 9.1 | 377 | 114.9 | 22.5 | 10.2 |
| 6000 | - | - | - | OWS-T6000-36 | 25.00 |  | 126.5 | 11.8 | 40 | 12.2 | 502 | 153.0 | 30.0 | 13.6 |

Represents the area covered by the mat including 3’’ (76 mm) spacing between the mat strips but excluding fixed elements to be bypassed and any other required clearances.

2 Represents the total length of the heating cable if it is detached from the flexible strips.

208V = 75% of wattage at 240V.

Installation Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| OTM-CC1 | - | CableCheck – Electrical fault indicator |  |
| OW-G82-G | - | 82 ft. galvanized steel strapping |  |
| KIT-SP2 | - | Repair kit |  |
| - | 1002 | Optional 100’ (30 m) cold lead |  |
| OWS-CAUT | - | Caution playe |  |

* For factory installed options, add the option number to the product number. See the Product Code section inside the front cover.

1 With any OWS-T order, the accessory OTM-CC can be added free of charge upon costumer request.

2 Made to order only. Allow additional 9 to 12 weeks lead time.

Control and Sensor Options

| Product # Kit | Description | Amp. | Volts | Qty |
| --- | --- | --- | --- | --- |
| DS-2C1 | Aerial mounted sensor with controller to detect humidity and temperature | 30A | 277VAC |  |
| ETF-744-99 | 24V outdoor sensor for measuring temperature | - | 24V |  |
| ETO21 | Dual-zone electronic controller | 3 X 16A | 120-240 |  |
| ETO2-BOX | Mounting box for dual-zone electronic controller ETO2 | - | - |  |
| ETOG | Ground sensor to detect humidity and temperature with 33 ft. (10 m) side entry cable | - | - |  |
| ETOG-56 | Ground sensor to detect humidity and temperature with 80 ft. (25 m) bottom entry cable | - | - |  |
| ETOK-1 | Mounting tube for ground sensor ETOG-56 | - | - |  |
| ETOR-55 | Gutter sensor to detect humidity with 33 ft. (10 m) cable | - | - |  |

Requires a ground fault circuit interrupter (GFCI) in the electrical panel.

GFCI control panel with contactors built-in ETO2 and 120+24V control transformers

| Product # Kit | H x W x D (in.) | Description | Qty |
| --- | --- | --- | --- |
| WS-4CIR-208 | 20x16x8 | 4 circuits 208V 30A |  |
| WS-6CIR-208 | 24x20x8 | 6 circuits 208V 30A |  |
| WS-8CIR-208 | 24x24x8 | 8 circuits 208V 30A |  |
| WS-12CIR-208 | 30x24x8 | 12 circuits 208V 30A |  |
| WS-4CIR-240 | 20x16x8 | 4 circuits 240V 30A |  |
| WS-6CIR-240 | 24x20x8 | 6 circuits 240V 30A |  |
| WS-8CIR-240 | 24x24x8 | 8 circuits 240V 30A |  |
| WS-12CIR-240 | 30x24x8 | 12 circuits 240V 30A |  |
| WS-3CIR-600-600 | 24x24x8 | 3 circuits 600V 24A |  |
| WS-6CIR-600-600 | 30x24x8 | 6 circuits 600V 24A |  |
| WS-9CIR-600-600 | 30x24x8 | 9 circuits 600V 24A |  |
| WS-12CIR-600-600 | 30x30x8 | 12 circuits 600V 24A |  |
| WS-15CIR-600-600 | 36x30x10 | 15 circuits 600V 24A |  |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electricial Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |