- Series

ETO2 	Dual-zone Electronic Controller 
for Heating Cables

- Features

Type	- Dual-zone electronic controller.

- Automatic detection of humidity and temperature, in combination with a ground sensor (see available ground sensor models, sold separately).

Color	- Standard : white.

Power	- 120-240V, 50/60 Hz.

Rating	- 1-zone: 3 output relays of 16A each.

- 2-zone: 2 output relays of 16A each.

Construction	- IP20/NEMA 1 rated enclosure, for indoor installation.

Control	- Dry contact relay output, NO (normally open).

Trigger temperature range	- -22 °C (-7.6 °F) to 50 °C (122 °F).

Display	- Backlit digital display.

Mode	- 1-zone.

- 2-zone, individually controlled.

Installation	- DIN-rail mounted in approved panel (not included) or wall mounted in ETO2-BOX mounting box (see options, sold separately).

- Install ground sensor inside heated surface in a clear space that will receive precipitations.

Other feature	- Choice of temperature display format (°C or °F).

- Adjustment of temperature offset per zone.

- Adjustment of afterrun time per zone.

- Alarm relay ouput (5A).

Warranty	- 3-year warranty against defects.

Application	- For controlling snow and ice melting cable applications.

- Residential driveway, sidewalk, access ramp, underground parking ramp,

roof and gutter.

| ETO2 | ETOG (sold separately) | ETOG-56 and ETOK-1 (sold separately) | ETO2-BOX (sold separately) |
| --- | --- | --- | --- |

ETOR-55

ETF-744-99

Controller Model

| Product # | Description | Amp. | Volts | Qté |
| --- | --- | --- | --- | --- |
| ETO2 | Dual-zone electronic controller | 3 X 16A | 120-240 |  |

Ground Sensor Models

| Product # | Description | Qté |
| --- | --- | --- |
| ETOG | Ground sensor to detect humidity and temperature with 33 ft. (10 m) side entry cable |  |
| ETOG-56 | Ground sensor to detect humidity and temperature with 80 ft. (25 m) bottom entry cable |  |
| ETF-744-99 | 24V outdoor sensor fore measuring temperature |  |
| ETOR-55 | Gutter sensor to detect humidity with 33ft.(10m) cable |  |

Options

| Product # Kit | Description | Qté |
| --- | --- | --- |
| ETO2-BOX | Mounting box for dual-zone electronic controller ETO2 |  |
| ETOK-1 | Mounting tube for ground sensor ETOG-56 |  |

1 Requires a ground fault circuit interrupter (GFCI) in the electrical panel.

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electricial Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |

Requires a ground fault circuit interrupter (GFCI) in the electrical panel.