- Series

OTH5007	Non Programmable Electronic Thermostat

- Features

Type	-	Non programmable electronic proportional pulsing thermostat.

Color	-	Standard: white.

Voltage	-	277/347V, 50/60 Hz.

Rating	-	Maximum load: 14.4A at 277/347V.

-	Minimum load: 1.25A at 277/347V.

Control	- 10-minute or 15-second cycle.

-	Resolution: ± 0.5 °C (1 °F).

-	Precision: ± 0.15 °C (0.27 °F) on 15-second cycle.

-	Precision: ± 0.5 °C (1 °F) on 10-minute cycle.

Temperature range	-	5 °C (41 °F) to 30 °C (86 °F).

Display	-	Digital screen.

- Display ambient temperature and heating level indicator (in continuous),

setpoint temperature setting (on demand),

fan-forced heater mode and keypad lock (when activated).

Other feature	-	Choice of temperature display format (°C or °F).

- Temperature setpoints recorded permanently in case of a power outage.

- Ability to set minimum and maximum setpoints.

- Thermistor temperature sensing element.

Installation	-	2-wire thermostat, easy to install.

- Adapter plate for 347V junction box included.

Application	-	Baseboard heater, radiant ceiling, radiant ﬂoor (without ﬂoor sensor), 
convector (short cycle) and fan-forced convector (long cycle).

Warranty	-	3-year warranty against defects.

- Model

| Product # | Description | Volts | Watts | Qty |
| --- | --- | --- | --- | --- |
| OTH5007 | Non programmable electronic proportional pulsing thermostat | 277/347 | 4000/5000 |  |

Standard color is white.

- Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |