Series

W

BW

GR

C

SB

XCE 	Xlerator High Velocity Hand Dryer

Features

Color	-	Standard: white (W).

Finish	-	Standard: epoxy/polyester powder paint.

- Optional : White polymer BMC (BW), chrome (C), graphite (GR)

or brushed stainless steel (SB).

Voltage	-	110-120V, 208-240V, 1-phase.

Construction	-	Standard die cast zinc alloy monobloc housing.

Fan	-	Motor operating at 20,000 rpm.

- Air volume at high velocity of 19,000 lfm

- Adjustable motor speed.

- Includes 1.1 in. noise reduction nozzle which brings the sound level down to 71 dBA (81 dBA with hands).

Heating element	-	Helical coil nickel-chromium alloy wire protected by mica insulator.

- Adjustable heat control : High, Medium, Low and Off.

Control	-	Infrared optical sensor to activate hand dryer.

- 35 second lockout feature if hands are not removed.

Installation	-	Surface mounted.

Warranty	-	5-year warranty against defects.

Application 	-	Bar, school, penitentiary, restaurant, commercial mall.

Models

| Product # | Qty | Color1 | Volts | Watts | No heat watts | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | lb | kg |
| XL-W-11N-110-120V |  | W | 110-120 | 1240-1450 | 460-530 | 17 | 7.7 |
| XL-BW-11N-110-120V |  | BW |  |  |  | 15 | 6.8 |
| XL-GR-11N-110-120V |  | GR |  |  |  | 17 | 7.7 |
| XL-C-11N-110-120V |  | C |  |  |  | 17 | 7.7 |
| XL-SB-11N-110-120V |  | SB |  |  |  | 16 | 7.3 |
| XL-W-11N-208-240V |  | W | 208-240 | 1160-1490 | 425-500 | 17 | 7.7 |
| XL-BW-11N-208-240V |  | BW |  |  |  | 15 | 6.8 |
| XL-GR-11N-208-240V |  | GR |  |  |  | 17 | 7.7 |
| XL-C-11N-208-240V |  | C |  |  |  | 17 | 7.7 |
| XL-SB-11N-208-240V |  | SB |  |  |  | 16 | 7.3 |

1 W (White), BW (White polymer BMC), GR (Graphite), C (Chrome), SB (Brushed stainless steel).

Eco model available on demand (-ECO).

Options

| Product # | Qty | Description |
| --- | --- | --- |
| XL-40502 |  | ADA compliant stainless steel recess |
| XL-89S |  | Wall guard – stainless steel, 2 per box |
| XLP-89B |  | Microban® anti-microbial plastic wall guard - black |
| XLP-89W |  | Microban® anti-microbial plastic wall guard - white |

### Others

| Type | Quantity | Model et description Complementary information | Volts | Phase | Watts | Length mm/in. | Information Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |